Memorandum

Planning and Urban Development Department

Planning Division

____________________________________________________________

To: Stuart O’Brien, Chair and Members of the Portland Planning Board

From:	Richard Knowland, Senior Planner

Date:	July 18, 2014

Re:	Bayside Anchor; Corner of Boyd St. and East Oxford St.; Portland Housing

Development Corp.

Project #: 2014-079			 CBL:  22-01-04 and 01

Meeting Date:  July 22, 2014

______________________________________________________

- INTRODUCTION

A public hearing has been scheduled to consider a proposal by Bayside Anchor Development, LLC  for a 45 unit housing development on the corner of Boyd Street and  East Oxford Street. The development is called Bayside Anchor. Recently the City Council approved a rezoning of this property from R-6 to R-7 after review and recommendation by the Planning Board.

The property is part of the East Bayside residential campus of the Portland Housing Authority (PHA) which includes Kennedy Park, Bayside Terrace and Bayside East. For development and financing purposes, Bayside Anchor will be owned by an affiliate corporation and have its own lot but function as part of the PHA campus.

The development is subject to site plan and subdivision review. The Board of Appeals will be reviewing the Head Start use as a conditional use. The public hearing takes place on July 17th.

Attachment 1 includes the initial Bayside Anchor site plan and subdivision application. xxxxxxxxxxxxxxxxxxx Attachment P includes site plans and other drawings.

Notice of this public hearing consisted of a legal ad item appearing in the Portland Press Herald on July xxxx  and Julyxxxx.  Notices were sent to103 notices area property owners.

- FINDINGS

Existing Zoning:  R-7

Land Area:  20,147 sf

Existing Use: Parking lot

Proposed Uses:  Residential, first floor (Head Start and day care, PHA offices,

community policing office)

Dwelling Units:  45  (5 efficiencies; 34 one bedrooms; 6 two bedrooms)

Building Footprint:  9,610 sf

Building Floor Area:  38,770 sf

Residential Space: 35,870 sf

Non-Residential Space: 2,900 sf

Lot Coverage:  68%

Impervious:   7,532 sf (existing)

13,767 sf (proposed)

Building Height:  45 ft

Existing Parking Spaces:  26 (on-site)

Proposed Parking Spaces:  34 (off-site from campus parking surplus)

Proposed Bicycle Spaces: 30 inside building; 8 outside

Project Cost:  $5,293,654

The submission lists Portland Development Corporation and Bayside Anchor Development Company, LLC as co-applicants. The property owner is listed as the Portland Housing Authority. Bayside Anchor will be owned by an affiliate corporation for development and financing purposes but will function as part of the PHA campus.

The Bayside Anchor was the subject of a zone change from R-6 to R-7 which was reviewed by the Planning Board at a public hearing on April 22, 2014 and then subsequently approved by the City Council on May 19, 2014. For purposes of holding a public hearing the entire East Bayside R-6 was advertised for a R-7 designation although in the end the Board’s recommendation reduced the size of the map change to several blocks near Franklin Street which are part of the PHA campus. Bayside Anchor is within the new R-7 district. See enacted zoning map (Attachment A).

- PROPOSED DEVELOPMENT

PHA proposes to construct a 45 unit multi-family building at the corner of Boyd Street and Oxford Street on the site of an existing parking lot. A significant portion of the building’s first floor will be devoted to non-residential space including PHA campus offices, Head Start pre-school and a community policing office. These services are presently located elsewhere on the PHA campus but will now be consolidated in one location. Of the 45 units, 9 are expected to be market rate.

The building hugs the street frontage of Oxford Street and is within several feet of the street line. The building continues this pattern along Boyd Street except that a portion of the building has been recessed to accommodate an outdoor play area for Head Start as well as a patio (court yard) and open space on the northerly side of property. The patio (1,385 sf) and Head Start outdoor area (370 sf) feature an unspecified “special paving surface”. Other amenities within the patio area include an extended seating wall, space for picnic tables, and a rain garden area. Between the patio area and the property line is a lawn area encompassing about 6,000 sf.

No on-site parking is provided. Off-street parking will be provided within the existing PHA campus parking lots.

The submission describes Bayside Anchor as an example of  “innovative sustainable design practices including incorporation of Passivhaus ultra-low-energy design principles, including plentiful fresh air, passive and active solar design, superinsulation and airtightness, used in conjunction with highly efficient energy recovery ventilation, to dramatically reduce energy demand and cost of expensive multi-family mechanical systems.”

Submitted building elevation drawings indicate the four story building is just under 45 feet tall. See Attachment P. Solar panels are shown on top of the flat roof with screening for mechanical equipment below.

The primary façade treatment is described as fiber-cement panel siding. A manufacturer’s catalogue sheet has been provided. See Attachment 2.

The primary residential entrance appears to be along Oxford Street with a separate entrance for PHA offices.  A canopy and accompanying signage has been integrated into the building to demark the building entrances.

Along Boyd Street a similar canopy has been provided for the HeadStart entrance although the sign is smaller in scale. The site plan indicates that a ramp is proposed along the face of the building (5 feet wide and 16 feet long with railing) to provide access into the entrance. Presumably this will require an easement or license from the City. Applicant indicates grade differences require this arrangement.  As the ramp and rail are located within the street right-of-way we have requested an explanation why this is necessary and whether the entrance could be located outside the street right-of-way (closer to the corner of the building within the patio area). See Attachment 2 for this explanation.

Legal Agreements

Bayside Anchor proposes a number of improvements that are within the right-of-way. Licenses and easements will be required for the following improvements.

- Concrete foundations and footings within the Oxford and Boyd Street rights-of-way

- Permanent canopies extending from the building into the right-of-way.

- Pedestrian ramp/steps on Boyd Street.

- A pedestrian easement to the City where sidewalk encroaches on BA land on Oxford

Draft language for these easements and licenses is submitted as part of the application.

Upon approval and according to the proposed Parking Management Plan, the PHDC/Bayside Anchor Development Company LLC will enter into a formal agreement with PHA for the long term use of these spaces.

- DEVELOPMENT REVIEW

The site plan has been reviewed by City Staff including Planning Staff, Zoning Administrator, Public Services (and review consultants), City Arborist and the Fire Department for conformance with the review standards of the Site Plan Standards (sec 14-526) andxxxxx

- Transportation Standards

- Impact on Surrounding Street Systems

Gorrill-Palmer Consulting Engineers has submitted a traffic analysis entitled “Traffic Forecast, Parking Assessment and TDM Plan for Bayside Anchor” dated May 2014.  See Attachment xxxxxComments from Tom Errico (Traffic Review Consultant) are shown as Attachment xxx

The G-P analysis indicates the following  trip generation forecast  for all  uses (residential and non-residential) associated with Bayside Anchor.

Weekday                237 trip ends

AM Peak Hour         24  trip ends

PM Peak Hour         29  trip ends

G-P indicates “this level of traffic increase will not have a notable effect on the adjacent roadway system”. Consultant also indicates a review of 2010-2012 found no high crash locations in the project vicinity.

- Access and Circulation

- Site Access and Circulation

The project does not have any on-site parking or driveways. Parking will take place either on the street or at various PHA parking lots on the East Bayside campus.

Although no vehicle circulation will take place on the site it is important that  functional circulation be provided between the street and the proposed uses. A 15 minute parking space will be designated along Boyd Street to accommodate potential drop-offs from Head Start. Along Oxford Street two 15 minute drop-off spaces will be designated to accommodate truck deliveries, drop-offs  or  assist tenants in transferring grocery bags up to their apartment rather than walking an extended distance from the campus parking. A police parking space will also be designated along Oxford Street.

Loading and Servicing

See above discussion for designated parking spaces along the project street frontage.

Dumpsterxxxxxx

- Sidewalks

Applicant is proposing new granite curb and brick sidewalk along the Boyd and Oxford project frontage.

The sidewalk along Oxford Street varies from 6’- 4” to 8’- 4” wide with a typical 5 foot wide esplanade. The esplanade includes 5 street trees and 2 bike racks.

A new handicap ramp and crosswalk will line up with Bayside Anchor residential entrance. The mid-block crosswalk extends across the street where a new handicap ramp will be installed which provides direct access to two handicap parking spaces in a PHA parking lot.

Along Boyd Street the curb line has been extended out to accommodate a ramp that runs along the face of the building. The ramp is about 16 feet long (with railing) and 5 feet wide. The curb line tapers back to the normal curb line just past the northerly edge of the building. Along Boyd Street space limitations preclude an esplanade but 6 street trees are proposed. The sidewalk along Boyd

- Public Transit Access

Not applicable. Development is not located along an existing public transit route.

- Parking

As PHA is not proposing on-site parking for Bayside Anchor and is relying on existing campus parking resources, documentation of existing campus parking supply and demand is critical.

G-P has provided documentation that PHA’s existing East Bayside campus has a parking demand of 0.56 spaces per dwelling unit. This was based on a May 4, 2014 Sunday night survey (9:00 PM to midnight) that found the highest demand of PHA parking lots was 91. With 157 spaces this translates into a surplus of 66 spaces.

As some PHA residents are likely to park on-street, G-P also surveyed the number parking spaces occupied by vehicles during the study period which was found to be 58 spaces. Adding these spaces to the 91 spaces results in parking ratio of 0.92 spaces per occupied housing unit.

G-P determined the number of unoccupied on and off-street parking spaces within certain distances of the Bayside Anchor site.

Unoccupied parking within 300 feet of the project:

- Parking lots:18

- On Street spaces: 73

- Total: 91

Unoccupied parking within 500 feet of the project:

- Parking lots: 58

- On Street spaces:130

- Total:188

G-P also reviewed occupancy counts for other peninsula projects including 53 Danforth St., 645 Congress St., Walker Terrace, Oak Street Lofts and Franklin Towers. The average parking demand for these projects was calculated by G-P at 0.48 spaces per unit.

Based on the above analysis, G-P is forecasting an appropriate parking demand for Bayside Anchor to be 0.75 spaces per dwelling unit. Based upon Bayside Anchor’s 45 units, this translates into a parking demand of 34 spaces.

Parking Supply is summarized below

Within 300 feet of the project: 118 spaces on street; 36 in PHA lots   [total:154]

Within 500 feet of the project: 227 spaces on street; 109 in PHA lots [total:336]

Parking Demand is summarized below

Within 300 feet of the project: 123 spaces

Within 500 feet of the project: 208 spaces

Net Available Parking to Bayside Anchor: A comparison of parking demand and parking supply shows there is projected to be 31, and 128 spaces within 300 and 500 feet respectively upon completion of the project for the projected demand of 34 spaces. Most of these spaces are on-street.

- Transportation Demand Management

The submission includes a brief description of the elements of a proposed TDM plan

- The PHA Property manager will coordinate the TDM plan.

- PHA will adopt policies limiting each household to one parking permit per household.

- Automobile Parking Reduction Strategies…38 bicycle parking spaces are proposed with 30 of these located within the building.

(b) Environmental Quality Standards

(1) Preservation of Significant Features

The site is located in an urban site that was previously disturbed. The site does not have any significant natural features as defined in this section of the site plan ordinance.

(2) Landscaping and Landscape Preservation

There are 6 mature trees on the site. Four of the trees will be removed while 2 others will be “removed and stockpiled for future use”. A tree across the street on Oxford Street where a crosswalk will be installed for handicap parking spaces is also slated to be removed and stockpiled for reuse.

Site Landscaping

(3) Water Quality, Stormwater Management and Erosion Control

Stormwater Management

The proposed development will result in a net increase of 7,000 sf. in impervious area.

Quantity - Applicant proposes to disconnect an existing stormdrain on the site that discharges a combined sewer so that stormwater runoff will be by surface flow. Detention will be provided to temporarily store stormwater and release it slowly over time, thus keeping the post development discharge rates below the existing rates. Storage will be provided by a modular box underdrain system that will be installed under the proposed patio.

Quality - Stormwater treatment is achieved by a rain garden that will be installed adjacent to the patio area. Site plan indicates that a FocalPoint biofiltration system will be used which will treat at a minimum, the first inch of rainfall from the site’s new impervious area.

xxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxx

© Public Infrastructure and Community Safety Standards

- Consistency with City Master Plans

The proposal appears to be consistent with City master plans.

- Public Safety and Fire Prevention

- Availability and Adequate Capacity of Public Utilities.

The site plan indicates there are power, water, gas and sewer utilities in either Oxford Street or Boyd Street. The latest submission from the applicant does not contain any ability to serve letters from these utilities.

- Site Design Standards

- Massing, Ventilation and Wind Impact

- Shadows

The submission states: The Bayside Anchor project is 46’-0” in building height, with an approximate length of 195’-0’ in length on the winter solstice. There are no significant public resources within 195’-0” of the project site as defined in the City of Portland Technical Manual, Section 11.3, therefore no significant shadow impacts are anticipated and no further shadow study is required.

- Snow and Ice Loading

- View Corridors

The site will not obstruct public views identified in the Downtown Vision Corridor Protection Plan.

- Historic Resources

- Exterior Lighting

- Noise and Vibration

- Signage and Wayfinding

- Zoning Related Design Standards

Residential Developments: Shall integrate with and respect the character of surrounding residential development in terms of architectural form, landscaping and open space, façade materials, roof pitch, massing and height as demonstrated by compliance

Landscaping

Eleven street trees are proposed with 5 on Oxford Street and 6 on Boyd Street. Seven of the trees are Ginkos while the remainder are Eastern Redbuds. Six Serviceberry trees are proposed near the patio along with a Katsuratree.

The site plan indicates that the rain garden vegetation will be composed of 9 different types of plants.

Two Serviceberry and one Katsuratree are shown along the northerly property line.

Utilities

A utility plan has not been submitted to date but applicant indicates water, sewer, power, gas service is available in the adjoining streets.

Head Start Use

The R-7 zone tracks the R-6  zoning text in terms of uses. A nursery school or kindergarten is a conditional use with review by the Board of Appeals. Applicant has submitted material in support of their assertion that the Head Start program is an accessory use to the PHA campus and not a conditional use. See Attachment 2. This material will be reviewed by the Zoning Administrator.

Subdivision Plan

A multi-family development constitutes a subdivision therefore a subdivision plan will need to be submitted.

Next Steps

A second workshop could be scheduled on June 24 if necessary. If not a public hearing will be scheduled for a future date.

Attachments:

Staff Attachments:

- Enacted R-7 Zoning Map

- Tom Errico (Traffic Review Consultant) Memo dated June 5, 2014

- David Senus (Development Review Consultant) dated May 30, 2014

Applicant Submissions

1.   Site and Subdivision Submission dated May 12, 2014

2.   Responses to staff comments from Carroll Associates dated June 5, 2014

P- Site Plans and Other Drawings