Copyright 1997, The American Institute of Architects (AIA)

- SECTION 12356 - KITCHEN CASEWORK

This Section uses the term "Architect."  Change this term to match that used to identify the design professional as defined in the General and Supplementary Conditions.

Verify that Section titles referenced in this Section are correct for this Project's Specifications; Section titles may have changed.

- PART 1 - GENERAL

- 1.1	RELATED DOCUMENTS

A.	Site drawings produced by Pinkham and Greer Consulting Engineers, foundation/basement plan drawings produced by TFH Architects PA and Structural Design Consulting, building drawings produced by TFH Architects, mechanical drawings produced by TFH Architects, electrical and lighting drawings produced by TFH Architects.  Drawings and general provisions of the Contract, including General and Supplementary Conditions and other Division 1 Specification Sections apply to all sections.

- 1.2	SUMMARY

A.	This Section includes the following:

Adjust list below to suit Project.

1.	Laminate-faced kitchen and vanity cabinets.

2.	Plastic-laminate countertops.

- 1.3	SUBMITTALS

A.	Product Data:  Provide for each casework and hardware type specified.

Delete items if not applicable or for which Product Data are not required.

B.	Shop Drawings:  For cabinets and countertops.  Include plans, elevations, details, and attachments to other work.  Show materials, finishes, filler panels, hardware, edge and backsplash profiles, cutouts for plumbing fixtures, and methods of joining countertops.

C.	Samples for Initial Selection:  Manufacturer's color charts consisting of units or sections of units showing the full range of colors, textures, and patterns available for each type of material exposed to view.

Delete paragraph above if colors and other characteristics are preselected and specified or scheduled.  Retain paragraph below with or without above.

D.	Samples for Verification:  For the following materials; in sets showing the full range of color, texture, and pattern variations expected:

Delete subparagraphs below if not applicable to types of casework selected or not considered necessary for verification of selections.

1.	Plastic laminate for casework finish, 8 by 10 inches (200 by 250 mm).

2.	Plastic laminate for countertops, 8 by 10 inches (200 by 250 mm).

3.	One unit of each type of exposed hardware.

Usually retain paragraph and appropriate subparagraphs above and delete below; retain paragraph and subparagraphs below if full-size Samples are required.  Add or delete sample units below to suit Project.  Below will add cost to Project and may not be necessary if manufacturers are restricted to a list of those known to be acceptable.

- 1.4	QUALITY ASSURANCE

Revise paragraph below if more than one type of cabinet is used and all types are not required to be from same manufacturer.

A.	Source Limitations for Cabinets:  Obtain cabinets through one source from a single manufacturer.

Delete paragraph below if catalog numbers are not used on Drawings; revise if substitutions are not allowed.

B.	Product Designations:  Drawings indicate size, configurations, and finish material of casework by referencing designated manufacturer's catalog numbers.  Other manufacturers' casework of similar sizes, similar door and drawer configurations, same finish material, and complying with the Specifications may be considered.  Refer to Division 1 Section "Substitutions."

C.	Quality Standards:  Unless otherwise indicated, comply with the following standards:

HUD's "Minimum Property Standards" requires compliance with KCMA A161.1, which specifies performance requirements suitable for specifying basic- and medium-quality cabinets.

1.	Cabinets:  KCMA A161.1.

Certification below does not require that cabinet manufacturer be member of KCMA.  It does require testing and in-plant inspection by KCMA-authorized independent agency.

a.	KCMA Certification:  Provide cabinets with KCMA's "Certified Cabinet" seal affixed in a semiexposed location of each unit and showing compliance with the above standard.

2.	Plastic-Laminate Countertops:  KCMA A161.2.

- 1.6	PROJECT CONDITIONS

A.	Environmental Limitations:  Do not deliver or install kitchen casework until building is enclosed, wet-work is complete, and HVAC system is operating and will maintain temperature and relative humidity at occupancy levels during the remainder of the construction period.

B.	Field Measurements:  Where kitchen casework is indicated to fit to existing construction, verify dimensions of existing construction by field measurements before fabrication and indicate measurements on Shop Drawings.  Provide fillers and scribes if necessary.

C.	Field Measurements for Countertops:  Verify dimensions of countertops by field measurements after base cabinets are installed but before countertop fabrication is complete.  Coordinate fabrication schedule with construction progress to avoid delaying the Work.

- PART 2 - PRODUCTS

- 2.1	MANUFACTURERS

- A.	Available Manufacturers:  Subject to compliance with requirements, manufacturers offering products that may be incorporated into the Work include, but are not limited to, the following:

Retain above for nonproprietary or below for semiproprietary Specification.  Refer to Division 1 Section "Materials and Equipment."

- B.	Available Products:  Subject to compliance with requirements, cabinets that may be incorporated into the Work include, but are not limited to, the following:

Retain above for nonproprietary or below for semiproprietary Specification.  Refer to Division 1 Section "Materials and Equipment."

- C.	Products:  Subject to compliance with requirements, provide one of the following:

Below are examples for format only for use with either paragraph above.  Replace with trade names and manufacturers of products that have desired characteristics.

1.	Breckenridge; American Woodmark Corp.

2.	Rockingham; Merillat Industries, Inc.

Delete both paragraphs and subparagraphs above and retain below for proprietary Specification.  Trade name and manufacturer below are examples for format only.  Replace with selected trade name and manufacturer.

- D.	Product:  Subject to compliance with requirements, provide "Rurledge" by IXL Cabinets.

- 2.2	CASEWORK HARDWARE

Revise paragraph below if hardware is specified in detail or if cabinets are specified by manufacturer's name and model designation that includes hardware selection.

A.	General:  Manufacturer's standard units complying with ANSI A156.9, of type, material, size, and finish as selected from manufacturer's standard choices.

Select one of five hinges below.  Add BHMA designation or other descriptive information if required.

- 2.3	PLASTIC-LAMINATE COUNTERTOPS

A.	  General:  Comply with ANSI A161.2.

Delete paragraph below if countertop configuration is shown on Drawings.

B.	Configuration:  Provide countertops with the following front, cove and backsplash:

Select one subparagraph from each set below.

1.	Front:  No drip.

2.	Cove:  Direct bond.

3.	Backsplash and End Splash Style.:  Curved or waterfall shape with scribe.

B.	Plastic-Laminate Substrate:  Comply with ASTM D 1037..

Usually retain subparagraph below for good-quality work.  Prefabricated tops will not usually comply.

- 1.	Particleboard:  Comply with ANSI A208.1, 45 lb. density, not less than ¾” thick.

Revise thickness below to suit Project.  Delete if 3/4-inch (19-mm) thickness is required.

- PART 3 - EXECUTION

- 3.1	INSTALLATION

A.	Install casework with no variations in flushness of adjoining surfaces; use concealed shims.  Where casework abuts other finished work, scribe and cut for accurate fit.  Provide filler strips, scribe strips, and moldings in finish to match casework face.

B.	Install casework without distortion so doors and drawers fit openings and are aligned.  Complete installation of hardware and accessories as indicated.

C.	Install casework and countertop level and plumb to a tolerance of 1/8 inch in 8 feet.

D.	Fasten cabinets to adjacent units and to backing.

1.	Fasten wall cabinets through back, near top and bottom, at ends and not less than 24 inches (600 mm) o.c. with No. 10 wafer-head screws sized for 1-inch (25-mm) penetration into wood framing, blocking, or hanging strips.

Select subparagraph above for wood frame construction.  Select subparagraph below for metal studs, or select above and show wood blocking between studs.  Revise below if plaster is used.  Revise either subparagraph for seismic loads.

- 2.	Fasten wall cabinets through back, near top and bottom, at ends and not less than 24 inches (600 mm) o.c., with toggle bolts through metal backing behind gypsum board.

E.	Fasten plastic-laminate countertops by screwing through corner blocks of base units into underside of countertop.  Spline and glue joints in countertops and provide concealed mechanical clamping of joint.

- 3.2	ADJUSTING AND CLEANING

A.	Adjust casework and hardware so doors and drawers are centered in openings and operate smoothly without warp or bind.  Lubricate operating hardware as recommended by manufacturer.

B.	Clean casework on exposed and semiexposed surfaces.  Touch up factory-applied finishes to restore damaged or soiled areas.

- END OF SECTION 12356