Planning and Urban Development Department

Planning Division

Subject:		R-6 Small Infill Design Review – 70 Anderson Street

Written by:		Caitlin Cameron, Urban Designer

Date of Review	:  	Wednesday, July 20, 2016

A design review according to the City of Portland Design Manual Standards was performed for the proposed new construction of a multi-family dwelling at 70 Anderson Street.  The review was performed by Caitlin Cameron, Urban Designer, Nell Donaldson, Planner, and Shukria Wiar, Planner, all within the Planning Division of the Department of Planning & Urban Development.  The project was reviewed against the R-6 Small Infill Development Design Principles & Standards (Appendix 7 of the Design Manual).

Findings of the Design Review:

The proposed design does not pass all of the criteria – please refer to comments below.  The applicant may either revise the design according to the review comments and resubmit or appeal the decision of the design review to the Planning Board.

Design Review Comments (red text denotes principles or standards that are not met):

- Roof plans lacking information – slopes, ridge lines.

- Building height is measured to the mid-point of a sloped or gabled roof, not the ridge.

- Either provide scaled drawings or dimension key components like building height, height from sidewalk of ground floor windows, roof slopes, etc.

- Why is the second floor taller than the other two floors?

Principle A Overall Context – Not Met – see below.

- A-1 Scale and Form:  The scale the project is larger than most buildings on this small street, but the height is three stories and does not overshadow the neighboring 2.5 or 3 story residential buildings.  The form is mostly defined by the gabled roof which is a traditional New England roof form, however, see comments below about proportion and slope of roof.

- A-2 Composition of Principal Facades:  The composition of the street-facing facades is consistent with context in terms of using symmetrical bays (two or three bays).

- A-3 Relationship to the Street: The building placement is consistent with the spacing of the residential fabric – slightly setback from sidewalk to allow for stoops and provide some privacy.  The ground floor is not raised and is not consistent with residential development patterns.

Principle B Massing – Not Met – The roof pitch and height of the gable roofs do not reflect and reinforce the traditional building character of the neighborhood.

- B-1 Massing: The proposed mass is wider on the street than the typical building context but the use of row houses mitigates that massing.

- B-2 Roof Forms: The proposed gable roof is not correctly proportioned and sloped for the context or the scale of the building.  Gable roofs are steeper and typically would start after the second floor with the roof space occupied and expanded with dormers.

- B-3 Main Roofs and Subsidiary Roofs:  Met – there is a clear main roof form.

- B-4 Roof Pitch: The standard requires gable roofs to have a slope between 7:12 and 12:12.  This roof is too shallow, especially in consideration of the building height and width, throwing the proportion of the building ends off.  There are several options to revise to meet the standard (in order of preference):

  - Lower the height of the roof to start at the second floor and use dormers/dustpan design to accommodate the living spaces of the third floor.  This is in keeping with the massing and form of residential buildings.

  - Correct the slope of the gable roof to meet the standard.

  - Change the roof to a hipped roof

- B-5 Façade Articulation: The project employs only one of the required articulation elements – covered entries.  An additional articulation element (bays, gables/dormers, recessed entry, balconies) must be part of the design to meet the standard.

- B-6 Garages: Not applicable.

Principle C Orientation to the Street – Not Met – The project is oriented to the streets, however, the privacy associated with private residential architecture is not created – ground floor living spaces too low, windows too low, lack of transition at entries.

- C-1 Entrances: The entry to each unit is well-emphasized with the canopy.

- C-2 Visual Privacy:  Visual privacy is not adequately addressed; many ground floor windows do not appear to be higher than 48” above adjoining sidewalk grade; the ground floor is not adequately raised above sidewalk grade appropriate for private residential buildings with living space on the ground floor (at least 24” is required by the standard).

- C-3 Transition Spaces: The project uses a canopy at each entrance but very little transition space is provided – ground floor living spaces are not elevated or separated with vestibules.

Principle D Proportion and Scale – Not Met – While the scale of the buildings overall are harmonious and human-scaled, the overall proportion of the gable-end of the buildings is awkward – the roof height and slope is out of proportion with the width and height of the building (as discussed above).

- D-1 Windows: The majority of windows are rectangular and have vertical proportion; window proportion is not a proportion found in the context, however.

- D-2 Fenestration:  The project appears to meet the 12% fenestration requirement and appropriately scaled to the massing of the building.

- D-3 Porches: Not applicable

Principle E Balance – Met – The building façade composition creates a sense of balance with good use of overall and local symmetry and articulation of façade materials.

- E-1 Window and Door Height:  The majority of window and door head heights align along a common horizontal datum.

- E-2 Window and Door Alignment: The majority of windows shall stack so that centerlines of windows are in vertical alignment.

- E-3 Symmetricality: Primary window compositions are arranged symmetrically around discernable vertical axes.

Principle F Articulation – Met – Based on the information given, it appears the project employs visually interesting and well composed facades.

- F-1 Articulation: It is a little bit difficult to discern the trim and window details of the project however, if there is a little bit of a shadow line created with these details, as it appears there would be, then the standard is met.

- F-2 Window Types: Two window types

- F-3 Visual Cohesion: The visual cohesion of the façade is good.

- F-4 Delineation between Floors: The first floor is delineated through trim and change of clapboard reveal.

- F-5 Porches, etc.: The stoop and canopy are well integrated into the overall design.

- F-6 Main Entries: The main entry is emphasized with prominent placement facing the street, the use of a canopy.

- F-7 Articulation Elements:  It appears the rake of the roof meets the 6” requirement; the trim details are not clear; there do not appear to be offsets in the principal façades.

Principle G Materials – Met – The material choices are well-placed and clapboard is contextual for residential architecture in this neighborhood.

- G-1 Materials: The residential context is predominantly clapboards with occasional shingle or brick.

- G-2 Material and Façade Design: The materials are appropriately placed according to their nature.

- G-3 Chimneys: Not applicable.

- G-4 Window Types:    Two window types.

- G-5 Patios and Plazas: More info needed: Internal patio space design not clear – what are material choices?