August 8, 2012

To:   	Barbara Barhydt

Jean Fraser

From:	David Margolis-Pineo

Public Services Review Comments

Re:	Opechee

Public Services staff has the following comments on this project.

- The sidewalk brick on India St and the brick on Fore St, proposed to be removed and reset, are two different style bricks.  During the removal and resetting process, it is expected that a percentage of brick will be damaged or broken requiring replacement.  The applicant will be require to replace the brick in kind.  It is my understanding that both bricks are available at LaChance.  Please add a note informing the Contractor of this condition.

- There are several locations where sewer lines are proposed to be removed.  Please provide a detail on how the penetrations into manholes and catchbasins will be sealed.  Also please add a note informing the contractor that the sealed penetrations shall be inspected by John Emerson (318-0239) prior to backfilling.

- Please indicate HC detectable panels on the ramps at Middle and India Streets.

- Please relocate the sidewalk ramp at Middle and India crossing India Street to create a more perpendicular crossing.  This may mean eliminating a street tree.

- HC detectable panels on the ramps at the drive entrance on Middle Street are not required.

- Fore Street is under moratorium until October 21, 2016.  To avoid cutting into Fore St. with the foundation drain, the City would consider tying into the back of the adjacent catchbasin on Fore St.  The applicant is requested to contact me to discuss.  207-874-8850, 207-400-6695

- The City of Portland will need an access easement for the portion of sidewalk which encroaches the applicant’s property at the corner of India and Fore.

We have no further comments at this time.