## Memorandum

Planning and Urban Development Department

Planning Division

To:		Carol Morrissette, Chair and Members of the Portland Planning Board

From:		Shukria Wiar, Planner

Date:		October 25, 2012

Re:		Hotel Conversion, 119 Exchange Street, 119 Development, LLC

Project #:	2012-602	CBL:  032-C-001 and 032-C-002

Meeting Date:  October 29, 2012

I.	INTRODUCTION

The applicant, 119 Development, LLC, has submitted preliminary plans and requested a Level III Site Plan review for the Guy Gannett Building on Congress Street.  The proposal is to convert the existing building to a hotel and restaurant.  The building will comprise seven- (7) story mixed use building of 84,491 sq. ft. of hotel and restaurant floor space.  The restaurant entrance will be oriented to Congress Street and the Hotel lobby entrance will be from Exchange Street

| Requested Waiver | Standard and any waiver provisions |
| --- | --- |
| Transit Facility Under the Site Plan ordinance requirements for public transit improvements, this project may be required to provide an improvement consisting of a bus shelter and pullout bay unless there is an existing transit shelter and/or pullout bay within 1350 feet of the closest primary proposed building. | Public Transit Access: b. Transit facilities shall consist of a transit shelter and a transit pullout bay. c. Transit facilities shall be connected to the public sidewalk system. |
| Bicycle Parking The applicant is required to install 10 (ten) parking spaces for the 50 leased spaces. The applicant is proposing to add two parking spaces and seeking a waiver of the remaining. | Parking: b. Location and Required Number of Bicycle Parking Spaces: (i) The site plan shall provide secure bicycle parking in conformance with Section 1 of the Technical Manual and shall meet the following requirements: (b) Non-residential structures. Two (2) bicycle parking spaces for every ten (10) vehicle parking spaces for the first one hundred (100) required vehicle parking spaces, plus one (1) bicycle parking space for every twenty (20) required motor vehicle parking space over one hundred (100) required vehicle parking spaces. |
| Motorcycle and Scooter Parking The applicant is not proposing any parking for motorcycles or scooters. | Motorcycle and Scooter Parking. (i) The site plan shall accommodate access and parking for two-wheeled motorized vehicles such as motorcycles and scooters. |
| Landscaped Islands in Parking Lot Since the applicant is leasing existing surface parking lot spaces, they are requesting a waiver of this standard. | Parking Lot Landscaping: (a) Developments with five (5) or more parking spaces shall include at least two (2) trees (or one (1) tree and three (3) shrubs) per five (5) parking spaces planted in landscaped islands to screen shade and break up parking. Trees and shrubs in parking lots may be in informal groups, straight rows, or concentrated in clusters as detailed in Section 4 of the Technical Manual. (b) Landscaped islands shall be distributed so that uninterrupted pavement does not exceed forty (40) parking spaces. |

The parking for the building will be across Congress Street at the surface parking lot located at 385 Congress Street.

Required reviews:   The proposal is being reviewed under the Land Use Code provisions 14-526 (Site Plan). The applicant has asked for waivers as listed below:

Applicant Name	119 Development, LLC

Consultants		Steve Bushey, Deluca-Hoffman Associates, Inc.

II.	PROJECT DATA

Existing Zoning:  				Downtown Business B-3

Existing Use:  					Vacant Office Building

Proposed Use:					Hotel and Restaurant

Existing number of lots:				Two

Proposed number of lots:			Two

Parcel Size:					0.23 acres

Impervious Surface Area:

Existing:				10,049 sq. ft.

Proposed:				10,049 sq. ft.

Net Change:				Zero (0)

Building Area:

Existing Building Area:			84,491 sq. ft.

Proposed Building Area:		84,491 sq. ft.

Proposed Building Net Change:		Zero (0)

Existing Building Footprint:		10,049 sq. ft.

Proposed Building Footprint:		10,049 sq. ft.

Building  Height:				77’ at the highest part of building

Parking Spaces:

Existing:				Zero (0) - Only on-street available

Proposed:				Zero (0)

Number of Handicapped Spaces:	N/A

Lease of Spaces:			50 spaces at 385 Congress Street

Bicycle Parking Spaces:

Existing:				Two (2)

Proposed:				Two (2)

Estimated Cost of Project:			$8 Million

Uses in Vicinity:				Mix of surface parking and multi-storied development 				including retail, office and restaurant with some upper 				story residential

III.	EXISTING CONDITIONS

The overall parcel includes two lots and consists of approximately 0.23 acres of land.   Both parcels are owned by one owner and therefore will be considered as one lot.   Parcel One is located at the north end (along Congress Street) of the property and comprises building that once held offices.  The property is 0.1028 acre and the total building space is approximately 33,035 sq. ft.   This building has a main entrance on Congress Street.  The building was built in 1947.

Lot 2 is bounded by Federal, Market and Exchange Streets.  The property is 0.1277 acre and the total building space is approximately 51,456 sq. ft.   A portion of the basement of this building lies below the sidewalk.  There is a main entrance of Exchange and Federal Streets.  The building was built in 1925.

The surrounding area is at the head of the Old Port District links to the Congress Street corridor.  Upper Exchange Street and lower Market Street is in the Pedestrian Activities District Overlay Zone.  Existing development on the opposite side of abutting streets includes a mix of surface parking and multi-storied development including retail, office and restaurant with some upper story residential.   Portland City Hall is across the street from the proposed site and Central Station is on the westerly side.    There are also office building and mixed use buildings with retail on the next block.    The surface parking lot adjacent to the former printing building will be used for parking for the proposed development.  The parking lot is across from Central Station.  The parking lot can be accessed through a tunnel under Congress Street to the building at 385 Congress Street or by crossing at the Congress/Market intersection.

The existing sidewalks along all the four streets are in brick with some street trees (mainly on Exchange Street).

Existing Licenses:  There are two licenses associated with these properties.  The first is a license granted by the City of Portland for the occupying under sidewalks on Exchange, Federal, and Market Streets.  The second license is for the installation and maintenance of a tunnel under Congress Street from the proposed building to 385 Congress Street.  Copies of the licenses will need to be submitted with the final application.  The following language on the survey explains it in more detail:

IV.	PROPOSED DEVELOPMENT

The applicant is proposing to convert the former Gannett Building and operate a 110-room luxury boutique hotel.   The property has maintained a prominent role in the City's Downtown District for almost a century as the home to Guy Gannett Publishing and the Portland Press Herald production facilities.  The development site is bounded by Congress, Exchange, Federal and Market Streets.   The project is a reuse of a former office high-rise property, which has been vacant for the past few years.  The development includes primarily interior refit construction to create the 110-room hotel layout.  The plans include the installation of the following:

- An 80-seat restaurant and kitchen on the first floor level (street grade).  The restaurant's primary access will be off Congress Street and the hotel entrance will be off Exchange Street.

- 110 room units on Floors 2-7.

- A pool area within the basement space.

The building's exterior will remain unchanged except for the renovation of the entrances on all public streets. Modifications to the hotel entrance off Exchange Street are also necessary to allow the entry to meet ADA requirements.  The entrance on Congress Street will be improved to provide a new Mahogany and glass entry door.  Similarly, a new Mahogany and glass door will be installed on the Market Street side where currently there were former overhead doors.  Finally, a new Mahogany and glass door will be installed at the existing Federal Street entrance.  The building is within the Congress Street Historic District and exterior changes must be reviewed by the Historic Preservation Board for a certificate of appropriateness.

The applicant is proposing a bump out of the curb line on the Exchange Street side.  The exterior work will also include waterproofing the basement space.  This will require removal and excavation of the existing sidewalk system around the perimeter of the building.  New brick sidewalks will be constructed as part of the waterproofing process.

V.	PUBLIC COMMENT

The neighborhood meeting for this project was held on October 11, 2012.   The minutes for the meeting are included in Attachment J.  As of the date of writing this report, no public comments have been submitted.

VI.	RIGHT, TITLE AND INTEREST AND FINANCIAL/TECHNICAL CAPACITY

a.	The owner of the property is MTM Portland Properties, LLC.  The applicant has provided a copy of sale and purchase agreement (Attachment G).   Right, title and interest in the property has not been submitted and will need to be submitted for the final review of the project.

b.         The estimated cost of the development is $ 8 Million.  The applicant has not submitted a financial capacity letter as demonstration of their financial capacity to complete the proposed development.

VII.	Zoning Assessment

A.	ZONING REVIEW- Marge Schmuckal

This project is to change the use of the previous Press Herald/Guy Gannett building. The proposed use is a 110 room hotel with amenities along with an 80 seat restaurant. A new elevator tower is proposed on the rear roof. The entire property is located within the B-3 Downtown Business Zone.  Both the hotel and restaurant are allowable uses in the B-3 zone. There is also a Historic overlay zone and a Pedestrian Activities Encouragement District and a Downtown Entertainment Overlay Zone on this property.  The DEOZ refers more to the dispersal of entertainment licenses through the City Clerk’s office.

Per the submitted elevations, the current building is approximately 77’ high for the portion of the building that is highest. The downtown height overlay map allows a maximum structure height of 125’ with a 90’ maximum street wall height. The current building will be meeting the current height regulations.

The B-3 zone does not require any parking to be shown for changes of use. The Historic District overly also does not require any more parking other than what may be currently on site.  However, it is understood that the hotel use will want to provide parking for their guests to be competitive.

All other B-3 zone requirements are being met.

Separate permits are required for any new signs. HVAC systems will need separate permits and will need to meet the maximum noise regulations within the B-3 zone. The applicants will be required to detail the emitting dBAs for the units being installed at the time of permitting.

B.	Site Plan Standards (Section 14-526)

The proposed development has been reviewed by staff for conformance with the relevant review standards of Portland’s site plan ordinance and applicable regulations.  Staff comments are listed below.

- Transportation Standards

The property is situated on a block that is surrounded by four different city streets.   To the north of the property is Congress Street, to the south is Federal Street, to the west is Exchange Street and to the east is Market Street.  The building has entrances on all four streets.   Congress Street is the main downtown street which permits traffic movement in both directions.  Exchange Street permits one way movement from Congress Street towards the Old Port, while Market Street allows one way movement towards Congress Street.   There are approximately six (6) parallel on-street parking on Exchange Street, three (3) parallel spaces on Federal Street and eleven (11) angled spaces on Market Street.  Four spaces on Exchange Street are metered while the remaining spaces are unmetered.  Parking meters are not located along a portion of the sidewalk due to the basement extends below the public sidewalk.   There is a license in place that the applicant refers to on the survey plan (see paragraph III above) but had not submit the actually document.  There is no parking on Congress Street in front of the site.

As part of the project proposal, the applicant is constructing a landscaped bump out at the main entry. The applicant does not state to the purpose for the bump out at this location and the City staff has requested more information.  Tom Errico, Consultant Traffic Engineer, has reviewed the submitted plans and offers the following comment in regard to the bump out:

The applicant needs to provide justification for the “bump-out” on Exchange Street. Minimizing the loss of on-street parking is important. I also do not believe the bump-out is necessary as a traffic calming strategy given existing traffic characteristics.

The applicant is also proposing to remove the on-street parking spaces from the lower portion of Exchange Street to the Federal Street for valet service. The removal of any on-street parking will require City Council approval.  Currently the on-street parking spaces on Market Street are head-in angle and the applicant has requested that these spaces be converted to back-in angle parking spaces.   Mr. Errico is in support of this change, but will need to evaluate effective means to implement and educate drivers on the re-orientation of the parking spaces.

The applicant is proposing to use the surface parking lot at the former printing press building at 385 Congress Street (across from Central Station).   The total number of parking spaces being leased is fifty (50) spaces (Attachment I).  The parking lot can be accessed through a tunnel under Congress Street to the building at 385 Congress Street or by crossing at the Congress/Market intersection.  In addition to these parking spaces, the applicant states that additional parking will be leased in the Temple Street garage.   The lease agreement for parking spaces at Temple Street garage has not been submitted.

The applicant has also requested waivers of the requirement for bicycle parking, motorcycles and scooter parking, transit facility.

Mr. Errico has the following comments in regard to the proposed project:

- The applicant is requesting a waiver for providing bicycle parking. In my professional opinion this project does not warrant a waiver from City standards.

- The proposed project will be providing 50 parking spaces at 385 Congress Street. Based upon similar type projects in the City, the proposed parking supply is not expected to meet the parking needs of a hotel with 110 rooms and an 80 seat restaurant.

- The applicant should provide information on truck deliveries including types of vehicles, times of unloading activity, and how they will be accommodated on the street system.

- The applicant has made a request to convert the head-in angle parking spaces to back-in angle parking spaces. I support this change and will need to think about implementation and driver education actions.

- The project will be upgrading sidewalks surrounding the building. Coordination with DPS on appropriate sidewalk ramp design and ADA compliance will need to take place.

- Parking regulation changes on abutting streets may need City Council action. If necessary, the applicant shall be responsible for providing all necessary materials for seeking a parking schedule change within the City Ordinance.

- Environmental Quality Standards

- Landscaping and Landscape Preservation

There are a few street trees on Exchange Street that applicant is proposing to keep.   The applicant is also proposing to landscape the bump out on Exchange Street.   Staff recommends street trees be proposed on all street frontages.  Jeff Tarling, City Arborist, review is pending and comments will be provided to the applicant.  There is no formal landscaping plan submitted.

The applicant is not proposing any landscaped island at the surface parking lot across the street.   The applicant is requesting a waiver of this standard.

- Water Quality, Storm Water Management and Erosion Control

The plans that have been submitted are preliminary.  David Senus, Consultant Engineer with Woodard and Curran, has reviewed the plans and offer comments as to what needs to be submitted to meet the stormwater standards, please see Attachment 3 for the full comments.

David Margolis-Pineo, Deputy Engineer with the Department of Public Services, has also reviewed the plans and is requesting more information about the drainage on site.  Below is his comment regarding stormwater:

- The applicant states that all stormwater runoff is proposed to discharge to the Fore River.  If so, please show how and redirect all roof water runoff to that system.

- Public Infrastructure and Community Safety Standards

- Sidewalks

The existing sidewalks along all four streets (Congress, Exchange, Federal and Market) are in brick.  Upper Exchange Street and lower Market Street is in the Pedestrian Activities District Overlay Zone.  As part of the waterproofing of the basement space, the work will require removal and excavation of the existing sidewalk system around the perimeter of the building.  New brick sidewalks will only be constructed on the potions that that is part of the waterproofing process.  Mr. Margolis-Pineo has recommended that all the sidewalks and ADA ramps on the four streets be replaced (see Attachment 4 for complete comments).

- Public Safety and Fire Prevention

Captain Chris Pirone of Fire Department has reviewed the submitted site plans and finds the proposed site plan to be acceptable

- Availability and Adequate Capacity of Public Utilities

The applicant will need to submit capacity letters for the proposed utilities as part of the final application.

- Site Design Standards

- Snow and Ice Loading

The site is located in the Portland Downtown District (PDD) and therefore PDD, along with the City, will be responsible for plowing of sidewalks.

- Historic Resources

The project site is located in the Congress Street Historic District and will have to be reviewed by the Historic Preservation Division.

- Exterior Lighting

A photometric plan and catalogue cuts for all proposed lighting fixtures must be submitted for the final plan.  The lighting fixtures will need to be reviewed by the Historic Preservation.

- Noise and Vibration

The applicant is proposing new HVAC and mechanical equipment.  The proposed noise levels for all the HVAC and mechanical equipment must be submitted for the final plan.

- Signage and Wayfinding

A signage and wayfinding plan must be submitted as part of the final review.

- Zoning Related Design Standards

The site is located in the Downtown Business B-3 Zone and in the Congress Street Historic District.   Since the site is a historic district, the Historic Preservation will be reviewing the exterior design of the building.

- NEXT STEPS

Next steps include:

- Address comments of the Transportation Engineering Reviewer Tom Errico

- Address comments from the Public Services Department

- Submit further information on the outstanding items mentioned in this memo

- Submit copy of the licenses’ with the City

- Address Planning Board comments

- Submit final application

ATTACHMENTS:

PLANNING BOARD MEMO ATTACHMENTS

- Marge Schmuckal Review

- Tom Errico Review

- David Senus Review

- David Margolis-Pineo Review

APPLICANT’S SUBMITTAL

- Written Description of Project

- Application

- Development Description

- Photos of Existing Site

- Maps

- Technical and Financial Capacity

- Purchase and Sale Agreement

- Conformity with Applicable Design Standards

- Parking Lease

- Neighborhood Meeting Minutes

PLANS

Plan 1	Cover Sheet

Plan 2	Alta/Acsm Land Title Survey

Plan 3	Existing Conditions Plan

Plan 4	Site Improvement Plan

Plan 5	Building Floor Plans

Plan 6	Elevations

Plan 7	Sub-Basement and Basement Floor Plans

Plan 8	Building Section

Plan 9	Elevator Floor Plan and Section