Level III Site Plan Review

Canal Plaza Hotel

433 Fore Street

Cow Plaza Hotel, LLC

BACKGROUND & REVIEWS

Cow Plaza Hotel, LLC is requesting a Level III site plan review for an 80,000 SF mixed-use development, the Canal Plaza Hotel.  The development site is located at the southwest corner of the Canal Plaza block, at the corner of Fore and Union Streets, directly across from the Old Port Historic District.   A CMP substation lies directly to the north, and the Fore Street parking garage is immediately east.  The site is currently occupied by a 45-space surface parking lot.

The proposed development includes:

- A 124-room hotel on seven floors

- 700 SF of retail space at the corner of Fore and Union Streets

- Pedestrian access via new sidewalk, doors on Fore Street

- Vehicular access via a porte cochere with openings onto Fore and Union Streets

- Parking at the adjacent garage with access via the mid-block alley on Union Street

- Minor landscaping improvements, consisting mainly of street trees

Project is subject to site plan review, including provisions related to historic district compatibility.  Project is also subject to design review under the Downtown Urban Design Guidelines.

MAJOR STAFF COMMENTS OF NOTE

Right, Title, and Interest

There is a CMP electrical duct bank on the property, although it does not actually lie in the existing easement.  The applicant has advised that this situation is being corrected with a new easement, and the project has been designed so that this new easement would lie under the porte-cochere.  Staff has requested evidence that CMP is amenable to this arrangement.  Additional easements, as well as some revocable licenses, will be required in order for the project to proceed.

Transportation

Separation waiver

The applicant has requested a waiver from the 150’ separation requirement for the driveways in the porte cochere area.  As proposed, the vehicular ingress via the porte cochere is 50’ from the intersection on Fore Street, and the egress is 75’ from the intersection on Union Street.

L turns into porte cochere

The applicant proposes to design the porte cochere to minimize left-turning movements.  However, there are concerns regarding the adequacy of these plans.  Queuing at this location could cause congestion at the intersection of Fore and Union Streets.  Mr. Errico suggested prohibiting left turns and geometric changes to restrict L turns altogether.

Stacking in porte cochere

There are concerns regarding stacking in the porte cochere at the entrance doors, which could cause congestion on Fore Street.   Mr. Errico has suggested crosswalk enhancements to differentiate this area and discourage stacking.

Service access

The alley above the CMP substation will serve as the service entrance for the hotel building.  Mr. Errico has asked for turning templates showing that this access will function effectively.

Ramp layout waiver

Pedestrian access is proposed via new brick sidewalks on Fore and Union Streets.  The doors to both the retail and the hotel front on Fore Street.  The applicant has requested a waiver for a diagonal ramp at the corner of Fore and Union Streets, given existing site constraints in this area.   Minimizing the radius at this corner has been suggested as a way to allow the preferred ramp alignment.

Parking

There is some ambiguity regarding the total parking demand, which is estimated in the Traffic Impact Study at between 100 and 170 spaces.  The city is asking for clarification, and Tom Errico has asked for the use of additional data sources in estimating parking demand.  Per the city’s zoning ordinance, only 31 spaces are required for the hotel (at one space/four rooms).  However, since the proposed building is over 50,000 SF, the Planning Board ultimately establishes the number of required parking spaces.

The applicant proposes to use the adjacent garage (which is under the same ownership) for parking purposes.  The application states that 218 of the 409 total spaces in the garage, currently occupied to some degree by month-to-month parkers, are currently available for this purpose.  It has come to the city’s attention that deed restrictions may exist at the Cow Plaza Hotel, which may limit the number of available spaces in that facility.  The city is asking for clarification on this arrangement.

Environmental Quality

Landscaping and Landscape Preservation

The proposal includes street trees on both Fore and Union Streets, as well as landscaped areas along the Fore Street façade and the service access.  Jeff Tarling, the city’s arborist, has requested additional landscaping near the in the non-window areas of the building and along the building frontages.

Site Design

Massing, Ventilation, and Wind Impact

The hotel is proposed as a seven-story building (up to the 65’ foot street wall limit of the height overlay zone, then step back 15 feet to accommodate a seventh floor within the 85’ maximum allowable height).  The applicant has requested that the tower at the Fore/Union Street corner extend past the 65’ height limit at the street wall.  This massing is similar to the Portland Harbor Hotel and 2 Portland Square, the buildings across Union.  However, it is significantly higher than the garage and those buildings across Fore St. in the historic district.

Historic Resources

The site is located adjacent to the Old Port Historic District.   As proposed, the building would be contemporary in design, with use of glass on the ground floor, a glass tower at the Fore Street/Union Street corner, and rain-screen paneling on all facades.  As of the writing of the Planning Board memo, elevations and renderings had not been provided, although they have been made available for the meeting today.  Deb Andrews, Historic Preservation Manager, and the Historic Preservation Board, will review these graphics for general compatibility with the Historic District under Section 14-526(d)5.b.  These comments will be provided to the Planning Board for final review.

Zoning-Related Design Standards

As the site is located in the B-3 zone, it is also subject to the Downtown Urban Design Guidelines.  Again, renderings were not available prior to the writing of this memo, and as such the city has not completed a comprehensive design review.  While graphics showing the proposed building have been provided here today, the staff recommends that the Board’s discussion of design should be deferred until the city is able to complete its design review process.

QUESTIONS/CLARIFICATION