PLANNING BOARD REPORT

PORTLAND, MAINE

Bayside Bowl Expansion

138 Kennebec Street

Level III Site Plan

Project ID #2015-230

BOPPO, LLC., Applicant.

Submitted to:  Portland Planning Board		Prepared by:  Rick Knowland, Senior Planner

Public Hearing Date:  January 26, 2016			Date:  January 22, 2016

- INTRODUCTION

A public hearing has been scheduled to consider an amendment to the Bayside Bowl site plan  approved by the Planning Board on June 9, 2015. The address of the site is 58 Alder Street which  occupies the entire block defined by Alder-Hanover- Kennebec-Lancaster Streets. The applicant is BoPo, LLC.  The development previously received  approvals for Site Plan  Review including Traffic Movement Permit and B-7 Design Standards. Certain waivers from the Technical and Design Manual were also granted.

The primary change in the site plan  is a reduction in the scope of the proposed building size and conversion of gravel parking to permanent parking.

View of site from corner of Alder St. and Kennebec St.

View of site from corner of Hanover St. and Lancaster St.

Notice of the public hearing includes notices sent to 88 area property owners  as well as those on the Interested Parties List. A notice appeared in the January 18, 2016 and January 19, 2016 editions of the Portland Press Herald.

- PROJECT DATA

|  | Previous Proposal | Proposed |
| --- | --- | --- |
| Zoning | B-7 | B-7 |
| Land Area | 55,290 SF | 55,290 SF |
| Proposed Use | Bowling Alley; Restaurant; Squash Courts | Bowling Alley; Restaurant |
| Existing Building Footprint | 13,600 SF | 13,600 SF |
| Proposed Building Footprint | 29,823 SF | 30,148 SF |
| Proposed Building Floor Area | 48,857 SF | 33,550 SF |
| Building Height | 45 FT | 34 FT |
| Parking Spaces | 36 on-site; xx off-site leased | 36 on-site; xx off-site leased |
| Project Cost | $2.4 million | $2,480,000 |

- BACKGROUND

On June 9, 2015 the Planning Board approved an expansion for the Bayside Bowl facility with a number of conditions. See attached approval letter (xxxxx). Since the approval, the applicant has modified the building program by eliminating the squash courts which has reduced the height and floor area of the building addition. The earlier planned expansion of  increasing the number of  bowling lanes from 12 to 20 remains the same.

On October 27, 2009, the Planning Board approved a site plan to establish the Bayside Bowl within the former Skillful Vending building at 58 Alder Street. The approval letter is shown as Attachment  A. The proposal at that time included 12 bowling lanes, 60 seat restaurant space, 33 parking spaces (7 onsite and 26 shared offsite) as well as various site improvements.

Applicant acquired a 23,450 sf vacant lot in 2011 adjacent to Bayside Bowl which extended the applicant’s landholdings to Kennebec Street. That same year applicant received administrative site plan approval for an outdoor seasonal patio for this parcel. With the prospect of the Somerset Street extension  looming in the future, certain improvements such as landscaping, sidewalks and drainage were deferred to the future. See Attachment B. The present submission proposes this area as a parking lot.

On March 17, 2015, the applicant completed a transaction with the City of Portland to purchase the “salt shed” parcel at 71 Hanover Street. This acquisition is intended to facilitate an expansion of the Bayside Bowl which abuts this property. With this property purchase the applicant now controls the entire Alder-Hanover-Kennebec- Lancaster block.

Lancaster Street between Alder and Hanover was discontinued by the City Council on May 1, 1995 to facilitate Public Services operations in this area. Based on that action, the applicant owns to the middle of the street although a public easement was retained. The Bayside Bowl expansion does not encroach within the right of way. The City owns the other half of the street which abuts the City’s Traffic Division building. By way of background, the  City evaluated a proposal last year by AVESTA to purchase the Traffic Division  property for a housing development.

The Somerset Street Extension Study has a direct impact on the Bayside Bowl site. The study is comprised of two components. The first part focuses on the design and construction of extending Somerset Street from Elm to Hanover Street while the second part evaluates various alignments of extending Somerset Street (Kennebec Street) and the Bayside Trail to Forest Avenue.

The submitted Bayside Bowl site plan incorporates the projected street alignment on the plan which crosses their property near the corner of Kennebec and Hanover. The proposed parking lot has been designed outside the projected alignment footprint. Engineering plans for the Elm Street to Hanover Street section are expected to be completed later in 2016.  Construction of  the Elm to Preble section is likely to be initiated in 2016 with the Preble and Hanover section likely to be completed a year or so later.

- DEVELOPMENT PROPOSAL

The primary change in the revised site plan is a reduction in building height and floor area reflecting  removal of the squash court use. The existing gravel area will be converted into a permanent parking lot.  Applicant proposes to construct a two story addition to the existing bowling alley. See site plan on following page of this memo. The expansion includes additional bowling alleys, a second floor mezzanine level  and a rooftop bar.  The addition increases the net floor area of the facility by  16,000 SF resulting in a total building floor area of  33,550 SF. The building addition will be 34 feet high rather the previous proposal of 45 feet.

The project expands the number of bowling alleys from 12 to 20. Restaurant space will be increased from 60 seats to 210 seats. The mezzanine level has a 171 person capacity but application indicates that it is “not expected to be fully occupied”. A partially covered 136 seat rooftop deck is proposed.

The existing public entrance on Alder Street remains and a second public entrance (facing Kennebec Street) will be incorporated into the addition.

The proposal fills in about two-thirds of  the Alder-Hanover-Kennebec- Lancaster block except for the parking area along Kennebec Street.

Parking includes a permanent 36 space lot on-site as well as references to a number of off-site parking lot resources although no agreement(s) documenting permission from applicable property owners has been provided. The 36 space parking lot was originally approved by the Board in 2015 as a gravel temporary parking lot pending enactment of a B-7 zoning amendment which allowed flexible parking lot setbacks. The amendment was approved by the City Council enabling Bayside Bowl (like the recently approved Schlotterbeck Foss site plan) to pursue a parking lot closer to the street line than otherwise permitted.

The 7 existing parking spaces on the Hanover Street side of the property are eliminated to accommodate the building addition footprint.

An 8 foot wide brick sidewalk is proposed along Hanover Street while the existing brick sidewalk along Alder Street is being extended about 22 feet but both sidewalks stop at the parking lot driveway in deference to the future alignment of Somerset Street which will likely be constructed within the next two years. The proposed alignment is shown on the plan below.

Three Bayside street light fixtures are proposed along Hanover Street.

Building Elevations

The original building elevations were approved without conditions. The updated building elevations look virtually identical to the original elevations except the building is about 11 feet shorter. See Attachment Plans. Kennebec Street and Hanover Street views are shown below. The proposed addition makes a major statement in overall building mass on the site. While the building has only  one full floor plus a mezzanine level, it reads as a taller building with its 34 foot height.   The building is surrounded by 4 streets so the building is viewed by the public from all 4 streets.

Updated Enlarged Rendering

- STAFF  DEVELOPMENT REVIEW

The proposed development  has been reviewed for conformance with the applicable review standards of Site Plan Ordinance, Traffic Movement Permit and B-7 Design Standards.

- B-7 Zoning

The site is located in the B-7 zone. Significant zoning review issues are highlighted below.

Height…The project site is located in District B of the Bayside Height Overlay Zone which requires a minimum height of 3 floors and a maximum height of 105 feet. The Bayside Bowl addition is 34 feet high down from the previous proposed 45 feet. The present proposal has a first  floor with a second floor mezzanine (equivalent to about 24 percent of a normal floor area) plus a roof deck bar which is short of the minimum height of 3 floors.  Assuming the project does not meet the minimum height standard, sec. 14-298 (h)(5) provides for an exception to the minimum height requirement - “such restriction shall not apply to those portions of the building addition(s) that are closer to the street line than the building footprint existing as (March 9, 2005)”. The proposed addition will be built closer to the street. Based on this exception the proposal does meet the minimum height requirement.

Surface Parking Setback…The B-7 zone requires new surface parking be located a minimum of 35 feet from the street [Sec. 14-295 (a)(25)]. The purpose of the language is to encourage buildings near the street line in creating an urban building edge along streets. The City Council approved an amendment which provides for flexibility from the 35 foot setback for converting small gravel parking areas to permanent parking that supports  a significant level of building development as exemplified by the Sclotterbeck Foss and Bayside Bowl site plans. The 2015 Bayside Bowl site plan could not meet the 35 foot setback thus the 36 space gravel lot was approved as a temporary parking lot pending City Council approval of the amendment.xxxxxxxxxxxxxxxxxxxxx

The enacted amendment is shown below with a response on Bayside Bowl’s compliance with the standards.

Sec. 14-295 (a) (26)  Notwithstanding Sections 14-295 (a) (24) and (25) above, surface parking that does not meet the 35-foot parking setback, provided that:

- All or a portion of the 35-foot setback area had a gravel surface on September 29, 2015;

Comment: The parking area had a gravel surface on September 29, 2015

- The total gravel surface area on the lot and any contiguous lots did not 15,000 square feet on September 29, 2015;

Comment: The gravel surface was less than 15,000 square feet on September 29, 2015.

- The parking spaces provide parking to a principle building on the same lot or a principle building on a contiguous lot:

Comment: The parking spaces provide parking to a principle building on the lot.

- One of the buildings described in paragraph c above meets the minimum height requirements of the Bayside Height Overlay Map and/or a building on the site has a floor area of  25,000 square feet or greater;

Comment: Bayside Bowl has a total floor area of over 30,000 square feet.

- The total number of spaces within the 35-foot setback in combination with other spaces on the lot does not exceed the minimum parking spaces required in Sec. 14-526 (a) (4) (a) (1);

Comment: The documented parking requirement for Bayside Bowl is over 100 spaces. The number of spaces within the 35 feet is well below that number.

- The proposed parking spaces meet the landscape and buffer requirements of 14-299 (f);

and:

Comment: The proposed parking lot does meet the landscape and buffer requirements of this section.

- Parking spaces within the 35-foot setback shall provide stormwater quality treatment if required by the City of Portland Stormwater Management Standards. If not required, an alternative low impact development treatment system approved by the Planning Board shall be provided.

Comment: An alternative low impact development treatment system has been provided

since this project is not required to provide stormwater treatment under the City of

Portland Stormwater Management Standards. David Senus, Development Review

Engineer, indicates the proposed alternative treatment system is acceptable.

xxxxxxxMindful of  the future Somerset Street extension project, the site plan approval  letter for the outdoor patio  allowed a 5 year window (to June 16, 2016) to complete site related improvements. See Attachment B. Note that sec. 14-522 of the Site Plan ordinance allows temporary parking lots for up to 1 year. Staff is therefore recommending the parking area be treated as a temporary parking lot with final review taking place by June 16, 2016.xxxxxxxxxx

Signs…A formal signage plan needs to be submitted.

- Site Plan Review Standards, Sec 14-526

The site plan standards language is summarized below in italics, with planning staff comment and analysis in regular font.

- Transportation

Impact on Surrounding Street Systems:

The development qualifies for a Traffic Movement Permit which was previously approved. The removal of the squash program from the development has reduced peak hour traffic demand from 147 trips to 100 trips. The parking study initially prepared by Gorrill-Palmer (Attachment xxxx)I-12) has been updated reflecting the downsizing of the project.xxxxxxxxx . Comments from Tom Errico, Traffic Review Consultant, are highlighted below and on Attachment D in their entirety. These comments incorporate relevant comments from his previous review as well as comments on the most recent submission.

- Construction Management Plan: The construction management plan is generally acceptable. I would note that the width of Lancaster Street will be significantly narrowed during construction and activities shall be coordinated with the City, given that it is used for DPS vehicle parking and circulation. Given the information provided, it is likely that contractor vehicles will utilize on-street parking in the area.  The applicant has noted that carpooling and other vehicle reduction strategies will be suggested.  I would recommended that this issue be discussed at the Pre-Construction meeting and strategies formalized to minimize impacts to on-street parking conditions. Construction will reduce the existing parking supply for the project.  The applicant should provide a plan for addressing this temporary parking issue.

- Traffic Movement Permit: Applicant has submitted requirements for a Traffic Movement Permit.  A Traffic Movement Permit Application has been submitted and a Traffic Study scoping meeting was held on May 15, 2015.  A traffic study supplement has been submitted that complies with discussions at the scoping meeting. The supplement notes that the full-build project will generate 147 trips during the PM peak hour. [An updated traffic demand analysis indicates a peak hour demand of 100 trips reflecting the elimination of the squash program from the development.] A capacity analysis was conducted at the Kennebec Street intersections with Hanover Street and Preble Street.  Both locations are estimated to work at excellent levels of service following project completion.  I find the project complies with MaineDOT Traffic Movement Permit requirements, with the Off-Site mitigation contributions noted below.

- Parking Demand: Applicant has identified a demand estimate of 134 parking spaces for the full-build project. Given the complexities of estimating parking generation for the project, this estimate seems reasonable (a significant effort was undertaken by the applicant to estimate traffic volumes for the project according to project site use and operations.  The PM peak hour traffic volume generation entering volume was used as the basis for the parking demand estimate).

- Parking Aisle Width: Plans propose a 24 foot wide aisle which is acceptable.

- On-street parking:  It is suggested that on-street parking be provided on the east side of Hanover Street along the property frontage. This action will require changes to the City's Traffic Schedule and will need to be approved by the TS&E Committee and City Council.  The applicant shall be responsible for providing materials in support of the TS&E and City Council packets.

- Monetary Contributions: Projects in the Bayside area have been required to make monetary contributions towards implementation of the Marginal Way Master Plan and the Somerset Street Extension Project.  Based upon the traffic study provided by the applicant and the "new" traffic added to the local street system the following contributions are required in conjunction with the MaineDOT TMP.

- Pursuant to Chapter 305, of the MDOT Rules and Regulations Pertaining to Traffic Movement Permits, the applicant shall make a $2,300 contribution towards implementation of the Marginal Way Master Plan.  This requirement is to address traffic issues at the Marginal Way intersections with Preble Street and Forest Avenue and general multi-modal improvements along the corridor.

- Pursuant to Chapter 305, of the MDOT Rules and Regulations Pertaining to Traffic Movement Permits, the applicant shall make a $6,600 contribution towards the implementation of the Somerset Street extension project. This requirement is to address traffic issues along Marginal Way, particularly at Forest Avenue, Preble Street, and Franklin Street.

- Sidewalk:  The short sidewalk section between the project driveway and Kennebec Street is bituminous and therefore I find conditions to be acceptable. I would note that a detectible warning panel shall be included at Kennebec Street.

The project is forecast to generate a total of 100 PM peak hour trip ends (85 entering/15 exiting). The study area for completing capacity analysis was identified early in the review process and included the intersections of Hanover Street/Kennebec Street and Preble Street/Kennebec Street. The capacity analysis completed for these intersections was adjusted to reflect the redistribution of generated traffic due to the use of area parking spaces vs. all traffic assumed on-site. This analysis was completed and the PM pre versus  PM post level of service  remained  unchanged at level of service A. The report states “ the proposed project is forecast to have minimal effect on the surrounding roadway network, and the surrounding roadway network is forecast to operate at a high level of service”.

Site Access and Circulation:

On-site vehicle circulation is limited to a proposed  36 space parking lot between Bayside Bowl and Kennebec Street. The parking lot has access on Alder Street and Hanover Street. Both driveway openings are 24 feet wide and the internal driveway aisle is 24 feet as well. Both dimensions were increased from the original plan to address concerns expressed by the traffic and fire review staff.

Staff is recommending the applicant install a guard rail, curb,  or other physical barrier to prevent cars from entering and exiting the parking lot from Kennebec Street since the street curb reveal is presently negligible.

Loading and Servicing:

Truck deliveries will take place near  the corner of  Lancaster Street and Hanover Street where an internal loading door is located.

Sidewalks:

An 8 foot wide brick sidewalk is proposed along Hanover Street while the existing brick sidewalk along Alder Street is being extended about 22 feet but both sidewalks stop at the parking lot driveway entrance rather than continuing along the lot frontage to Kennebec Street. This was in deference to the future alignment of Somerset Street which will likely be constructed within the next two years. Staff  is recommending that the Alder Street brick sidewalk be extended a distance of xxx feet to the Kennebec Street right-of-way. The Hanover Street sidewalk north of the driveway is shown as black top which Staff finds acceptable since some adjustments will need to be made when Somerset Street Extension is built.

A brick sidewalk was installed along the frontage of the Alder Street building as part of the previous review.

A sidewalk  is shown along the face of the building  (Kennebec side) and the parking lot which connects to Alder Street and Hanover Street.

Vehicle Parking:

Application is proposing a 36 on-site spaces as well as an indeterminate number of off-site shared use spaces. There is no parking requirement in the B-7 zone so the Planning Board determines the appropriate number based upon the applicant’s parking study and recommendation from the City Transportation Engineer which was determined to be appropriate during the 2015 review.

A parking location map in the original submission indicated there are 260 private spaces in the vicinity of Bayside Bowl and suggests that these spaces could be available for Bayside Bowl use. If these spaces are to be credited toward Bayside Bowl’s parking supply,  documentation of off-site parking leases needs to be submitted. The original application stated that “this project is fortunate to have its peak hour starting when most other surrounding uses are leaving for the day, leaving their parking areas vacant”.

Staff anticipates that on-street parking will be established along the Hanover Street side of the property.

Bicycle parking:

The site has two existing bike racks with capacity for 14 bikes. A 7 bike capacity rack will be added near the Kennebec Street building entrance. A new bike rack is proposed on the Kennebec Street side of the building having a 4 bike capacity. Based on the required number of vehicle parking spaces (134), the site plan standard would require a total of 21 bike spaces (2 spaces for every 10 vehicle spaces and 1 space for every 20 vehicle spaces). Therefore applicant needs to increase the number of bike spaces from 18 to 21.

Public Transit:

The site is not located along a public transit route.

Snow storage:

Potential snow storage areas include the landscaped area between the northerly edge of the parking lot and the Kennebec Street right of way.

TDM:

All development qualifying for Planning Board site plan review in the B-7 zone is required to submit a Transportation Demand Management plan. In  the 2015 review a TDM was not submitted but in the most recent submission a TDM was submitted. Staff comments were not available as of the writing of this report so the review of this document will remain a condition of approval.

B.    Environmental Quality Standards

### Preservation of Significant Natural Features:

### The site is located on a urban infill site which was disturbed many years ago and has no  significant vegetation or wildlife as contemplated by this standard.

### Landscaping:

A review of the plan indicates that 7 trees will be planted along the Hanover Street side of the property while 8 trees (of which 3 are relocated) will be planted along the edge of the building (Kennebec Street side). A variety of understory plantings are proposed.

A dumpster is proposed along the Lancaster Street side of the addition. The dumpster will be screened by a solid fence (non-stockade).

A note shows concept landscaping between the parking lot and Somerset Street but with no landscape species identified. The note states “landscape buffering to be determined subject to Somerset Street Extension”.  Staff is recommending that the planting plan be subject to further review  as part of the final site plan for the parking lot. Note that the location and species of the trees will need to be carefully considered since a storm drain swale is proposed within the landscape area. Existing trees along Alder Street were planted as part of the earlier review.

Water Quality, Stormwater Management and Erosion Control:

A stormwater management plan has been submitted. See Attachment I-13.

The site’s impervious surface has been reduced by 7,702 sf or 14 percent with the introduction of landscaped areas on the site. As such, the project is not required to include any specific stormwater management features for stormwater quantity  or quality. In general water quality is improved by the introduction of an underdrain vegetated swale  proposed adjacent to Kennebec Street. Most of the site’s stormwater will find its way to Alder Street  which has a separated sewer system.

Currently the salt shed portion of the site drains into Hanover Street which has a combined system. The development eliminates this source of contamination and reduces the amount of stormwater flowing into the combined system in Hanover Street.

The submission states that with a reduction in impervious area, stormwater runoff has been reduced by 6% to 36% depending on the storm event. The I inch storm is the most frequent and has been reduced by 36%.

An erosion and sedimentation control plan is shown on sheet 7 of the site plan.

David Senus, Development Review Engineer, finds that stormwater related issues have been addressed with the following conditions of approval. See Attachment E for his review memo.

- The Applicant is considering installing a roof runoff treatment system, and given that the treatment of roof runoff is not a requirement for their approval, is requesting that design information for this system be allowed to be provided prior to receiving a certificate of occupancy. We would consider this to be an acceptable approach, and would recommend including a condition of approval that states that if the Applicant elects to install a roof runoff treatment system, that design details and maintenance requirements for the system be submitted for review prior to granting a certificate of occupancy.  If the system is external to the building and requires changes to the site design, then design information shall be submitted in advance of performing the work.

.

- The Applicant has stated that they will file for a Notice of Intent to Comply with the Maine Construction General Permit and will file a copy with the City. The Applicant is requesting that receipt to of this Notice be made as a condition of approval.

David Margolis-Pineo, Deputy City Engineer, indicates that the  concept plan landscape plan for the green strip along Kennebec Street poses a potential conflict with the proposed infiltration under drain system. See Attachment F. Landscaping needs to be setback from the underdrain otherwise the vegetation may clog the underdrain.   This area of the site should be subject to final review when the permanent parking lot comes in for site plan review. Initially applicant is not proposing any plantings in this area.

C.   Public Infrastructure and Community Safety Standards	.

Consistency with City Master Plans:

Proposal appears consistent with applicable approved master plans.  The  parking lot has been designed to avoid the proposed new alignment for  Somerset Street extension. The project is compatible with the goal of extending the Bayside Trail to Deering Oaks as expressed in A New Vision For Bayside.

Public Safety and Fire Prevention:

Natural surveillance that promotes visibility of public spaces and areas.

The site is surrounded by 4 streets which promotes visibility of outdoor spaces.

Access control that promotes authorized and/or appropriate access to the site.

The interior circulation of the building provides for designated public entrances and service (non-public) doorways. Walkways are provided to public entrances and on-site parking provides for customer parking.

Territorial reinforcement that promotes a sense of ownership and responsibility through environmental design.

The design promotes territorial reinforcement through the use of site improvements that demarcate the edge of public spaces and private property.

Provide adequate emergency vehicle access to the site in accordance with City standards for street widths and turning radii, as described in Section 1 of the Technical Manual.

Access to the site for fire vehicles is acceptable according to Keith Gautreau of the Fire Department provided that the driveway width is 24 feet.

#### Be consistent with City public safety standards, Section 3 of the City of Portland Technical Manual, including but not limited to availability and adequacy of water supply and proximity of fire hydrants to structures.

#### An existing fire hydrant is located at the corner of Preble Street and Kennebec Street. The Portland Water District indicates there is sufficient water flow for fire protection services. Full comments of Keith Gautreau are shown as Attachment G.

### Availability and Adequate Capacity of Public Utilities:

### A letter has been received from the Portland Water District confirming their ability to serve this project. See Attachment IV-2. The District indicates that a recent test of a hydrant in the area indicated static pressure of 106 psi, residual pressure of 80 psi and water flow of 978 gpm which is adequate pressure and volume of water to serve domestic and fire protection water needs. The project will utilize an existing 6 inch water main in Alder Street that presently serves the site.

### Letters from Unitil (gas) and CMP (power) indicate their ability to serve the project. See Attachment I-6. The project intends to use existing utility lines from Alder Street that presently service the project.

### Applicant has requested a sanitary sewer capacity letter from Public Services but a response has not been received to date.

## D.   Site Design Standards

### Massing, Ventilation and Wind Impact:

The bulk, location or height of proposed buildings and structures shall not  result in health or safety problems from a reduction in ventilation to abutting structures or changes to existing wind climate that would result in unsafe wind conditions for users of the site and/or adjacent public spaces.

The nearest abutting structure is a minimum 50 feet away  so ventilation issues

should not be a concern. The building is 45 feet high which is unlikely to

result in unsafe wind conditions.

### The bulk, location or height of proposed buildings and structure shall minimize, to the extent feasible, any substantial diminution in the value or utility to neighboring structures under different ownership.

### The proposed infill project replaces a salt storage shed  with a 2.4 million dollar building investment, improves the urban streetscape of this block which should result in a positive contribution toward  the property values of  neighboring properties. The project does not appear to impact the utility of any neighboring properties.

### Development shall locate all HVAC venting mechanisms to direct exhaust way from public spaces and residential properties.

### HVAV units are proposed on the roof of the building and “are not visible from the street nor near any residential use.” Development locates all HVAC venting mechanisms to direct exhaust away from public spaces and residential properties directly adjacent to the site. HVAV units are proposed

### Shadows: N/A

### Snow and Ice Loading:

### The development appears to be designed to prevent significant amounts of accumulated snow and ice from falling onto adjacent properties or pubic ways. Snow storage areas are available along the Kennebec Street side of the property.

### View corridors:

### The site is not located within a city designated view corridor.

### Historic Resources:

### The site and any adjacent buildings are not designated as historic landmarks nor located within a designated historic district or within designated landscape districts.

### .	Exterior Lighting:

Site lighting consists of new pole mounted fixtures (four) in the parking lot, wall mounted fixtures located at egress locations and wall pac fixtures along the former Lancaster Street side of the addition. The existing building has small wall wash fixtures located along the Alder Street and Kennebec Street façade.

The Beacon Viper (small) pole mounted fixture and the Beacon Traverse (wall mounted) appear to have a cut-off feature. Four pole mounted light fixtures are proposed for the parking lot which will be mounted on 18 foot high poles. Nine wall mounted fixtures are proposed along the perimeter of the building addition.

A photometric plan was submitted in the original submission. See Attachment VI Plans (P2). For purposes of calculating photometric values, the  plan divides the site into 4 sub areas. The maximum photometric value (3 fc) for these areas is well below the allowable maximum of  5 fc. The parking lot value is slightly elevated at 1.26 fc but the site in general appears to be well below the maximum average standard of 1.25 fc.

Three Bayside street light fixtures are proposed along Hanover Street. The applicant previously installed street lights along Alder Street but a third light is needed north of the driveway. Staff is recommending this as a condition of approval.

Noise and Vibration:

HVAC units are located on the roof of the building.

Signage and Wayfinding:

A concept signage plan has been submitted. We are recommending that the final site plan be

submitted to Planning staff for review and approval.

Zoning Related Design Standards:   NA

Financial and Technical capacity:

Submission includes a letter from BNY Mellon Wealth Management

regarding applicant’s financial capacity to undertake this project (Attachment I-7). Technical

capacity is referenced as the following consultants that participated in preparing the

application: Mitchell & Associates (landscape architects and site planners), Ryan Senatore

(architecture), Ransom Consulting (engineers).

- B-7 Design Standards

Caitlin Cameron, Urban Designer, has conducted a design review of the project using the B-7 Mixed Use Urban District Zone Design Principles & Standards (Appendix 4 of the City of Portland Design Manual. The design review indicates the proposed design does pass the design standards. Below are comments regarding Standards reviewed in the previous workshop memo. These are the most significant standards for review in this project and the staff’s recommendations for findings are incorporated under each standard below.

Principle A: Urban Design

A-5: Pedestrian Environment – Development on public streets or public spaces shall . . . enhance the pedestrian environment through the use of elements at the first floor such as fully functioning entries oriented to the street; active windows and storefronts; awnings and weather protection; appropriately scaled streetlights; trees and landscaping; . . .

- The Planning Board finds the entrances as proposed are adequate and meet the requirement.

A-7: Building Orientation – The primary facades and entrances of buildings shall be oriented to streets, major pedestrian routes, or open spaces in order enhance the pedestrian-oriented environment. The primary facades and entrances of buildings shall not be oriented toward parking lots.

- The Planning Board finds the entrances are proposed are adequate and meet the requirement.

Principle B: Access and Circulation

B-6: Multi-modality – New development shall create a functional and safe environment that provides a continuous travel corridor for pedestrians and bicycles which serves the same major destinations as automobiles.

- The Planning Board finds the sidewalks as proposed are adequate and meet the requirement.

B-9: Streetscape Design – New development in the public realm shall utilize the City’s streetscape standards for Bayside which include specifications for sidewalks, streetlights, street furniture, fencing and walls, landscaping and signage in order to create a unified image of the neighborhood.

- The Planning Board finds the sidewalks as proposed are adequate and meet the requirement.

Principle C: Parking, Loading and Service Areas

C-7: Bike Racks – Bike racks shall be provided in a convenient location, proximate to the entry or entries of the building(s), either immediately adjacent to or no further than the associated motor vehicle parking, and shall be visible from the street or provided with prominent directional signage visible from the street as detailed in the Technical and Design Standards and Guidelines Manual and in compliance with the City’s Off-street bicycle parking standards

- Bike racks appear to be on private property. If any bike racks are proposed in the public right-of-way, they must conform to the Technical Manual in placement as well as type specification (Dero).   The Planning Board finds the bike racks as proposed are adequate and meet the requirement, subject to a condition that a site plan shall be revised for Planning Staff review and approval reflecting a minimum of 21 bike parking spaces on the site.

C-8: Service, Utility and Mechanical Infrastructure – Areas for outdoor storage and trash collection or compaction shall not be visible from public rights-of-way, or located within 20 feet of any public street, sidewalk, or open space.

- Materials for screening trash storage were revised to be a corresponding material choice to the rest of the building materials. The Planning Board finds the screening as proposed is adequate and meets the requirement.

Principle E: Architectural Design

E-6: Entrances – Buildings along public streets shall have the primary entrances oriented to the street. Primary entrances shall not be oriented to a parking lot or structure. . . . Commercial and mixed use buildings shall be permeable and accessible on all sides from the public way, unless the building program precludes such design.

- The Planning Board finds the entrances as proposed are adequate and meet the requirement.

E-7: Windows – Windows shall be located on all facades visible from public rights of way. . . . The first floor transparency (minimum visible transmittance (VT) of .7 or greater) along public streets and the trail shall be equal to at least 50% of the wall area between the height of 2 and 9 feet.

- Applicant chose not to add fenestration to the Dining/Lounge along Hanover Street area to increase the visibility of the active uses on the street.  The Planning Board finds that the revised exterior treatment along the facades visible from the public ways provides architectural interest and waives the standard for window. transparency.

E-12: Materials – Materials such as thing gauge metal panels . . . shall not be used on facades visible from public rights-of-way.

- Submission was revised according to Planning Board comments. Refer to the enlarged rendering in the applicant submission regarding material choices. The Planning Board finds that the revised exterior treatment along the facades visible from the public ways provides architectural interest  and that the materials are adequate and meet the requirement.

E-13: Transparency – Windows that have daylighting application on all levels of the façade shall use glass with a visible transmittance (VT) value of .7 or greater, which looks clear. . . . Opaque, heavily tinted or reflective glass shall not be used at the pedestrian level unless it can be demonstrated that the building program precludes the use of transparent glass.

- VT of .7 or greater is required by the standard. The Planning Board finds that the intended uses require walls and the revised exterior treatment along the facades visible from the public ways provides architectural interest and  thus, waives the standard for transparency.

E-16: Signage – Each building may have one sign per storefront tenant at the pedestrian level, one sign board at each entrance with a tenant roster, and a street number sign at entrances as necessary.

- Applicant will work with staff on the final placement of building signage. Staff recommends signage to the East-facing (Alder-facing) side of the primary building entry stair tower/vestibule to provide a strong presence and orientation to the street. The Planning Board finds the finds the proposed signage concept is  adequate and meet the requirement, subject to a condition a final signage plan be submitted for Planning Authority review.

- APPLICANT’S WAIVER REQUESTS

Applicant is requesting a number of  waivers that are more specifically described in Attachment  I-9. These waivers are summarized below. In addition, 5 “temporary waiver requests”  are listed which are not necessary.

Street Tree Requirement (Sec. 14-526 2.b. iii a and Technical Design Manual)…Submission indicates based on street frontage requirements 17 street trees are required. Twelve trees are proposed but due to pending Somerset Street project applicant requests a waiver and will provide the fee in lieu for the remaining 5…Staff supports this waiver.

Parking Lot and Parking Space (Technical Design Manual 1.14)…Applicant requests “a waiver to retain the existing gravel parking area until the final design of Somerset Street Extension has been determined” …Staff basically supports this waiver but has drafted a condition of approval that requires that a site plan be submitted for a permanent parking lot be submitted for review and approval by February 1, 2016.

Street Lighting (Technical Design Standards 10.4 Standards for Special Lighting District)…Request to waive street lighting fixtures along Kennebec Street frontage due to pending roadway improvements for Somerset Street Extension…Staff supports this waiver.

Travel Aisle Width…Request to reduce parking lot aisle width from 24 feet to 22 feet…Staff does not support this waiver. Applicant has attempted to accommodate the projected Somerset Street alignment (and shortened the parking aisles to 22 feet) but Staff believes a 24 foot aisle width can be accommodate on the site and is necessary for circulation.

Landscape Buffer (Technical Design Standards 4.5.5 Parking Areas…Request to waive the landscape buffer requirements around the surface parking until the final design of Somerset Street Extension has been determined…Staff basically supports this waiver. We are recommending a condition of approval that a final site plan be submitted for review and approval by February 1, 2016.

MOTIONS FOR THE BOARD TO CONSIDER

- Waivers

Street Trees

The Planning Board (finds/does not find) that the applicant has demonstrated that due to site constraints preventing the planting of required street trees in the right of way, the requirements of  Sec. 14-526(b)(3)(a) of the Site Plan Ordinance cannot be met; and the Board (finds/does not find) that the applicant has satisfied the waiver criteria set out in Section 14-526(b)(3)(b). The Planning Board therefore (waiver/does not waive) Section 14-526(b)(3)(a) Street Trees of the Site Plan Ordinance,  provided that the applicant shall contribute to the City of Portland Tree fund an amount proportionate to the cost of  the 5 required street trees.

Temporary Parking Lot

The Planning Board (finds/does not find) that  extraordinary conditions exist or undue hardship may result from strict compliance from, substantial justice and the public interest are secured, and the variation is consistent with the intent of the ordinance for a temporary parking lot to be used for less than one year, and therefore (waives/does not waive) Sec 14-526(4)(a)(5) for a permanent parking surface, provided a final site plan for the permanent parking lot is submitted by February 1, 2015 and that performance guarantee is provided to  cover the cost of the permanent improvements.

Street Lighting on Kennebec Street

The Planning Board (finds/does not find)  that installing new street lights along Kennebec Street in which the future alignment of that street has not been finalized at this time, would result in a situation in which extraordinary conditions exist or undue hardship may result from strict compliance, substantial justice and the public interest are secured with the variation, and therefore (waives/does not waive) the requirement of street lighting along Kennebec Street regarding Section 10 (Municipal Street Lighting Standards) of the Technical Manual.

Reduce Travel Aisle Width From 24 Feet to 22 Feet

The Planning Board (finds/does not find) that extraordinary conditions exist or undue hardship may result from strict compliance, substantial justice and the public interest secured with the variation, and therefore (waives/does not waive) the requirement of a 24 foot travel aisle (to 22 feet wide) for a parking lot regarding Section 1 (Parking Lot Design) of the Technical Manual.

Note: City Staff does not support this waiver request since we believe there is sufficient space on the site to accommodate a 24 foot travel aisle width.

Landscape Buffer for Parking Areas

The Planning Board (find/does not find) that planting required landscaping along Kennebec Street to buffer a temporary parking lot in which the future street alignment has not been finalized results in  extraordinary conditions exist or undue hardship from strict compliance, substantial justice and the public interest secured with the variation, and therefore (waiver/does not waive) the requirement of  parking lot landscape buffering regarding Standard 4.5.5 of the Technical Manual provided that a performance guarantee is submitted to the City to cover the cost of installing an appropriate landscape buffer for the future permanent parking lot.

- Traffic Movement Permit

On the basis of the application, plans, reports and other information submitted by the applicant, findings and recommendations contained in the Planning Board Report for application #2015-049 relevant to the Traffic Movement Permit, Site Plan reviews and other regulations, as well as the Planning Board deliberations and the testimony presented at the Planning Board hearing, the Planning Board finds the following:

That the plan (is/is not) in conformance with the standards of the Traffic Movement Permit, as reviewed by Thomas Errico, P.E. and comments submitted on June 4, 2015, subject to the following conditions of approval to be met prior to the issuance of a building permit unless otherwise stated:

- The applicant shall make a $3,400 contribution towards implementation of the Marginal Way Master Plan. This requirement is to address traffic issues at the Marginal Way intersections with Preble Street and Forest Avenue and general multi-global improvements along the corridor.

- The applicant shall make a $9,750 contribution towards the implementation of the Somerset Street extension project. This requirement is to address traffic issues along Marginal Way, particularly at Forest Avenue, Preble Street and Franklin Street.

- Site Plan Review

On the basis of the application (#2015-049), plans, reports and other information submitted by the applicant, findings and recommendations contained in the Planning Board Report for application #2015-049 relevant to the Site Plan Ordinance,  the MaineDEP Chapter 500 Stormwater Management Standards, the B-7 Design Standards as presented in the report, and other regulations, as well as the  Planning Board deliberations and the  testimony presented at the Planning Board hearing, the Planning Board finds the following:

That the plan (is/is not) in conformance with the site plan standards of the Land Use Code and DEP Stormwater Permit, subject to the following conditions of approval to be met prior to the issuance of a building permit unless otherwise stated:

- The proposed on-site parking lot is approved as a temporary parking lot for a period of one year to June 9, 2016.  Applicant shall submit a final site plan for a permanent parking lot for  Administrative site plan review and approval no later than  February 1, 2016. The final site plan shall incorporate all permanent infrastructure improvements such as pavement surfaces, landscaping, drainage improvements, guard rails or curb along the parking lot perimeter, lighting, sidewalks and other related improvements. Note that the plan shall provide an appropriate separation of vegetation from underdrain system. The performance guarantee for the 2015 Bayside Bowl site plan approval shall incorporate the value of permanent parking lot improvements in the performance guarantee amount  unless applicant submits a separate performance guarantee for the parking lot improvements.

- The applicant shall submit documentation of control of off-site parking spaces either by ownership or lease with a term of not less than five (5) years with an option to renew for a minimum of  98 spaces. Evidence of such control shall be required by showing for review and approval by Corporation Counsel, at a minimum, a signed letter of intent, purchase  and sale agreement, or option for sale or lease prior to the issuance of a building permit, and an executed deed or lease prior to issuance of any certificate of occupancy.

- The plan shall be revised reflecting a wood guard rail, curb stop, stones or other acceptable barrier along the Kennebec Street side  of the temporary parking lot for Planning Staff review and approval. Note that a  jersey barrier is not acceptable.

- As part of the pre-construction meeting process, applicant shall develop strategies and a plan for Public Services review and approval, to minimize impacts on on-street parking conditions during site construction.

- The site plan shall be revised for Planning Staff review and approval reflecting a minimum of 21 bike parking spaces on the site.

- Applicant shall be responsible for providing materials in support of TS&E and City Council packets regarding a proposal to add on-street parking along the applicant’s Hanover Street frontage which requires a change to the City’s Traffic Schedule.

- Pursuant to Chapter 305, of the MDOT Rules and Regulations Pertaining to Traffic Movement Permits, the applicant shall make a $3,400 contribution towards implementation of the Marginal Way Master Plan. This requirement is to address traffic issues at the Marginal Way intersections with Preble Street and Forest Avenue and general multi-global improvements along the corridor.

- Pursuant to Chapter 305, of the MDOT Rules and Regulations Pertaining to Traffic Movement Permits, the applicant shall make a $9,750 contribution towards the implementation of the Somerset Street extension project. This requirement is to address traffic issues along Marginal Way, particularly at Forest Avenue, Preble Street and Franklin Street.

- If Applicant elects to install a roof runoff treatment system, than design details and maintenance requirements for the system be submitted for  review and approval by David Senus (Development Review Engineer) prior to issuance of a certificate of occupancy.  If the system is external to the building and requires changes to the site design, then design information shall be submitted for review and approval in advance of performing the work.

- The plan shall be revised for City Staff review and approval reflecting a temporary bituminous asphalt sidewalk from the project driveway to Kennebec Street.

.

- Applicant shall forward to the Planning Department a copy of the filed Notice of Intent to Comply with the Maine DEP Maine Construction General Permit for the site.

- Applicant shall revise the plan reflecting  a Great Basin 250 which could be installed in the basement or to install an appropriately sized external grease interceptor for Public Services review and approval.

- Applicant shall be responsible for the costs of all public improvements along project frontage of Alder Street and Hanover Street to Somerset/Kennebec Street.

- A Transportation Demand Management Plan (TDM) shall be submitted for City Staff review and approval.

- That a final signage plan shall be submitted for Planning Staff review and approval prior to the issuance of a building permit.

ATTACHMENT LIST:

Staff Attachments

A.	Bayside Bowl Planning Board Approval Letter (dated 10-27-09)

B.	Bayside Bowl Administrative Approval Letter (dated 6-11-11)

C.	Memo from Caitlin Cameron, Urban Designer (dated 5-29-15)

D.	Memo from Tom Errico, Traffic Review Consultant (dated 6-4-15)

E.	Memo from David Senus, Development Review Consultant (5-29-15)

F.	Memo from David Margolis-Pineo, Deputy City Engineer (6-4-2015)

G.       Memo from Keith Gautreau, Fire Prevention (4-3-15)

I - Applicant Initial Submission	(March 23, 2015)

1.	Development Review Application and Checklist

2.	Right, Title or Interest

3.	Project Description and Project Data

4.	Tax Map

5.	Existing Soils and Condition

6.	Public Utilities

7.	Technical Capability, Financial Capability and Letter of Authorization

8.	Compliance with Applicable Zoning

9.	Waiver Request

10.	Consistency with City’s Master Plan and Conformity with Design

Standards

11.	Fire Department Check list and HVAC Emissions Requirements

12.	Traffic and Parking Study

13.	Stormwater Management Plan

14.	Solid Waste Disposal and Snow Removal

15.	Light Fixtures

16.	Construction Management Plan – to be determined

17.	Easements

II – Applicant Submission   (Response to Comments, May 28, 2015)

- Cover Letter

- Assessors Reference Property Card

- Traffic Movement Permit Supporting Material

III – Applicant Submission (Response to Comments, May 7, 2015)

- Cover Letter

IV - Applicant Submission  (Response to Comments, April 22, 2015)

1.	Cover Letter

2.	Portland Water District Ability to Serve Letter (4-10-15)

3.	Neighborhood Meeting Certification

4.	Catalogue Cuts

V – Plans (May 28, 2015)

P1.  Layout and Lighting Plan

P2.  Grading and Utilities Plan

P3.  Planting Plan

P4.  Site Details

P5.  Site Details

P6.  Erosion and Sedimentation Control

P7.  Phase 1 Construction Plan

VI – Plans (May 7, 2015

P1.  Updated Enlarged Facade Rendering

P2.  Photometric Plan

P3.  Grading and Drainage Plan – Somerset St ext

P4.  Layout and Lighting Plan

P5.  Planting Plan

P6.  Site Details

P7.  Site Details

P8.  Erosion and Sedimentation Control

VII - Plans	(April 22, 2015)

P1.	Location Map

P2.	First Floor Plan

P3.	Second Floor Plan

P4.	Mezzanine Level Plan

P5.	Roof Plan

P6.	Birdseye View

P7.	West Elevation

P8.	South Elevation

P9.	East Elevation

P10.	North Elevation

P11.	Layout and Utilities Plan

P12.	Erosion and Sedimentation Control Plan

P13.	Portland Water District Infrastructure Plan

P14.	Construction Management Plan

P15.	Renderings

VIII – Initial Plans	(March 23, 2015)

P1.	Location Map

P2.	Existing Conditions

P3.	Layout and Lighting Plan

P4.	Grading and Drainage Plan

P5.	Planting Plan

P6.	Site Details

P7.	Site Details

P8.	Stormwater Plan (Pre-Development)

P9.	Stormwater Plan (Post-Development)

P10.	First Floor Plan

P11.	Second Floor Plan

P12.	Mezzanine Level Plan

P13.	Roof Plan

P14.	West Elevation

P15.	South Elevation

P16.	East Elevation

P17.	North Elevation

P18.	Rendering 1 Existing

P19.	Rendering 1 Proposed

P20.	Rendering 2 Existing

P21.	Rendering 2 Proposed

P22.	Rendering 3 Existing

P23.	Rendering 3 Proposed

P24.	Rendering 4 Existing

P25.	Rendering 4 Proposed

P26.	Rendering 5

P27.	Stamped Survey