PLANNING BOARD REPORT

PORTLAND, MAINE

Bayside Bowl Expansion

138 Kennebec Street

Level III Site Plan

Project ID #2015-049

BOPPO, LLC., Applicant.

Submitted to:  Portland Planning Board		Prepared by:  Rick Knowland, Senior Planner

Public Hearing Date:  June 9, 2015			Date:  June 5, 2015

- INTRODUCTION

A public hearing has been scheduled to consider an application to expand the Bayside Bowl facility in the vicinity of 58 Alder Street. The site occupies the entire block defined by Alder-Hanover- Kennebec-Lancaster Streets. The applicant is BoPo, LLC. The development is subject to Site Plan Review including Traffic Movement Permit and B-7 Design Standards. Applicant is also requesting certain waivers from the Technical and Design Manual.

A workshop was held on April 28, 2015.

View of site from corner of Alder St. and Kennebec St.

View of site from corner of Hanover St. and Lancaster St.

Notice of this workshop includes notices sent to 90 area property owners  as well as those on the Interested Parties List. A notice appeared in the June  1, 2015 and June 2, 2015 editions of the Portland Press Herald.

- PROJECT DATA

Zoning:          				B-7

Land Area:    				55,290 sq. ft.

Proposed Use: 			Bowling Alley/Squash Courts/Restaurant

Existing Building Footprint:          	13,600 sq. ft.

Proposed Building Footprint:        	29,823 sq. ft.

Existing Building Floor Area:        	13,606 sq. ft.

Proposed Building Floor Area:      	48,857 sq. ft.

Building Height:                               	45 ft.

Parking Spaces:

Onsite:                                           	36

Offsite (leased):                            	indeterminate

Project Cost:                                 	$2.4 million

- BACKGROUND

On October 27, 2009, the Planning Board approved a site plan to establish the Bayside Bowl within the former Skillful Vending building at 58 Alder Street. The approval letter is shown as Attachment  A. The proposal at that time included 12 bowling lanes, 60 seat restaurant space, 33 parking spaces (7 onsite and 26 shared offsite) as well as various site improvements.

Applicant acquired a 23,450 sf vacant lot in 2011 adjacent to Bayside Bowl which extended the applicant’s landholdings to Kennebec Street. That same year applicant received administrative site plan approval for an outdoor seasonal patio for this parcel. With the prospect of the Somerset Street extension  looming in the future, certain improvements such as landscaping, sidewalks and drainage were deferred to the future. See Attachment B. The present 2015 submission proposes this area as a parking lot.

On March 17, 2015, the applicant completed a transaction with the City of Portland to purchase the “salt shed” parcel at 71 Hanover Street. This acquisition is intended to facilitate an expansion of the Bayside Bowl which abuts this property. With this property purchase the applicant now controls the entire Alder-Hanover-Kennebec- Lancaster block.

Lancaster Street between Alder and Hanover was discontinued by the City Council on May 1, 1995 to facilitate Public Services operations in this area. Based on that action, the applicant owns to the middle of the street although a public easement was retained. The Bayside Bowl expansion does not encroach within the right of way. The City owns the other half of the street which abuts the City’s Traffic Division building. By way of background, the City is in the process of evaluating  a proposal by AVESTA to purchase the Traffic Division  property for a housing development.

The Somerset Street Extension Study has a direct impact on the Bayside Bowl site. The study is comprised of two components. The first part focuses on the design and construction of extending Somerset Street from Elm to Hanover Street while the second part evaluates various alignments of extending Somerset Street (Kennebec Street) and the Bayside Trail to Forest Avenue.

The submitted Bayside Bowl site plan incorporates the projected street alignment on the plan which crosses their property near the corner of Kennebec and Hanover. The proposed parking lot has been designed outside the projected alignment footprint. Land acquisition and engineering plans for the Elm Street to Hanover Street section are expected to be completed later in the year with construction taking place in 2016.

- DEVELOPMENT PROPOSAL

Applicant proposes to construct a two story addition to the existing bowling alley. See site plan on following page of this memo. The expansion includes additional bowling alleys, a second floor area for squash courts and a rooftop bar. A mezzanine level is proposed to observe the squash courts. The addition increases the net floor area of the facility by  35,251 sf resulting in a total building floor area of 48,857 sf. The building addition is 45 feet high.

The project expands the number of bowling alleys from 12 to 20. Restaurant space will be increased from 60 seats to 210 seats. A partially covered 100 seat rooftop deck is proposed.

Squash courts will be run by Community Squash and includes 8 single squash courts, 1 doubles court, a small gym, class rooms and locker rooms. A 48 seat mezzanine will be provided to observe matches.

The existing public entrance on Alder Street remains and a second public entrance (facing Kennebec Street) will be incorporated into the addition.

The proposal fills in about two-thirds of  the Alder-Hanover-Kennebec- Lancaster block except for the parking area along Kennebec Street.

Parking includes a temporary 36 space lot on-site (gravel surface) as well as references to a number of off-site parking lot resources although no agreement(s) documenting permission from applicable property owners has been provided. The temporary parking lot would have formal review at a later date once the B-7 zoning parking lot issue for this site has been resolved and the alignment of Somerset Street extension has been finalized. See next section (Surface Parking Setback under B-7 Zoning). The 7 existing spaces on the Hanover Street side of the property are eliminated to accommodate the building addition footprint.

An 8 foot wide brick sidewalk is proposed along Hanover Street while the existing brick sidewalk along Alder Street is being extended about 22 feet but both sidewalks stop at the parking lot driveway in deference to the future alignment of Somerset Street which will likely be constructed in 2016. The proposed alignment is shown on the plan below. We would suggest that when the parking lot comes in for formal review in the future that the applicant be responsible for extending the sidewalk to meet the new sidewalk along Somerset  Street. Another alternative would be for the applicant to forward a check to the City in the amount of these improvements which could be incorporated into the Somerset Street construction program.

Three Bayside street light fixtures are proposed along Hanover Street.

Building Elevations

Building elevations have been submitted. See Attachment Plans. Kennebec Street and Hanover Street views are shown below. The proposed addition makes a major statement in overall building mass on the site. While the building has only functionally two floors (high ceiling height for squash courts) plus a mezzanine level, it reads as a much taller building with its 45 foot height.   The building is surrounded by 4 streets so the building is viewed by the public from all 4 streets.

Kennebec Street View

Hanover Street View

Since the Planning Board workshop applicant has submitted an enlarged rendering of the building façade as shown below. An accompanying narrative states:

“Based on the Planning Board’s request for more detail in the design and visual character we have modified the design as depicted in the attached rendering. We have eliminated the CMU masonry base and replaced it with a brick base which is in keeping with the existing Bayside Bowl Building and predominant material in the neighborhood. The brick adds a scale and texture that relates well to the pedestrian scale of the street. We have designed a custom flashing profile at the joints between siding panels, this profile has a recessed reveal which will add visual interest and a shadow line to add texture and rhythm to the façade. The rendering highlights the numerous siding profiles and colors on the façade to help scale the building to the street level experience…”

Updated Enlarged Rendering

- STAFF  DEVELOPMENT REVIEW

The proposed development  has been reviewed for conformance with the applicable review standards of Site Plan Ordinance, Traffic Movement Permit and B-7 Design Standards.

- B-7 Zoning

The site is located in the B-7 zone. Significant zoning review issues are highlighted below.

Height…The project site is located in District B of the Bayside Height Overlay Zone which requires a minimum height of 3 floors and a maximum height of 105 feet. The Bayside Bowl addition is 45 feet high but has only two functional floors which is short of the minimum height of 3 floors although a third mezzanine level is also proposed. Assuming the project does not neet the minimum height standard, sec. 14-298 (h)(5) provides for an exception to the minimum height requirement - “such restriction shall not apply to those portions of the building addition(s) that are closer to the street line than the building footprint existing as (March 9, 2005)”. The proposed addition will be built closer to the street. Based on this exception the proposal does meet the minimum height requirement.

Surface Parking Setback…The B-7 zone requires new surface parking be located a minimum of 35 feet from the street [Sec. 14-295 (a)(25)]. The purpose of the language is to encourage buildings near the street line in creating an urban building edge along streets. The applicant’s proposal features parking along the Kennebec Street side of the lot, most of which are within 35 feet of the existing Kennebec Street right-of-way. The proposed parking area is currently an unimproved lot. The 35 foot setback standard would not apply if the parking lot was grandfathered but staff has not seen any information substantiating a grandfathered parking use for this lot. Applicant has submitted information on this issue but staff  feels it does not sufficiently substantiate a grandfathering of the parking lot.

Applicant is proposing the parking area shown on the site plan be considered a temporary parking lot until a zoning text amendment is implemented to address this issue. There are several reasons for this path. The projected alignment for Somerset Street extension  has not been finalized. This project  requires a “taking” of a portion of the Bayside Bowl property as well as another nearby  property. The site plan on the prior page shows the projected alignment of Somerset Street extension. Once the taking has been finalized (which will be later in the year) we will have a better idea of the relationship of the roadway to the parking lot setback.  In addition, this same issue has occurred on other development sites, where the majority of the lot is redeveloped and there are limited options for parking.

Mindful of  the future Somerset Street extension project, the site plan approval  letter for the outdoor patio  allowed a 5 year window (to June 16, 2016) to complete site related improvements. See Attachment B. Note that sec. 14-522 of the Site Plan ordinance allows temporary parking lots for up to 1 year. Staff is therefore recommending the parking area be treated as a temporary parking lot with final review taking place by June 16, 2016.

Over the next year staff will draft an amendment (narrow in scope) to address the parking setback issue unless the applicant is successful in documenting a grandfathered parking use. Bayside Bowl has a set of circumstances which make it unique to other properties in the B-7 which would be reflected in the amendment. In concept  the amendment would allow some flexibility, but require measures supporting the Bayside Vision such as a multi-story building and on-site surface parking not exceeding the normal zoning requirement.

Signs…A formal signage plan needs to be submitted.

- Site Plan Review Standards, Sec 14-526

The site plan standards language is summarized below in italics, with planning staff comment and analysis in regular font.

- Transportation

Impact on Surrounding Street Systems:

The development qualifies for a Traffic Movement Permit. An initial traffic and parking study was prepared by Gorrill-Palmer (Attachment I-12) which was most recently updated on May 28th (Attachment II-3). Comments from Tom Errico, Traffic Review Consultant, are highlighted below and on Attachment D in their entirety.

- Construction Management Plan: The construction management plan is generally acceptable. I would note that the width of Lancaster Street will be significantly narrowed during construction and activities shall be coordinated with the City, given that it is used for DPS vehicle parking and circulation. Given the information provided, it is likely that contractor vehicles will utilize on-street parking in the area.  The applicant has noted that carpooling and other vehicle reduction strategies will be suggested.  I would recommended that this issue be discussed at the Pre-Construction meeting and strategies formalized to minimize impacts to on-street parking conditions. Construction will reduce the existing parking supply for the project.  The applicant should provide a plan for addressing this temporary parking issue.

- Traffic Movement Permit: Applicant has submitted requirements for a Traffic Movement Permit.  A Traffic Movement Permit Application has been submitted and a Traffic Study scoping meeting was held on May 15, 2015.  A traffic study supplement has been submitted that complies with discussions at the scoping meeting. The supplement notes that the full-build project will generate 147 trips during the PM peak hour. A capacity analysis was conducted at the Kennebec Street intersections with Hanover Street and Preble Street.  Both locations are estimated to work at excellent levels of service following project completion.  I find the project complies with MaineDOT Traffic Movement Permit requirements, with the Off-Site mitigation contributions noted below.

- Parking Demand: Applicant has identified a demand estimate of 134 parking spaces for the full-build project. Given the complexities of estimating parking generation for the project, this estimate seems reasonable (a significant effort was undertaken by the applicant to estimate traffic volumes for the project according to project site use and operations.  The PM peak hour traffic volume generation entering volume was used as the basis for the parking demand estimate).

- Parking Aisle Width: Supports a 24 foot wide traffic aisle not a waiver that would allow a 22 foot wide width.

- On-street parking:  It is suggested that on-street parking be provided on the east side of Hanover Street along the property frontage. This action will require changes to the City's Traffic Schedule and will need to be approved by the TS&E Committee and City Council.  The applicant shall be responsible for providing materials in support of the TS&E and City Council packets.

- Monetary Contributions: Projects in the Bayside area have been required to make monetary contributions towards implementation of the Marginal Way Master Plan and the Somerset Street Extension Project.  Based upon the traffic study provided by the applicant and the "new" traffic added to the local street system the following contributions are required in conjunction with the MaineDOT TMP.

- Pursuant to Chapter 305, of the MDOT Rules and Regulations Pertaining to Traffic Movement Permits, the applicant shall make a $3,400 contribution towards implementation of the Marginal Way Master Plan.  This requirement is to address traffic issues at the Marginal Way intersections with Preble Street and Forest Avenue and general multi-modal improvements along the corridor.

- Pursuant to Chapter 305, of the MDOT Rules and Regulations Pertaining to Traffic Movement Permits, the applicant shall make a $9,750 contribution towards the implementation of the Somerset Street extension project. This requirement is to address traffic issues along Marginal Way, particularly at Forest Avenue, Preble Street, and Franklin Street.

- Sidewalk:  The short sidewalk section between the project driveway and Kennebec Street needs to be ADA compliant and therefore should not be gravel. I have spoken to DPS about this and they concur.  Given the uncertainty of the reconstruction of Kennebec Street, I would suggest that it be bituminous asphalt.

The project is forecast to generate a total of 147 PM peak hour trip ends (134 entering/13 exiting). The study area for completing capacity analysis was identified early in the review process and includes the intersections of Hanover Street/Kennebec Street and Preble Street/Kennebec Street. At the scoping meeting, it was requested that the previous capacity analysis completed for these intersections be updated to reflect the redistribution of generated traffic due to the use of area parking spaces vs. all traffic assumed on-site. This analysis was completed and the PM pre versus  PM post level of service  remained  unchanged at level of service A. The report states “ the proposed project is forecast to have minimal effect on the surrounding roadway network, and the surrounding roadway network is forecast to operate at a high level of service”.

Site Access and Circulation:

On-site vehicle circulation is limited to a proposed  36 space parking lot between Bayside Bowl and Kennebec Street. The parking lot has access on Alder Street and Hanover Street. Both driveway openings are 20 feet wide and the internal driveway aisle is 22 feet as well. Although these dimensions were drawn in deference to the future Somerset Street extension, the traffic and fire review staff are recommending that both dimensions be increased to 24 feet to meet the minimum driveway standard. There appears  to be adequate space on the site to accommodate this change.

Staff is recommending the applicant install a guard rail, curb,  rocks or other physical barrier to prevent cars from entering and exiting the parking lot from Kennebec Street since the street curb reveal is presently negligible. Applicant is proposing a jersey barrier as temporary although this seems like an extreme measure. While a curb or some other barrier will be required during the review of the permanent parking lot, at a minimum a  temporary measure needs to be installed on an interim basis.

Loading and Servicing:

Truck deliveries will take place near  the corner of  Lancaster Street and Hanover Street where an internal loading door is located.

Sidewalks:

An 8 foot wide brick sidewalk is proposed along Hanover Street while the existing brick sidewalk along Alder Street is being extended about 22 feet but both sidewalks stop at the parking lot driveway entrance rather than continuing along the lot frontage to Kennebec Street. This was in deference to the future alignment of Somerset Street which will likely be constructed in 2016. As discussed previously we are recommending that when the parking lot comes in for  formal administrative review in the future that the applicant be responsible for extending the sidewalk to meet the new sidewalk along Somerset  Street.

A brick sidewalk was installed along the frontage of the Alder Street building as part of the previous review.

A sidewalk  is shown along the face of the building  (Kennebec side) and the parking lot which connects to Alder Street and Hanover Street.

Vehicle Parking:

Application is proposing a 36 temporary on-site spaces as well as an indeterminate number of off-site shared use spaces. Presumably the temporary spaces will be converted to permanent spaces in the future. There is no parking requirement in the B-7 zone so the Planning Board determines the appropriate number based upon the applicant’s parking study and recommendation from the City Transportation Engineer.

A parking location map in the submission indicates there are 260 private spaces in the vicinity of Bayside Bowl and suggests that these spaces could be available for Bayside Bowl use. If these spaces are to be credited toward Bayside Bowl’s parking supply,  documentation of off-site parking leases needs to be submitted.

The latest submission states: the proposed Bayside Bowl expansion is forecast to generate 134 PM peak hour trip ends arriving to use the proposed expanded facility…Subtracting the 36 on-site available parking spaces, there is a total of approximately 224 potentially available parking spaces. As discussed at the scoping meeting, this project is fortunate to have its peak hour starting when most other surrounding uses are leaving for the day, leaving their parking areas vacant.”

Staff anticipates that on-street parking will be established along the Hanover Street side of the property.

Bicycle parking:

The site has two existing bike racks with capacity for 14 bikes. A new bike rack is proposed on the Kennebec Street side of the building having a 4 bike capacity. Based on the required number of vehicle parking spaces (134), the site plan standard would require a total of 21 bike spaces (2 spaces for every 10 vehicle spaces and 1 space for every 20 vehicle spaces). Therefore applicant needs to increase the number of bike spaces from 18 to 21.

Public Transit:

The site is not located along a public transit route.

Snow storage:

Potential snow storage areas include the landscaped area between the northerly edge of the parking lot and the Kennebec Street right of way.

TDM:

All development qualifying for Planning Board site plan review in the B-7 zone is required to submit a Transportation Demand Management plan. Applicant did not submit one so this will need to be a condition of approval.

B.    Environmental Quality Standards

### Preservation of Significant Natural Features:

### The site is located on a urban infill site which was disturbed many years ago and has no  significant vegetation or wildlife as contemplated by this standard.

### Landscaping:

A review of the plan indicates that 7 trees will be planted along the Hanover Street side of the property while 8 trees (of which 3 are relocated) will be planted along the edge of the building (Kennebec Street side). A variety of understory plantings are proposed.

A dumpster is proposed along the Lancaster Street side of the addition. The dumpster will be screened by a solid fence (non-stockade).

A note shows concept landscaping between the parking lot and Somerset Street but with no landscape species identified. The note states “landscape buffering to be determined subject to Somerset Street Extension”.  Staff is recommending that the planting plan be subject to further review  as part of the final site plan for the parking lot. Note that the location and species of the trees will need to be carefully considered since a storm drain swale is proposed within the landscape area. Existing trees along Alder Street were planted as part of the earlier review.

Water Quality, Stormwater Management and Erosion Control:

A stormwater management plan has been submitted. See Attachment I-13.

The site’s impervious surface has been reduced by 7,702 sf or 14 percent with the introduction of landscaped areas on the site. As such, the project is not required to include any specific stormwater management features for stormwater quantity  or quality. In general water quality is improved by the introduction of an underdrain vegetated swale  proposed adjacent to Kennebec Street. Most of the site’s stormwater will find its way to Alder Street  which has a separated sewer system.

Currently the salt shed portion of the site drains into Hanover Street which has a combined system. The development eliminates this source of contamination and reduces the amount of stormwater flowing into the combined system in Hanover Street.

The submission states that with a reduction in impervious area, stormwater runoff has been reduced by 6% to 36% depending on the storm event. The I inch storm is the most frequent and has been reduced by 36%.

An erosion and sedimentation control plan is shown on sheet 7 of the site plan.

David Senus, Development Review Engineer, finds that stormwater related issues have been addressed with the following conditions of approval. See Attachment E for his review memo.

- The Applicant is considering installing a roof runoff treatment system, and given that the treatment of roof runoff is not a requirement for their approval, is requesting that design information for this system be allowed to be provided prior to receiving a certificate of occupancy. We would consider this to be an acceptable approach, and would recommend including a condition of approval that states that if the Applicant elects to install a roof runoff treatment system, that design details and maintenance requirements for the system be submitted for review prior to granting a certificate of occupancy.  If the system is external to the building and requires changes to the site design, then design information shall be submitted in advance of performing the work.

.

- The Applicant has stated that they will file for a Notice of Intent to Comply with the Maine Construction General Permit and will file a copy with the City. The Applicant is requesting that receipt to of this Notice be made as a condition of approval.

David Margolis-Pineo, Deputy City Engineer, indicates that the  concept plan landscape plan for the green strip along Kennebec Street poses a potential conflict with the proposed infiltration under drain system. See Attachment F. Landscaping needs to be setback from the underdrain otherwise the vegetation may clog the underdrain.   This area of the site should be subject to final review when the permanent parking lot comes in for site plan review. Initially applicant is not proposing any plantings in this area.

C.   Public Infrastructure and Community Safety Standards	.

Consistency with City Master Plans:

Proposal appears consistent with applicable approved master plans.  The  parking lot has been designed to avoid the proposed new alignment for  Somerset Street extension. The project is compatible with the goal of extending the Bayside Trail to Deering Oaks as expressed in A New Vision For Bayside.

Public Safety and Fire Prevention:

Natural surveillance that promotes visibility of public spaces and areas.

The site is surrounded by 4 streets which promotes visibility of outdoor spaces.

Access control that promotes authorized and/or appropriate access to the site.

The interior circulation of the building provides for designated public entrances and service (non-public) doorways. Walkways are provided to public entrances and on-site parking provides for customer parking.

Territorial reinforcement that promotes a sense of ownership and responsibility through environmental design.

The design promotes territorial reinforcement through the use of site improvements that demarcate the edge of public spaces and private property.

Provide adequate emergency vehicle access to the site in accordance with City standards for street widths and turning radii, as described in Section 1 of the Technical Manual.

Access to the site for fire vehicles is acceptable according to Keith Gautreau of the Fire Department provided that the driveway width is 24 feet.

#### Be consistent with City public safety standards, Section 3 of the City of Portland Technical Manual, including but not limited to availability and adequacy of water supply and proximity of fire hydrants to structures.

#### An existing fire hydrant is located at the corner of Preble Street and Kennebec Street. The Portland Water District indicates there is sufficient water flow for fire protection services. Full comments of Keith Gautreau are shown as Attachment G.

### Availability and Adequate Capacity of Public Utilities:

### A letter has been received from the Portland Water District confirming their ability to serve this project. See Attachment IV-2. The District indicates that a recent test of a hydrant in the area indicated static pressure of 106 psi, residual pressure of 80 psi and water flow of 978 gpm which is adequate pressure and volume of water to serve domestic and fire protection water needs. The project will utilize an existing 6 inch water main in Alder Street that presently serves the site.

### Letters from Unitil (gas) and CMP (power) indicate their ability to serve the project. See Attachment I-6. The project intends to use existing utility lines from Alder Street that presently service the project.

### Applicant has requested a sanitary sewer capacity letter from Public Services but a response has not been received to date.

## D.   Site Design Standards

### Massing, Ventilation and Wind Impact:

The bulk, location or height of proposed buildings and structures shall not  result in health or safety problems from a reduction in ventilation to abutting structures or changes to existing wind climate that would result in unsafe wind conditions for users of the site and/or adjacent public spaces.

The nearest abutting structure is a minimum 50 feet away  so ventilation issues

should not be a concern. The building is 45 feet high which is unlikely to

result in unsafe wind conditions.

### The bulk, location or height of proposed buildings and structure shall minimize, to the extent feasible, any substantial diminution in the value or utility to neighboring structures under different ownership.

### The proposed infill project replaces a salt storage shed  with a 2.4 million dollar building investment, improves the urban streetscape of this block which should result in a positive contribution toward  the property values of  neighboring properties. The project does not appear to impact the utility of any neighboring properties.

### Development shall locate all HVAC venting mechanisms to direct exhaust way from public spaces and residential properties.

### HVAV units are proposed on the roof of the building and “are not visible from the street nor near any residential use.” Development locates all HVAC venting mechanisms to direct exhaust away from public spaces and residential properties directly adjacent to the site. HVAV units are proposed

### Shadows: N/A

### Snow and Ice Loading:

### The development appears to be designed to prevent significant amounts of accumulated snow and ice from falling onto adjacent properties or pubic ways. Snow storage areas are available along the Kennebec Street side of the property.

### View corridors:

### The site is not located within a city designated view corridor.

### Historic Resources:

### The site and any adjacent buildings are not designated as historic landmarks nor located within a designated historic district or within designated landscape districts.

### .	Exterior Lighting:

Site lighting consists of new pole mounted fixtures (four) in the parking lot, wall mounted fixtures located at egress locations and wall pac fixtures along the former Lancaster Street side of the addition. The existing building has small wall wash fixtures located along the Alder Street and Kennebec Street façade.

The Beacon Viper (small) pole mounted fixture and the Beacon Traverse (wall mounted) appear to have a cut-off feature. Four pole mounted light fixtures are proposed for the parking lot which will be mounted on 18 foot high poles. Nine wall mounted fixtures are proposed along the perimeter of the building addition.

A photometric plan has been submitted. See Attachment VI Plans (P2). For purposes of calculating photometric values, the  plan divides the site into 4 sub areas. The maximum photometric value (3 fc) for these areas is well below the allowable maximum of  5 fc. The parking lot value is slightly elevated at 1.26 fc but the site in general appears to be well below the maximum average standard of 1.25 fc.

Three Bayside street light fixtures are proposed along Hanover Street. The applicant previously installed street lights along Alder Street but a third light is needed north of the driveway. Presumably this may be incorporated into the review of the permanent  parking lot.

Noise and Vibration:

HVAC units are located on the roof of the building.

Signage and Wayfinding:

A concept signage plan has been submitted. We are recommending that the final site plan be

submitted to Planning staff for review and approval.

Zoning Related Design Standards:   NA

Financial and Technical capacity:

Submission includes a letter from BNY Mellon Wealth Management

regarding applicant’s financial capacity to undertake this project (Attachment I-7). Technical

capacity is referenced as the following consultants that participated in preparing the

application: Mitchell & Associates (landscape architects and site planners), Ryan Senatore

(architecture), Ransom Consulting (engineers).

- B-7 Design Standards

Caitlin Cameron, Urban Designer, has conducted a design review of the project using the B-7 Mixed Use Urban District Zone Design Principles & Standards (Appendix 4 of the City of Portland Design Manual. The design review indicates the proposed design does pass the design standards. Below are comments regarding Standards reviewed in the previous workshop memo. These are the most significant standards for review in this project and the staff’s recommendations for findings are incorporated under each standard below.

Principle A: Urban Design

A-5: Pedestrian Environment – Development on public streets or public spaces shall . . . enhance the pedestrian environment through the use of elements at the first floor such as fully functioning entries oriented to the street; active windows and storefronts; awnings and weather protection; appropriately scaled streetlights; trees and landscaping; . . .

- The Planning Board finds the entrances as proposed are adequate and meet the requirement.

A-7: Building Orientation – The primary facades and entrances of buildings shall be oriented to streets, major pedestrian routes, or open spaces in order enhance the pedestrian-oriented environment. The primary facades and entrances of buildings shall not be oriented toward parking lots.

- The Planning Board finds the entrances are proposed are adequate and meet the requirement.

Principle B: Access and Circulation

B-6: Multi-modality – New development shall create a functional and safe environment that provides a continuous travel corridor for pedestrians and bicycles which serves the same major destinations as automobiles.

- The Planning Board finds the sidewalks as proposed are adequate and meet the requirement.

B-9: Streetscape Design – New development in the public realm shall utilize the City’s streetscape standards for Bayside which include specifications for sidewalks, streetlights, street furniture, fencing and walls, landscaping and signage in order to create a unified image of the neighborhood.

- The Planning Board finds the sidewalks as proposed are adequate and meet the requirement.

Principle C: Parking, Loading and Service Areas

C-7: Bike Racks – Bike racks shall be provided in a convenient location, proximate to the entry or entries of the building(s), either immediately adjacent to or no further than the associated motor vehicle parking, and shall be visible from the street or provided with prominent directional signage visible from the street as detailed in the Technical and Design Standards and Guidelines Manual and in compliance with the City’s Off-street bicycle parking standards

- Bike racks appear to be on private property. If any bike racks are proposed in the public right-of-way, they must conform to the Technical Manual in placement as well as type specification (Dero).   The Planning Board finds the bike racks as proposed are adequate and meet the requirement, subject to a condition that a site plan shall be revised for Planning Staff review and approval reflecting a minimum of 21 bike parking spaces on the site.

C-8: Service, Utility and Mechanical Infrastructure – Areas for outdoor storage and trash collection or compaction shall not be visible from public rights-of-way, or located within 20 feet of any public street, sidewalk, or open space.

- Materials for screening trash storage were revised to be a corresponding material choice to the rest of the building materials. The Planning Board finds the screening as proposed is adequate and meets the requirement.

Principle E: Architectural Design

E-6: Entrances – Buildings along public streets shall have the primary entrances oriented to the street. Primary entrances shall not be oriented to a parking lot or structure. . . . Commercial and mixed use buildings shall be permeable and accessible on all sides from the public way, unless the building program precludes such design.

- The Planning Board finds the entrances as proposed are adequate and meet the requirement.

E-7: Windows – Windows shall be located on all facades visible from public rights of way. . . . The first floor transparency (minimum visible transmittance (VT) of .7 or greater) along public streets and the trail shall be equal to at least 50% of the wall area between the height of 2 and 9 feet.

- Applicant chose not to add fenestration to the Dining/Lounge along Hanover Street area to increase the visibility of the active uses on the street.  The Planning Board finds that the revised exterior treatment along the facades visible from the public ways provides architectural interest and waives the standard for window. transparency.

E-12: Materials – Materials such as thing gauge metal panels . . . shall not be used on facades visible from public rights-of-way.

- Submission was revised according to Planning Board comments. Refer to the enlarged rendering in the applicant submission regarding material choices. The Planning Board finds that the revised exterior treatment along the facades visible from the public ways provides architectural interest  and that the materials are adequate and meet the requirement.

E-13: Transparency – Windows that have daylighting application on all levels of the façade shall use glass with a visible transmittance (VT) value of .7 or greater, which looks clear. . . . Opaque, heavily tinted or reflective glass shall not be used at the pedestrian level unless it can be demonstrated that the building program precludes the use of transparent glass.

- VT of .7 or greater is required by the standard. The Planning Board finds that the intended uses require walls and the revised exterior treatment along the facades visible from the public ways provides architectural interest and  thus, waives the standard for transparency.

E-16: Signage – Each building may have one sign per storefront tenant at the pedestrian level, one sign board at each entrance with a tenant roster, and a street number sign at entrances as necessary.

- Applicant will work with staff on the final placement of building signage. Staff recommends signage to the East-facing (Alder-facing) side of the primary building entry stair tower/vestibule to provide a strong presence and orientation to the street. The Planning Board finds the finds the proposed signage concept is  adequate and meet the requirement, subject to a condition a final signage plan be submitted for Planning Authority review.

- APPLICANT’S WAIVER REQUESTS

Applicant is requesting a number of  waivers that are more specifically described in Attachment  I-9. These waivers are summarized below. In addition, 5 “temporary waiver requests”  are listed which are not necessary.

Street Tree Requirement (Sec. 14-526 2.b. iii a and Technical Design Manual)…Submission indicates based on street frontage requirements 17 street trees are required. Twelve trees are proposed but due to pending Somerset Street project applicant requests a waiver and will provide the fee in lieu for the remaining 5…Staff supports this waiver.

Parking Lot and Parking Space (Technical Design Manual 1.14)…Applicant requests “a waiver to retain the existing gravel parking area until the final design of Somerset Street Extension has been determined” …Staff basically supports this waiver but has drafted a condition of approval that requires that a site plan be submitted for a permanent parking lot be submitted for review and approval by February 1, 2016.

Street Lighting (Technical Design Standards 10.4 Standards for Special Lighting District)…Request to waive street lighting fixtures along Kennebec Street frontage due to pending roadway improvements for Somerset Street Extension…Staff supports this waiver.

Travel Aisle Width…Request to reduce parking lot aisle width from 24 feet to 22 feet…Staff does not support this waiver. Applicant has attempted to accommodate the projected Somerset Street alignment (and shortened the parking aisles to 22 feet) but Staff believes a 24 foot aisle width can be accommodate on the site and is necessary for circulation.

Landscape Buffer (Technical Design Standards 4.5.5 Parking Areas…Request to waive the landscape buffer requirements around the surface parking until the final design of Somerset Street Extension has been determined…Staff basically supports this waiver. We are recommending a condition of approval that a final site plan be submitted for review and approval by February 1, 2016.

MOTIONS FOR THE BOARD TO CONSIDER

- Waivers

Street Trees

The Planning Board (finds/does not find) that the applicant has demonstrated that due to site constraints preventing the planting of required street trees in the right of way, the requirements of  Sec. 14-526(b)(3)(a) of the Site Plan Ordinance cannot be met; and the Board (finds/does not find) that the applicant has satisfied the waiver criteria set out in Section 14-526(b)(3)(b). The Planning Board therefore (waiver/does not waive) Section 14-526(b)(3)(a) Street Trees of the Site Plan Ordinance,  provided that the applicant shall contribute to the City of Portland Tree fund an amount proportionate to the cost of  the 5 required street trees.

Temporary Parking Lot

The Planning Board (finds/does not find) that  extraordinary conditions exist or undue hardship may result from strict compliance from, substantial justice and the public interest are secured, and the variation is consistent with the intent of the ordinance for a temporary parking lot to be used for less than one year, and therefore (waives/does not waive) Sec 14-526(4)(a)(5) for a permanent parking surface, provided a final site plan for the permanent parking lot is submitted by February 1, 2015 and that performance guarantee is provided to  cover the cost of the permanent improvements.

Street Lighting on Kennebec Street

The Planning Board (finds/does not find)  that installing new street lights along Kennebec Street in which the future alignment of that street has not been finalized at this time, would result in a situation in which extraordinary conditions exist or undue hardship may result from strict compliance, substantial justice and the public interest are secured with the variation, and therefore (waives/does not waive) the requirement of street lighting along Kennebec Street regarding Section 10 (Municipal Street Lighting Standards) of the Technical Manual.

Reduce Travel Aisle Width From 24 Feet to 22 Feet

The Planning Board (finds/does not find) that extraordinary conditions exist or undue hardship may result from strict compliance, substantial justice and the public interest secured with the variation, and therefore (waives/does not waive) the requirement of a 24 foot travel aisle (to 22 feet wide) for a parking lot regarding Section 1 (Parking Lot Design) of the Technical Manual.

Note: City Staff does not support this waiver request since we believe there is sufficient space on the site to accommodate a 24 foot travel aisle width.

Landscape Buffer for Parking Areas

The Planning Board (find/does not find) that planting required landscaping along Kennebec Street to buffer a temporary parking lot in which the future street alignment has not been finalized results in  extraordinary conditions exist or undue hardship from strict compliance, substantial justice and the public interest secured with the variation, and therefore (waiver/does not waive) the requirement of  parking lot landscape buffering regarding Standard 4.5.5 of the Technical Manual provided that a performance guarantee is submitted to the City to cover the cost of installing an appropriate landscape buffer for the future permanent parking lot.

- Traffic Movement Permit

On the basis of the application, plans, reports and other information submitted by the applicant, findings and recommendations contained in the Planning Board Report for application #2015-049 relevant to the Traffic Movement Permit, Site Plan reviews and other regulations, as well as the Planning Board deliberations and the testimony presented at the Planning Board hearing, the Planning Board finds the following:

That the plan (is/is not) in conformance with the standards of the Traffic Movement Permit, as reviewed by Thomas Errico, P.E. and comments submitted on June 4, 2015, subject to the following conditions of approval to be met prior to the issuance of a building permit unless otherwise stated:

- The applicant shall make a $3,400 contribution towards implementation of the Marginal Way Master Plan. This requirement is to address traffic issues at the Marginal Way intersections with Preble Street and Forest Avenue and general multi-global improvements along the corridor.

- The applicant shall make a $9,750 contribution towards the implementation of the Somerset Street extension project. This requirement is to address traffic issues along Marginal Way, particularly at Forest Avenue, Preble Street and Franklin Street.

- Site Plan Review

On the basis of the application (#2015-049), plans, reports and other information submitted by the applicant, findings and recommendations contained in the Planning Board Report for application #2015-049 relevant to the Site Plan Ordinance,  the MaineDEP Chapter 500 Stormwater Management Standards, the B-7 Design Standards as presented in the report, and other regulations, as well as the  Planning Board deliberations and the  testimony presented at the Planning Board hearing, the Planning Board finds the following:

That the plan (is/is not) in conformance with the site plan standards of the Land Use Code and DEP Stormwater Permit, subject to the following conditions of approval to be met prior to the issuance of a building permit unless otherwise stated:

- The proposed on-site parking lot is approved as a temporary parking lot for a period of one year to June 9, 2016.  Applicant shall submit a final site plan for a permanent parking lot for  Administrative site plan review and approval no later than  February 1, 2016. The final site plan shall incorporate all permanent infrastructure improvements such as pavement surfaces, landscaping, drainage improvements, guard rails or curb along the parking lot perimeter, lighting, sidewalks and other related improvements. Note that the plan shall provide an appropriate separation of vegetation from underdrain system. The performance guarantee for the 2015 Bayside Bowl site plan approval shall incorporate the value of permanent parking lot improvements in the performance guarantee amount  unless applicant submits a separate performance guarantee for the parking lot improvements.

- The applicant shall submit documentation of control of off-site parking spaces either by ownership or lease with a term of not less than five (5) years with an option to renew for a minimum of  98 spaces. Evidence of such control shall be required by showing for review and approval by Corporation Counsel, at a minimum, a signed letter of intent, purchase  and sale agreement, or option for sale or lease prior to the issuance of a building permit, and an executed deed or lease prior to issuance of any certificate of occupancy.

- The plan shall be revised reflecting a wood guard rail, curb stop, stones or other acceptable barrier along the Kennebec Street side  of the temporary parking lot for Planning Staff review and approval. Note that a  jersey barrier is not acceptable.

- As part of the pre-construction meeting process, applicant shall develop strategies and a plan for Public Services review and approval, to minimize impacts on on-street parking conditions during site construction.

- The site plan shall be revised for Planning Staff review and approval reflecting a minimum of 21 bike parking spaces on the site.

- Applicant shall be responsible for providing materials in support of TS&E and City Council packets regarding a proposal to add on-street parking along the applicant’s Hanover Street frontage which requires a change to the City’s Traffic Schedule.

- Pursuant to Chapter 305, of the MDOT Rules and Regulations Pertaining to Traffic Movement Permits, the applicant shall make a $3,400 contribution towards implementation of the Marginal Way Master Plan. This requirement is to address traffic issues at the Marginal Way intersections with Preble Street and Forest Avenue and general multi-global improvements along the corridor.

- Pursuant to Chapter 305, of the MDOT Rules and Regulations Pertaining to Traffic Movement Permits, the applicant shall make a $9,750 contribution towards the implementation of the Somerset Street extension project. This requirement is to address traffic issues along Marginal Way, particularly at Forest Avenue, Preble Street and Franklin Street.

- If Applicant elects to install a roof runoff treatment system, than design details and maintenance requirements for the system be submitted for  review and approval by David Senus (Development Review Engineer) prior to issuance of a certificate of occupancy.  If the system is external to the building and requires changes to the site design, then design information shall be submitted for review and approval in advance of performing the work.

- The plan shall be revised for City Staff review and approval reflecting a temporary bituminous asphalt sidewalk from the project driveway to Kennebec Street.

.

- Applicant shall forward to the Planning Department a copy of the filed Notice of Intent to Comply with the Maine DEP Maine Construction General Permit for the site.

- Applicant shall revise the plan reflecting  a Great Basin 250 which could be installed in the basement or to install an appropriately sized external grease interceptor for Public Services review and approval.

- Applicant shall be responsible for the costs of all public improvements along project frontage of Alder Street and Hanover Street to Somerset/Kennebec Street.

- A Transportation Demand Management Plan (TDM) shall be submitted for City Staff review and approval.

- That a final signage plan shall be submitted for Planning Staff review and approval prior to the issuance of a building permit.

ATTACHMENT LIST:

Staff Attachments

A.	Bayside Bowl Planning Board Approval Letter (dated 10-27-09)

B.	Bayside Bowl Administrative Approval Letter (dated 6-11-11)

C.	Memo from Caitlin Cameron, Urban Designer (dated 5-29-15)

D.	Memo from Tom Errico, Traffic Review Consultant (dated 6-4-15)

E.	Memo from David Senus, Development Review Consultant (5-29-15)

F.	Memo from David Margolis-Pineo, Deputy City Engineer (6-4-2015)

G.       Memo from Keith Gautreau, Fire Prevention (4-3-15)

I - Applicant Initial Submission	(March 23, 2015)

1.	Development Review Application and Checklist

2.	Right, Title or Interest

3.	Project Description and Project Data

4.	Tax Map

5.	Existing Soils and Condition

6.	Public Utilities

7.	Technical Capability, Financial Capability and Letter of Authorization

8.	Compliance with Applicable Zoning

9.	Waiver Request

10.	Consistency with City’s Master Plan and Conformity with Design

Standards

11.	Fire Department Check list and HVAC Emissions Requirements

12.	Traffic and Parking Study

13.	Stormwater Management Plan

14.	Solid Waste Disposal and Snow Removal

15.	Light Fixtures

16.	Construction Management Plan – to be determined

17.	Easements

II – Applicant Submission   (Response to Comments, May 28, 2015)

- Cover Letter

- Assessors Reference Property Card

- Traffic Movement Permit Supporting Material

III – Applicant Submission (Response to Comments, May 7, 2015)

- Cover Letter

IV - Applicant Submission  (Response to Comments, April 22, 2015)

1.	Cover Letter

2.	Portland Water District Ability to Serve Letter (4-10-15)

3.	Neighborhood Meeting Certification

4.	Catalogue Cuts

V – Plans (May 28, 2015)

P1.  Layout and Lighting Plan

P2.  Grading and Utilities Plan

P3.  Planting Plan

P4.  Site Details

P5.  Site Details

P6.  Erosion and Sedimentation Control

P7.  Phase 1 Construction Plan

VI – Plans (May 7, 2015

P1.  Updated Enlarged Facade Rendering

P2.  Photometric Plan

P3.  Grading and Drainage Plan – Somerset St ext

P4.  Layout and Lighting Plan

P5.  Planting Plan

P6.  Site Details

P7.  Site Details

P8.  Erosion and Sedimentation Control

VII - Plans	(April 22, 2015)

P1.	Location Map

P2.	First Floor Plan

P3.	Second Floor Plan

P4.	Mezzanine Level Plan

P5.	Roof Plan

P6.	Birdseye View

P7.	West Elevation

P8.	South Elevation

P9.	East Elevation

P10.	North Elevation

P11.	Layout and Utilities Plan

P12.	Erosion and Sedimentation Control Plan

P13.	Portland Water District Infrastructure Plan

P14.	Construction Management Plan

P15.	Renderings

VIII – Initial Plans	(March 23, 2015)

P1.	Location Map

P2.	Existing Conditions

P3.	Layout and Lighting Plan

P4.	Grading and Drainage Plan

P5.	Planting Plan

P6.	Site Details

P7.	Site Details

P8.	Stormwater Plan (Pre-Development)

P9.	Stormwater Plan (Post-Development)

P10.	First Floor Plan

P11.	Second Floor Plan

P12.	Mezzanine Level Plan

P13.	Roof Plan

P14.	West Elevation

P15.	South Elevation

P16.	East Elevation

P17.	North Elevation

P18.	Rendering 1 Existing

P19.	Rendering 1 Proposed

P20.	Rendering 2 Existing

P21.	Rendering 2 Proposed

P22.	Rendering 3 Existing

P23.	Rendering 3 Proposed

P24.	Rendering 4 Existing

P25.	Rendering 4 Proposed

P26.	Rendering 5

P27.	Stamped Survey