SimplexGrinnell

20 Thomas Dr.

Westbrook, ME  04092

Tel: 207-842-6440

PROPOSED INSTALLATION

Customer 			Local Sprouts Cafe

Address 			649 Congress St..

Portland, ME  04101

Hazard location 		Main 1st fl. kitchen.  Hood 1A; 11’-6” & Hood 1B: 12’

System location 	Cylinder and controls located at exterior exit @12’ to right side of the hoods as you’re facing the hoods.

System Type 	Pyro-Chem,  UL300 wet chemical per NGPA 17A & 96 & design manual UL EX 3830 (dated 3/1/06)

System size			6-gallon (1 cylinder), KNII model PCL-600

Remarks 	A.  System includes: (1) 6-gallon cylinder, (14) discharge nozzles, (6) fusible link detectors, (1) pull station, (1) 1-1/2” mechanical gas valve and micro-switches for fire alarm system connection .

B.  Nozzles include: (4) 1H’s in plenums, (2) 2D’s in ducts and (1) 2H, (2) 1H’s, (3) 2L’s and (2) 1L’s for appliances

Installation date 	Installation pending in May 2010.

Next inspection due		Semi-annual:  Nov. 2010, May. 2011 etc.

Installed by			Installation by Roger Blanchette