From: 	"Steve Bushey" <SBushey@DelucaHoffman.com>

To:	"Kandi Talbot (E-mail)" <KCOTE@ci.portland.me.us>

Date: 	Fri, Oct 12, 2001  3:14 PM

Subject: 	Maine Medical Center Parking Lot- 932 congress St. 2001-0264

Kandi,

I have reviewed the site plan application dated 9/20/01 prepared by

Sebago Technics on behalf of Maine Medical Center.  I offer the

following comments for your consideration.

1.  The application includes only a brief summary of runoff conditions

for the project.  I recommend that complete drainage computations be

submitted outlining the anticipated runoff conditions to the new system

that will connect to the street as well as the runoff amounts to

adjacent properties.  The engineer should provide data regarding the

capacity of the street system and determine if the street system is part

of a combined sewer or not.  Finally, this project will have more than

25 parking spaces, therefore the applicant should provide evidence that

adequate water quality treatment will be provided to the paved area

runoff.  Given the size of the parking lot I suggest at least a 50% TSS

removal efficiency for any BMP measures installed for the project.  The

MEDEP sliding scale graph suggests greater than 65% removal.

2.  Is the applicant providing any type of shelter at the shuttle stop

since they have stated that the buses will run every half hour?

Granted, users may stay in their car and wait for the bus therefore this

may not be necessary.  Or they can walk to the hospital since it is only

15 min. or less up the hill.

3.  The Public works dept. should comment on the need to reconstruct the

driveway entrance onto Congress St. since the parking lot traffic will

be substantially increased.  It is rather narrow currently.

4.  Given the increase of traffic in and out of the site, is a traffic

study warrented?  Larry Ash should review for sight lines, accessiblity,

signage etc.  I recommend signage be placed on the site to clarify that

left turns out only are allowed since Congress St. is one way.

5.  The engineer should review the grading within the parking lot and

try to increase the slopes to at least 1%.  Several areas are less than

1%.

6.  A snow storage plan should be provided.

7.  Staff and Jeff Tarling should review the adequacy of the Arborvitae

buffer along the southwest property line.  6' tall arborvitae are

generally thin and in my opinion will not sufficiently buffer the

neighboring houses from the increased activity that will occur in the

lot.

8.  Additional information should be provided regarding the drainage

conditions on the properties on the northwest and north sides of the

site.

9.  Is it intended that the White ash be only 24" high?

10.  A photometrics plan and should be provided.  Will the lights be on

all night?  What, if any, security provisions are being proposed.

If you have any questions please call.

Steve Bushey

Technical Reviewer