PLANNING BOARD REPORT

PORTLAND, MAINE

New Yard at Canal Landing

Boatyard and Boat Repair Facility

40 West Commercial Street

Amendment to

Level III Site Plan, Conditional Use, Shoreland Zone, Flood Plain,

New Yard, LLC, Applicant

| Submitted to: Portland Planning Board: Public Hearing Date: August 27, 2013 Project # 2013-201, amending #2012-575 CBL: 59-A-1,2,3,4,5,6,7,8,9,10,11,12 | Prepared by: Bill Needelman, Senior Planner Date: August 21,2013 Planning Board Report Number: #40-13 |
| --- | --- |

#### I.	Introduction

At the request of New Yard, LLC, represented by Steve Bushey, FST Engineers, the Planning Board is requested to hold a Public Hearing to review amendments to an approved site plan for a boat and ship repair and maintenance facility in the vicinity of 40 West Commercial Street.

In December 2012, the “New Yard at Canal Landing” was approved by the Planning Board for approximately 42,000 square feet of building space for a boat repair and storage facility with re-grading of up to7 acres of the site.  This approval is currently valid, but improvements on the site to date have been limited to brush clearing, concrete crushing, removal of derelict pilings and wharf structure, and general housekeeping.  Erosion and sedimentation control measures have been put in place in conjunction with this work. The approval included permits for Level III Site Plan, Site Location of Development, Conditional Use in the WPDZ, Shoreland Zoning, and Flood Plain Management.

The applicant is requesting an amendment to the approved plan to allow phasing the development.  Originally, the applicant was approved for a “Phase 1” including two boat ramps, a travel lift basin, floating docks, buildings totaling over 42,000 square feet of industrial space, significant fill and grading, stormwater improvements, utility work, lighting, and landscaping.  For the Board’s information, the applicant also showed a “Phase 2” full build out scenario for the site for future review.

With this application, the first intended “Phase 1a” of development would allow the applicant to provide only minimal improvements to support the launching, hauling and storage of vessels along with conducting limited associated repairs.  No permanent buildings are proposed in “Phase 1a.”  The previous approvals would remain in effect for ”Phase 1b” for up to three years from the original date of approval.  A condition of approval is suggested confirming that all previous conditions of approval remain in effect for the full Phase 1 build out.

Phasing Rationale:  The applicant makes this phasing request to allow time for continuing negotiations with the Maine Department of Transportation (MDOT) and the tenants of the adjacent International Marine Terminal (IMT). As has been reported in local news media, the operators and tenants of the IMT are a contemplating a westerly expansion of freight operations onto the New Yard property.  Mr. Phin Sprague, Principal of New Yard, LLC, would like to move forward with the boatyard project, while not making higher value improvements that may need to change to accommodate expansion of the IMT operations.

If the applicant’s and MDOT’s negotiations are successful, both the IMT and the New Yard project will present significant amendment proposals to the Planning Board showing an expansion of the IMT and/or associated activities, and a westerly relocation of all or a portion of the New Yard activities.  Such amendments may or may not include acquisition of additional rail land.

If the negotiations do not move forward, Mr. Sprague will have the option of completing the New Yard project on the basis of the existing plans and approvals.

The Planning Board is asked to review this material without a workshop due to the recent review and approval of the full project late in 2012, and the limited nature of the improvements proposed.

This hearing was advertised in the August 19 and 20, 2013 editions of the Portland Press Herald and was noticed by mail to 152 neighboring property owners and interested parties.

City staff has reviewed the application and written materials and the staff’s analysis is provided below.

Applicable Reviews

The original project was considered a Level III Site Plan subject to review under the Waterfront Port Development Zone, Shoreland, Flood Plain, and Conditional Use (marine retail and boat storage in the WPDZ) standards of the land use code.  The project was also reviewed under delegated local authority for a Site Location of Development permit.  The previous approval also included various waivers and conditions of approval, all of which remain in effect.  Staff has provided a copy of the previous approval letter and Planning Board report (attachments omitted) for the Board’s reference.  This report will concentrate on standards review and analysis applied only to the proposed “Phase 1a.”

II.	Project Data                          	Phase 1A Only

Total Site Acreage:			22.5 acres

Total Proposed Disturbed area:	3.8 acres+/-

Zone:			Waterfront Port Development Zone

Existing Uses:			Mostly Vacant.  Propane distribution and rail sidings

Proposed Use:			Boat repair, storage,

Continued propane distribution

Predevelopment Impervious Area: 	287,704 sq ft

Post-development Impervious Area: 	255,598 sq ft

Percentage Impervious Area, total:	26%

Proposed Building Footprint: 		0 sq ft

Existing Total Building Floor Area:  	0 sq ft

Proposed Total Building Floor Area:	0 sq ft

Required Vehicle Parking:			0

Proposed Vehicle Parking:			As needed in yard area

Required Bicycle Parking:			0

Proposed Bicycle Parking:			0

III.	Right, Title and Interest

The applicant has included a new deed for land formally owned by the Portland Terminal Company (Pan Am) in Attachment D.  The former gas plant parcels are subject to an option agreement for a 50 year lease to the applicant as was provided with the original application packet.

IV.	Site Description

The subject site is an aggregate of waterfront parcels located adjacent to the Casco Bay Bridge along the dredged channel of the Fore River.  Totaling over 22 acres, the site contains active rail lines and has a long history of industrial, marine, and rail development.  Along with the lone remaining rail siding, the only remaining active use of the site is a propane distribution facility located on 3.96 acres in the interior of the site.  The propane facility is proposed to remain in its existing condition for Phase 1a.

The subject site is the combination of a former coal gasification plant site with portions of an historic rail yard.  With the exception of the propane facility, the land is vacant and the rail yard portions of the site have recently re-vegetated with volunteer birch, poplar, oak and pine.

The gas plant use has left portions of the site contaminated with coal tars and the site is partially subject to a Maine DEP “VRAP” remediation agreement

The Fore River shoreline is characterized by derelict piers and stone bulkheads.  To the east, the former gas plant site has been stabilized by recent riprap installed as part of the VRAP.  There are some areas of erosion located between the former gas and rail properties.  This erosion is assumed to be problematic with potential coal tar contaminants entering the river.  Redevelopment of the site provides an opportunity to stabilize this area of concern.

The site has +/-1500 feet of frontage along West Commercial Street.  While the northerly side of West Commercial Street, along Nova Seafood, the Graybar building and the “Star Match” complex, have curb and sidewalk, none exist along the subject parcel.  The applicants were approved for a waiver of curb and sidewalk with the previous review.

The Beach Street entrance/exit from the Casco Bay Bridge intersects West Commercial Street across from the center of the site at the location an existing driveway for the propane facility.  The Beach Street ramp intersection is currently being evaluated for a traffic signal and the City is seeking funds for such a project.

An additional vehicle entrance existed to the site adjacent to the IMT.  This entrance is a shared access roadway serving the subject parcel and a MDOT maintenance building for the Casco Bay Bridge

V.	Proposed Development

The applicant’s submission includes a detailed description of Phase 1a in Attachment B of this report.

The proposed improvements included with the requested amendment are shown on Attachment P1 (Sheet C-2.1)  Sheet C-2.1 shows the entirety of the previously approved improvements with the Phase 1a proposal marked with red balloons and cross hatching

Landside improvements in Phase 1a are largely limited to grading portions of the site for use.  Fire Safety is requiring additional fire hydrants located to the south west of the site to serve the marine improvements and the focus of the boat storage activities.  Marine improvements are reduced to a single boat ramp and supporting floating docks.   The marine improvements are consistent with approvals issued by the Portland Harbor Commission.

Shared Access with MDOT:

The proposed development includes use of the access drive located at the far easterly boundary of the site.  This drive currently serves a MDOT bridge building located near to the water.  The subject parcel retains access rights to this drive as noted in the applicant’s submittal in Attachment C.1.

The above figure is an excerpt of the proposed site plan showing the West Commercial Street frontage located toward the northeast corner of the site with the IMT.  The existing site driveway is located at the upper right and the gas works drive is located at the far left.  Phase 1a improvements here are shown with the red balloon and are limited to clearing the site of brush and existing rail tracks.  Negotiations are on-going for reestablishing tracks in a new right of way as shown with a ___ . ___ . ___ symbol.

The plan excerpt to the left shows the south west corner of the site, which is proposed for Phase 1a improvements including the boat ramp, floats, and grading the yard for boat storage.

Additional Phase 1a improvements will include repair to the historic granite seawall bulkhead and environmental remediation (to be conducted by the gas company.)

The only utility work proposed will be overhead electric service to a temporary construction trailer and water lines for an expanded fire hydrant network.

The applicant is not proposing to install any new landscaping with Phase 1a, but will adhere to the previously approved tree preservation plan.

The approved landscaping plan with tree preservation is provided in Attachment 3.1 and the previously approved site plan is Attachment 3.2.

VI.	Conditions of Approval

At the time of the December 2012 approval, the Board imposed a number of conditions of approval.  The Board’s approval letter describing the conditions as passed is provided as Attachment 1.

In February 2013, the applicant provided an update letter describing adherence to three of the conditions (Attachment E.)  Additionally, included in a response to staff review comments on the current application (Attachment C) the applicant has updated the status of each condition and described the timing of their compliance.  Any additional conditions of approval suggested for this phasing amendment are intended to apply to Phase 1a, specifically, with the December 2012 conditions still in effect for development of the full Phase 1 plan.

# VII.	Staff Review:

# The application and supporting documents have been reviewed by City staff from the Departments of Planning and Urban Development, Fire Safety, and Public Services.  The plans have also been reviewed by consulting engineers for civil engineering and traffic.  The December 2012 approval letter with conditions of approval and the original Planning Board Report (attachments omitted) are provided in Attachments 1 and 2 respectively.

# On August 16, Planning staff provided a list of review comments to the applicant, and engineer, Steve Bushey, provided responses by letter dated August 21 as provided in Attachment C.

# a.	Zoning Review:  WPDZ, Shoreland, Flood Plain

i.	Waterfront Port Development Zone

Use: The project is located in a WPD Zone.  The proposed boat repair yard uses are permitted uses in this zone.  In the December 2012 approval boat storage and sales (yacht brokerage) were both approved as conditional uses in the WPDZ.    For Phase 1a, the yacht brokerage is not proposed.

No Adverse Impact on Marine Use:  The WPDZ includes a provision for protecting access function of water dependent uses in the form of  “Sec. 14-318.5. No adverse impact on marine uses.”  The No Adverse impact provision could be construed to apply to the boat storage use as this use does not necessarily require direct water access (though water access may be a component through launching and hauling.) While applicability of the provision could be argued, no portion of the proposed development appears to create an adverse impact as described in the Sec. 14-318.5 and quoted below:

Sec. 14-318.5

(a)	The proposed nonwater-dependent use will displace an existing water-dependent use;

(b)	The proposed use will reduce existing commercial vessel berthing space;

- The proposed nonwater-dependent use, structure or activities, including but not limited to access, circulation, parking, dumpsters, exterior storage or loading facilities, and other structures, will unreasonably interfere with the activities and operation of existing water-dependent uses or significantly impede access to vessel berthing or other access to the water by water-dependent uses; or

(d)	The siting of a proposed nonwater-dependent use will substantially reduce or inhibit existing public access to marine or tidal waters.

Dimensional Requirements:

The proposed development meets all provisions of the WPDZ dimensional requirements.

Parking:

With no building proposed, no parking spaces are technically required for Phase 1a.  For the limited parking demand that will be generated by the development, a formalized parking area is not identified with the expectation that employees and clients using the site will find more than adequate parking to fill both the zoning requirements and functional needs of the use.

ii.	Shoreland Zone:

At the time of the previous approval on this project, the project met with all Tree Clearance restrictions that apply to development in the shoreland portions of the site.  Subsequent to the December 2012 approval, new legislation passed that allows a greater degree of tree clearance on working waterfront property.  While future phases of development may seek to take advantage of this new flexibility, the current proposal adheres to the previously approved tree preservation plan.

In addition to the tree clearance standards, the Planning Board found that the plan met the applicable shoreland zone standards and the proposed phasing plan is anticipated to have fewer impacts than the previously approved plan

iii.	Flood Plain Management:

The subject parcel is partially located in a regulated Flood Plain. The FEMA, Flood Insurance Rate Maps show portions of the property within the flood zone A2 with a base flood elevation of 10’.

With no buildings proposed in Phase 1a, Flood Plain management standards, as previously addressed, will not be impacted by the phasing proposed.

iv.	Vehicle Loading, Division 21:  NA

v.	Signage:

Separate permits are required for any new signage prior to installation.

b.	Conditional Use Review for Boat Storage in the WPDZ

As noted above, the proposal is subject to a conditional use review for boat storage in the WPDZ.  Conditional Uses in the WPDZ are subject to the following provisions:

# Sec. 14-320. Conditional uses.

…such uses will not impede or preclude existing or potential water-dependent development on other lots, will allow for adequate right-of-way access to the water, are compatible with marine uses, and are physically adaptable or relocatable to make way for future development for water-dependent uses, and meet any additional standards set forth below:

With no permanent buildings proposed with Phase 1a, the above standard appears to be met.

All conditional uses are subject to the provisions of Section 14-474, Conditional Use:

Standards. Upon a showing that a proposed use is a conditional use under this article, a conditional use permit shall be granted unless the board determines that:

a.	There are unique or distinctive characteristics or effects associated with the proposed conditional use;

b.	There will be an adverse impact upon the health, safety, or welfare of the public or the surrounding area; and

c.	Such impact differs substantially from the impact which would normally occur from such a use in that zone.

Staff recommends that the proposal is in conformance with the above standards.

c.	Site Plan Review.

Attachment 2 of this report includes the full text of the December 2012 Planning Board Report, which provides a description and analysis of all applicable review standards.  This report will concentrate on standards impacted by the proposed phasing request.

# Site Plan Standards Review, Sec. 14-526

The site plan standards language is summarized below in italics, with planning staff comment and analysis in regular font.

(a)	Transportation Standards

Primary access to the site is provided through the existing easterly entrance drive located adjacent to the IMT.

Consulting Traffic Engineer, Tom Errico, and the City Transportation Systems Engineer, Jeremiah Bartlett, have reviewed the existing condition and anticipate that Phase 1a should function adequately and safely.  There are some concerns that the existing 16 foot wide gate could create a queuing issue if large truck traffic is larger than anticipated and recommend a monitoring condition as follows:

Regarding the Phase 1A proposal for use of the existing easterly entrance adjacent to the IMT, the City will monitor the entrance for safety and may require modifications to the gate and entrance design if safety issues become evident.

(b)	Environmental Quality Standards

### 1. and 2.	Preservation of Significant Natural Features:

### Phase 1a proposes to adhere to the December 2012 tree preservation plan and will have fewer impacts than the previously approved plans.  The approved landscape plan is provided as Attachment 3.1.

3.	Water Quality, Stormwater Management and Erosion Control:

With Phase 1a, stormwater is intended to continue to behave as in the “pre-developed” condition and the applicant is not proposing to install additional stormwater management at this time.  Given the industrial history of the site, its flat grades, and well-drained soils, the limited construction activity proposed is not anticipated to cause stormwater quality issues in the Fore River.  Stormwater volumes should in fact be reduced with the Phase 1a activity as the applicant proposes to reduce the existing impervious surface by +/-32,000 sq ft.

The erosion and sedimentation controls approved in December 2012 are still in effect and will need to be adhered to through construction of all phases of development.

## (c)	Public Infrastructure and Community Safety Standards.

### 1.	Consistency with City Master Plans:

#### The proposal appears consistent with applicable approved master plans.  Note: The City is applying for funds to engage in a West Commercial Street planning process to determine the appropriate design to accommodate multiple modes of transport consistent with both landside mixed use and water-side marine development along the corridor.

### 2.	Public Safety and Fire Prevention:

### (a)	Natural surveillance that promotes visibility of public spaces and 			areas.

##### All of the principle open spaces on the development are visible with clear lines of site from multiple vantage points and/or buildings with windows.

##### (b)	Access control that promotes authorized and/or appropriate access to the site.

##### The site appears to sufficient access control through fencing, entry gates, and its location on the water.

#### (c)	Territorial reinforcement that promotes a sense of ownership and responsibility through environmental design.

#### The use and maintenance of the facility as designed should sufficiently reinforce the site’s identity as an active boat yard.

#### (d)	Provide adequate emergency vehicle access to the site in accordance with City standards for street widths and turning radii, as described in Section 1 of the Technical Manual.

#### In compliance with a previous condition of approval, the applicant provided a fire access and circulation plan, which was approved by Fire Safety reviewer, Captain Chris Pirone.  An excerpt from this plan is provided below.

#### With the proposed amendments, this plan will need to be updated and a condition of approval is suggested.  The intent of the fire safety plan is to identify emergency access routes and their adequacy for supporting fire suppression equipment.

#### (e)	Be consistent with City public safety standards, Section 3 of the City of Portland Technical Manual, including but not limited to availability and adequacy of water supply and proximity of fire hydrants to structures.

#### The site is currently served by two fire hydrants on West Commercial Street and an additional hydrant in the interior of the site.  The proposed phasing plan, Attachment 3.2, shows hydrant locations within blue buffer circles.

#### Portland Water District has provided a letter indicating adequate capacity to serve the fire suppression needs of the proposal.

#### Captain Pirone has indicated that one or more additional hydrants will be needed to serve the boat ramp and storage area(s) and he suggests that the design of and placement of additional hydrants can be a condition of approval as an amendment to the fire safety plan.

### 3.	Availability and Adequate Capacity of Public Utilities:

The site is served by all public utilities and a letter of water capacity was previously provided.

## (d)	Site Design Standards

### 1.	Massing, Ventilation and Wind Impact: NA

### 2.	Shadows: NA

### 3.	Snow and Ice Loading: NA

### 4.	View corridors: NA

### 5.	Historic Resources:

With the limited amount of construction proposed, potential on-site historic and archeological resources will not be negatively impacted by proposed phasing amendment.  The 19th century seawall is currently being restored in conjunction with the proposed marine improvements.

### 6.	Exterior Lighting:

No exterior lighting is proposed in Phase 1a.

### 7.	Noise and Vibration:

### Based on the current Portland Yacht Services activities, exterior noise levels are anticipated to be within zoning requirements for sound levels.

### 8.	Signage and Wayfinding:

### Signage plans have not been provided, but will be reviewed and approved with the building permit applications for the site.  No signage may be installed without such approval.

### 9.	Zoning Related Design Standards   NA

##### VIII.	Staff Recommendation:

##### The applicants have demonstrated adherence to the applicable standards and Planning Staff recommends that the Planning Board approve the phasing of the New Yard at Canal Landing, subject to the conditions provided below in the suggested motions.

##### IX.	Recommended Motions:

b.	Conditional Use in the Waterfront Port Development Zone:

On the basis of the application, plans, reports and other information submitted by the applicant, findings and recommendations contained in the Planning Board Report for application # 2013-201 relevant to the Conditional Use Standards of the Waterfront Port Development Zone for boat storage, and Section 14-474 of Land Use Code and other regulations, and the testimony presented at the Planning Board hearing, the Planning Board finds that the plan (is/is not) in conformance with conditional use standards of the land use code.

c.   Site Plan:

On the basis of the application, plans, reports and other information submitted by the applicant, findings and recommendations contained in the Planning Board Report for application # 2013-201 relevant to the Site Plan Ordinance, Shoreland Zone, Flood Plain Management, and other regulations, and the testimony presented at the Planning Board hearing, the Planning Board finds that the plan (is/is not) in conformance with applicable standards of the land use code, subject to the following conditions of approval:

Conditions of Approval

- MDOT Shared Entrance:  Regarding the proposal for use of the existing easterly entrance adjacent to the IMT, the City will monitor the entrance for safety and may require modifications to the gate and entrance design if safety issues become evident.

- Fire Safety and Emergency Access:  Prior to occupancy and or use of the site, the applicant shall submit a revised fire safety plan for review and approval identifying additional hydrant locations, emergency access routes, and their adequacy for supporting fire suppression equipment.

- Previous Conditions of Approval:  For development approved on December 18, 2012, all conditions of approval remain in effect as described in Attachment 1 of this report.

Attachments

1.	Approval Letter from December 18, 2012 Public Hearing

2.	Planning Board Report #53-12, December 14, 2012

3.1	Previously approved Landscape Plan (showing tree preservation)

3.2.	Previously approved Site Plan

Applicant’s Submission Packet

A. 	Application and Project Information

B.	Cover Letter with Phase 1a description, August 16, 2013

C.	Response to Staff Comments, August 21, 2013

D.	Deed from Portland Terminal Co to New Yard, LLC (replacing previous option 	document)

E.	Conditions of approval compliance update letter, February 15, 2013

Plans

Plan 1	Phase 1a Plan (shown on updated Site Plan for full Phase 1 build out)