02100 - SITE PREPARATION AND LAYOUT

PART  1	GENERAL

SCOPE

A.	Under this item, the Contractor shall provide all labor, material, equipment and transportation for preparing the site for construction in accordance with the Contract Plans and these Specifications.

PART 2	PRODUCTS

(NONE REQUIRED)

PART 3	EXECUTION

3.01	CONSTRUCTION LAYOUT AND STAKING:

A.	The Contractor will provide the initial construction layout and staking, which will include; bench marks, offsets and grade stakes.  The Contractor shall be held responsible for maintaining all surveying data, including but not limited to stakes, offsets, control points, bench marks, etc.  Any additional offsets or control points that the Contractor requires shall be deemed for his convenience.  The cost for any additional surveying information shall be borne by the Contractor.

B.	Locations Shown on Contract Plans not Guaranteed:

1.	The structures, obstructions, hydrants, utilities, trees, shrubs and stone walls shown on the Contract Plans are those known to exist, but their location is not guaranteed to be exact, nor is it guaranteed that all structures, obstructions, utilities, trees, shrubs and stone walls are shown.  The Contractor shall, however, be responsible for the protection of all structures, obstructions, utilities, trees, shrubs, and stone walls whether shown or not on the Contract Plans.

3.02	SAFEGUARDS AND PROTECTION

A.	The Contractor shall provide all necessary safeguards including the installation of shoring, signing, structural supports, protective fencing and barriers, etc., as may be required to prevent damage to adjacent property or injury to persons.  All work shall be performed in accordance with the requirements of the local building codes and the rules, regulations and ordinances of all governing bodies having jurisdiction.  The Contractor shall be responsible for any claims arising from his failure to provide proper safeguards or for his failure to conduct his operation in a manner consistent with the rules, regulations and ordinances of the governing agencies having jurisdiction.

3.03	NOTIFICATION OF UTILITY OWNERS

A.	CALL BEFORE YOU DIG: The Contractor shall notify Dig Safe and/or other applicable utility owners at least seventy-two (72) hours prior to the start of any excavation operation.  The Contractor will be held responsible for any claims arising from his failure to make such notification, or for his failure to do the work in accordance with the rules and regulations of the governing authorities.

3.04	PROTECTION OF EXISTING STRUCTURES AND UTILITIES

A.	The Contractor shall be responsible throughout the course of the work for protection from injury or damage of all existing structures and utilities that are to remain.

B.	All existing gas and water lines, telephone and electric poles, wires, conduits, sewers, drains, culverts, fire hydrants and other utilities which are to remain shall be carefully supported, maintained in operation and protected from injury or damage by the Contractor.

3.05	DEMOLITION FOR SITE WORK

A.	The Contractor shall remove all existing plants, pavements, buildings or other structures as indicated on the Contract Drawings.  All material obtained from demolition operations shall be removed from the site and disposed of in a timely manner, in accordance with applicable laws and codes, or, as directed by the project engineer.  Items of value (as determined by the owner’s representative) encountered during demolition operations will remain the property of the owner, until determined to be disposable by the owner’s representative.

3.06	CLEARING AND GRUBBING

A.	Prior to commencing Demolition, the Owner will be responsible to remove and salvage any vegetation desired for reuse. The Owner’s Representative shall have the final authority on the removal of all trees.  The Contractor, at the Contractor’s expense shall replace any trees removed contrary to the orders of the Owner’s Representative.  Any trees or vegetation damaged during construction shall be repaired immediately by an approved tree surgeon at the Contractor's expense.

B.	Prior to commencing clearing operations, the Contractor shall provide all sediment and erosion control improvements required related to the area to be disturbed.  Sediment and erosion controls shall be installed and maintained throughout the construction period as specified on the drawings.

3.07	DISPOSAL OF MATERIAL

A.	All material obtained from the removal of structures, stones and boulders and obstructions, including, but not limited to, metal, concrete pavements, asphaltic pavements and all other miscellaneous debris, shall be removed from the site, or dealt with as directed by the Owner’s Representative.

B.	In addition to the above-mentioned items the Contractor shall also be responsible for the off-site removal of construction related debris.  This disposal shall be in accordance with all applicable codes, rules and regulations.  This material shall be defined as anything used by the Contractor to complete his work and shall include such items as leftover pipe, pallets and garbage.  Under no circumstances shall on-site burning of the material be permitted.

3.08	PROTECTION OF EXISTING VEGETATION AND NATURAL FEATURES

A.	The Contractor shall protect, throughout the course of construction, all such trees, brush, vegetation and existing improvements as are shown on the Drawings which are not marked by the Owner’s Representative or indicated on the Drainage Plans to be removed.  Tree protection shall be installed as shown on the Drawings.

B.	The Contractor shall also protect throughout the course of construction all landscaping, vegetation and natural features on public and private property.  The Contractor shall use every precaution to prevent injury, damage, pollution, erosion or destruction of existing landscaping, water courses, vegetation and other surrounding natural features.

C.	The Contractor shall construct and maintain a proper and substantial protective fence around trees that are to be saved adjacent to or within the construction area.  The fence shall be constructed as specified on the Drawings.  All protective fencing shall be subject to the approval of the Owner’s Representative.

D.	Grading and/or filling operations adjacent to trees shall be carried out with extreme care.  If the soil over the root area of the trees has been compacted, it shall be restored by proper cultivation to permit entrance of water and proper aeration of roots.

E.	Roots and limbs of trees are not to be cut unless authorized by the Owner’s Representative.  Should it become necessary to do so, the Contractor shall treat the remaining exposed portion of roots and/or limbs to prevent damage, loss or injury to the tree and perform the necessary pruning of the tree.  This treatment shall be performed in accordance with accepted horticulture practice and by personnel experienced in that field of work.

##### END OF SECTION

02751 - CONCRETE PAVEMENT

PART 1	GENERAL

1.01	SUMMARY

A.	Work includes exterior Portland cement concrete paving for the following:

1.	Walkways.

2. 	Ramps

B.	Prepared subbase is specified in Section 02200 - Earthwork.

1.02	SUBMITTALS

A.	Submit design mixes for each class of concrete.  Include revised mix proportions when characteristics of materials, project conditions, weather, test results, or other circumstances warrant adjustment.

1.03	QUALITY ASSURANCE

A.	Installer Qualifications:  An experienced installer who has completed pavement work similar in material, design, and extent to that indicated for this Project and whose work has resulted in construction with a record of successful in-service performance.

B.	Manufacturer Qualifications:  Manufacturer of ready-mixed concrete products complying with ASTM C 94 requirements for production facilities and equipment.  Manufacturer must be certified according to the National Ready Mix Concrete Association's Plant Certification Program.

C.	Comply with provisions of the following standards, except where more stringent requirements are indicated.

1.	American Concrete Institute (ACI) 301, "Specifications for Structural Concrete for Buildings."

2.	ACI 318, "Building Code Requirements for Reinforced Concrete."

3.	Concrete Reinforcing Steel Institute (CRSI) "Manual of Standard Practice."

D.	Comply with testing requirements specified in Section 03310.

E.	Provide a mockup sample of concrete paving minimum 4’ x 4’ indicating expansion and score joints and finished and cured in accordance with the specifications.  Obtain Landscape Architect’s approval of sample prior to start of work.  Make all changes or provide additional samples until Architect’s approval is obtained.  Maintain mockup on site until directed to remove.  Dispose of mockup consistent as per demolition specifications.

1.04	JOB CONDITIONS

A.	Maintain access for vehicular and pedestrian traffic as required for other construction activities.

PART 2	PRODUCTS

2.01	FORMS

A.	Forms:  New plywood, metal, or other approved panel-type materials to provide full-depth, continuous, straight, smooth exposed surfaces.  Use flexible or curved forms for curves of a radius 100 feet (30.5 m) or less.

B.	Form-Release Agent:  Commercially formulated form-release agent that will not bond with, stain, or adversely affect concrete surfaces and will not impair subsequent treatments of concrete surfaces.

2.02		STEEL REINFORCEMENT

A.	Plain-Steel Welded Wire Fabric:  ASTM A 185, fabricated from as-drawn steel wire into flat sheets.

B.	Reinforcement Bars:  ASTM A 615/A 615M, Grade 60 (Grade 420), deformed.

C.	Bar Supports:  Bolsters, chairs, spacers, and other devices for spacing, supporting, and fastening reinforcement bars, welded wire fabric, and dowels in place.  Manufacture bar supports according to CRSI's "Manual of Standard Practice" from steel wire, plastic, or precast concrete or fiber-reinforced concrete of greater compressive strength than concrete, and equip wire bar supports with sand plates or horizontal runners where base material will not support chair legs.

2.03	CONCRETE MATERIALS

A.	Aggregate:  ASTM C 33, uniformly graded, from a single source, with coarse aggregate as follows:

1.	Class:  4S.

2.	Maximum Aggregate Size:  1 inch (25 mm) nominal.

3.	Do not use fine or coarse aggregates containing substances that cause spalling.

B.	Water:  ASTM C 94.

2.04	ADMIXTURES

A.	General:  Admixtures certified by manufacturer to contain not more than 0.1 percent water-soluble chloride ions by mass of cement and to be compatible with other admixtures.

B.	Air-Entraining Admixture:  ASTM C 260.

C.	Water-Reducing Admixture:  ASTM C 494, Type A.

2.05	CURING MATERIALS

A.	Absorptive Cover:  AASHTO M 182, Class 2, burlap cloth made from jute or approved equal, weighing approximately 9 oz./sq. yd. (305 g/sq. m) dry.

B.	Moisture-Retaining Cover:  ASTM C 171, polyethylene film or white burlap-polyethylene sheet.

C.	Water:  Potable.

D.	Clear Waterborne Membrane-Forming Curing Compound:  ASTM C 309, Type 1, Class B.  subject to compliance with requirements, provide one of the following or equivalent:

Aqua Resin Cure; Burke Group, LLC (The).

Safe-Cure Clear; ChemMasters.

W.B. Resin Cure; Conspec Marketing & Manufacturing Co., Inc.

L&M Cure R; L&M Construction Chemicals, Inc.

1100 Clear; W. R. Meadows, Inc.

Resin Cure E; Nox-Crete Products Group, Kinsman Corporation.

2.06	ELASTOMERIC JOINT SEALANTS

A.	Multicomponent Nonsag Urethane Sealant: Where joint sealants of this type are indicated, provide products complying with the following:

Provide one of the following:

1. Sikaflex-2c, NS; Sika

2. NP 2; Sonneborn Building Products Div., ChemRex Inc.

3. Dynatrol 2; Pecora Corporation

4. Vulkem 922; Mameco International

5. Chem-Calk 500; Bostik Inc.

2.07	JOINT SEALANT BACKING

A.	General: Provide sealant backings of material and type that are nonstaining; are compatible with joint substrates, sealants, primers and other joint fillers; and are approved for applications indicated by sealant manufacturer based on field experience and laboratory testing.

B.	Backer Rod: ASTM C 1330, of type indicated below and of size and density to control sealant depth and otherwise contribute to producing optimum sealant performance:

1. Type C: Closed-cell material with a surface skin.

C.	Bond-Breaker Tape: polyethylene tape or other plastic tape recommended by sealant manufacturer for preventing sealant from adhering to rigid, inflexible joint-filler materials or joint surfaces at back of joint where such adhesion would result in sealant failure. Provide self-adhesive tape where applicable.

2.08	CONCRETE MIX

A.	Prepare design mixes, proportioned according to ACI 211.1 and ACI 301, for each type and strength of normal-weight concrete determined by either laboratory trial mixes or field experience.

B.	Use a qualified independent testing agency for preparing and reporting proposed mix designs for the trial batch method.

C.	Proportion mixes to provide concrete with the following properties:

1.	Compressive Strength (28 Days):  3500 psi (24.1 MPa).

2.	Maximum Water-Cementitious Materials Ratio:  0.45.

3.	Slump Limit:  3 inches (75 mm).

D.	Cementitious Materials:  Limit percentage, by weight, of cementitious materials other than Portland cement according to ACI 301 requirements for concrete exposed to deicing chemicals.

E.	Add air-entraining admixture at manufacturer's prescribed rate to result in concrete at point of placement having an air content of 6.0 percent for 1-inch (25-mm) maximum aggregate within a tolerance of plus or minus 1.5 percent.

2.09	CONCRETE MIXING

A.	Ready-Mixed Concrete:  Comply with requirements and with ASTM C 94.

B.	When air temperature is between 85 deg F (30 deg C) and 90 deg F (32 deg C), reduce mixing and delivery time from 1-1/2 hours to 75 minutes; when air temperature is above 90 deg F (32 deg C), reduce mixing and delivery time to 60 minutes.

PART 3	EXECUTION

3.01	SURFACE PREPARATION

A.	Remove loose material from compacted subbase surface immediately before placing concrete.

B.	Proof roll prepared subbase surface to check for unstable areas and need for additional compaction.  Do not begin paving work until such conditions have been corrected and are ready to receive paving.

3.02	FORM CONSTRUCTION

A.	Set forms to required grades and lines, rigidly braced and secured.  Install sufficient quantity of forms to allow continuous progress of work and so that forms can remain in place at least 24 hours after concrete placement.

B.	Check completed formwork for grade and alignment to following tolerances:

1.	Top of forms, not more than 1/8" in 10'.

2.	Vertical face on longitudinal axis, not more than 1/4" in 10'.

C.	Clean forms after each use, and coat with form release agent as often as required to ensure separation from concrete without damage.

3.03	REINFORCEMENT

A.	General:  Comply with CRSI's "Manual of Standard Practice" for fabricating reinforcement and with recommendations in CRSI's "Placing Reinforcing Bars" for placing and supporting reinforcement.

B.	Clean reinforcement of loose rust and mill scale, earth, ice, or other bond-reducing materials.

C.	Arrange, space, and securely tie bars and bar supports to hold reinforcement in position during concrete placement.  Maintain minimum cover to reinforcement.

D.	Install welded wire fabric in lengths as long as practicable.  Lap adjoining pieces at least one full mesh, and lace splices with wire.  Offset laps of adjoining widths to prevent continuous laps in either direction.

3.04	CONCRETE PLACEMENT

A.	Inspection:  Before placing concrete, inspect and complete formwork installation, reinforcement steel, and items to be embedded or cast in.  Notify other trades to permit installation of their work.

B.	Remove snow, ice, or frost from subbase surface and reinforcement before placing concrete.  Do not place concrete on frozen surfaces.

C.	Moisten subbase to provide a uniform dampened condition at the time concrete is placed.  Do not place concrete around manholes or other structures until they are at the required finish elevation and alignment.

D.	Comply with requirements and with recommendations in ACI 304R for measuring, mixing, transporting, and placing concrete.

E.	Do not add water to concrete during delivery, at Project site, or during placement.

F.	Deposit and spread concrete in a continuous operation between transverse joints.  Do not push or drag concrete into place or use vibrators to move concrete into place.

G.	Consolidate concrete by mechanical vibrating equipment supplemented by hand-spading, rodding, or tamping.  Use equipment and procedures to consolidate concrete according to recommendations in ACI 309R.  Consolidate concrete along face of forms and adjacent to transverse joints with an internal vibrator.  Keep vibrator away from joint assemblies, reinforcement, or side forms.  Use only square-faced shovels for hand-spreading and consolidation.  Consolidate with care to prevent dislocating reinforcement, dowels, and joint devices.

H.	Place concrete in two operations; strike off initial pour for entire width of placement and to the required depth below finish surface.  Lay welded wire fabric or fabricated bar mats immediately in final position.  Place top layer of concrete, strike off, and screed.  Remove and replace portions of bottom layer of concrete that have been placed more than 15 minutes without being covered by top layer, or use bonding agent if approved by Architect.

I.	Screed pavement surfaces with a straightedge and strike off.  Commence initial floating using bull floats or darbies to form an open textured and uniform surface plane before excess moisture or bleed water appears on the surface.  Do not further disturb concrete surfaces before beginning finishing operations or spreading dry-shake surface treatments.

J.	Cold-Weather Placement:  Comply with ACI 306.1 and as follows.  Protect concrete work from physical damage or reduced strength that could be caused by frost, freezing actions, or low temperatures.

1.	When air temperature has fallen to or is expected to fall below 40 deg F (4.4 deg C), uniformly heat water and aggregates before mixing to obtain a concrete mixture temperature of not less than 50 deg F (10 deg C) and not more than 80 deg F (27 deg C) at point of placement.

2.	Do not use frozen materials or materials containing ice or snow.

3.	Do not use calcium chloride, salt, or other materials containing antifreeze agents or chemical accelerators, unless otherwise specified and approved in mix designs.

K.	Hot-Weather Placement:  Place concrete according to recommendations in ACI 305R and as follows when hot-weather conditions exist:

1.	Cool ingredients before mixing to maintain concrete temperature at time of placement below 90 deg F (32 deg C).  Chilled mixing water or chopped ice may be used to control temperature, provided water equivalent of ice is calculated to total amount of mixing water.  Using liquid nitrogen to cool concrete is Contractor's option.

2.	Cover reinforcement steel with water-soaked burlap so steel temperature will not exceed ambient air temperature immediately before embedding in concrete.

3.	Fog-spray forms, reinforcement steel, and subgrade just before placing concrete.  Keep subgrade moisture uniform without standing water, soft spots, or dry areas.

3.05	JOINTS

A.	General:  Construct construction, expansion, and score joints and tool edgings true to line with faces perpendicular to surface plane of concrete.  Construct transverse joints at right angles to centerline, unless otherwise indicated.  When joining existing pavement, place transverse joints to align with previously placed joints, unless otherwise indicated.

B.	Construction Joints:  Set construction joints at side and end terminations of pavement and at locations where pavement operations are stopped for more than one-half hour, unless pavement terminates at isolation joints.

1.	Provide preformed galvanized steel or plastic keyway-section forms or bulkhead forms with keys, unless otherwise indicated.  Embed keys at least 1-1/2 inches (38 mm) into concrete.

2.	Continue reinforcement across construction joints, unless otherwise indicated.  Do not continue reinforcement through sides of pavement strips, unless otherwise indicated.

3.	Provide tie bars at sides of pavement strips where indicated.

4.	Use a bonding agent at locations where fresh concrete is placed against hardened or partially hardened concrete surfaces.

C.	Expansion Joints:  Form expansion joints of preformed joint-filler strips abutting concrete curbs, catch basins, manholes, inlets, structures, walks, other fixed objects, and where indicated.

1.	Locate expansion joints as specified on the drawings, at intervals 15 feet (15.25 m) maximum.

2.	Extend joint fillers full width and depth of joint.

3.	Terminate joint filler less than 1/2 inch (12 mm) or more than 1 inch (25 mm) below finished surface if joint sealant is indicated.

4.	Place top of joint filler flush with finished concrete surface if joint sealant is not indicated.

5.	Furnish joint fillers in one-piece lengths.  Where more than one length is required, lace or clip joint-filler sections together.  Provide sample for approval prior to installation.

6.	Protect top edge of joint filler during concrete placement with metal, plastic, or other temporary preformed cap.  Remove protective cap after concrete has been placed on both sides of joint.

D.	Install dowel bars and support assemblies at joints where indicated.  Lubricate or asphalt-coat one-half of dowel length to prevent concrete bonding to one side of joint.

E.	Score Joints:  Form weakened-plane score joints, sectioning concrete into areas as indicated. Construct score joints for a depth equal to at least one-fourth of the concrete thickness.  Form score joints with power saws equipped with shatterproof abrasive or diamond-rimmed blades.  Cut 1/8-inch- (3-mm-) wide joints into concrete when cutting action will not tear, abrade, or otherwise damage surface and before developing random score cracks.

F.	Edging:  Tool edges of pavement, gutters, curbs, and joints in concrete after initial floating with an edging tool to the following radius.  Repeat tooling of edges after applying surface finishes.  Eliminate tool marks on concrete surfaces.  Radius:  1/4 inch (6 mm).

3.06	CONCRETE FINISHING

A.	General:  Wetting of concrete surfaces during screeding, initial floating, or finishing operations is prohibited.

B.	Float Finish:  Begin the second floating operation when bleed-water sheen has disappeared and the concrete surface has stiffened sufficiently to permit operations.  Float surface with power-driven floats, or by hand floating if area is small or inaccessible to power units.  Finish surfaces to true planes.  Cut down high spots, and fill low spots.  Refloat surface immediately to uniform granular texture.

C.	Fine-Textured Broom Finish:  Draw a soft bristle broom across float-finished concrete surface perpendicular to line of traffic to provide a uniform, fine-line texture.

3.07	CONCRETE PROTECTION AND CURING

A.	General:  Protect freshly placed concrete from premature drying and excessive cold or hot temperatures.  Comply with ACI 306.1 for cold-weather protection and follow recommendations in ACI 305R for hot-weather protection during curing.

B.	Begin curing after finishing concrete, but not before free water has disappeared from concrete surface.

C.	Curing Methods:  Cure concrete by moisture curing, moisture-retaining-cover curing, curing compound, or a combination of these as follows:

1.	Moisture Curing:  Keep surfaces continuously moist for not less than seven days with the following materials:

a.	Water.

b.	Continuous water-fog spray.

c.	Absorptive cover, water saturated, and kept continuously wet.  Cover concrete surfaces and edges with 12-inch (300-mm) lap over adjacent absorptive covers.

2.	Moisture-Retaining-Cover Curing:  Cover concrete surfaces with moisture-retaining cover for curing concrete, placed in widest practicable width, with sides and ends lapped at least 12 inches (300 mm), and sealed by waterproof tape or adhesive.  Immediately repair any holes or tears during curing period using cover material and waterproof tape.

3.	Curing Compound:  Apply uniformly in continuous operation by power spray or roller according to manufacturer's written instructions.  Recoat areas subjected to heavy rainfall within three hours after initial application.  Maintain continuity of coating and repair damage during curing period.

3.08	PAVEMENT TOLERANCES

A.	Comply with tolerances of ACI 117 and as follows:

B.	Elevation:  1/4 inch (6 mm).

1.	Thickness:  Plus 3/8 inch (9 mm), minus 1/4 inch (6 mm).

2.	Surface:  Gap below 10-foot-(3-m-) long, unleveled straightedge not to exceed 1/4 inch (6 mm).

3.	Lateral Alignment and Spacing of Tie Bars and Dowels:  1 inch (25 mm).

4.	Vertical Alignment of Tie Bars and Dowels:  1/4 inch (6 mm).

5.	Alignment of Tie-Bar End Relative to Line Perpendicular to Pavement Edge:  1/2 inch (13 mm).

6.	Joint Spacing:  3 inches (75 mm).

7.	Contraction Joint Depth:  Plus 1/4 inch (6 mm), no minus.

8.	Joint Width:  Plus 1/8 inch (3 mm), no minus.

3.09	REPAIRS AND PROTECTION

A.	Remove and replace concrete pavement that is broken, damaged, or defective, or does not meet requirements in this Section.

B.	Drill test cores where directed by Architect when necessary to determine magnitude of cracks or defective areas.  Fill drilled core holes in satisfactory pavement areas with Portland cement concrete bonded to pavement with epoxy adhesive.

C.	Protect concrete from damage.  Exclude traffic from pavement for at least 14 days after placement.  When construction traffic is permitted, maintain pavement as clean as possible by removing surface stains and spillage of materials as they occur.

D.	Maintain concrete pavement free of stains, discoloration, dirt, and other foreign material.  Sweep concrete pavement not more than two days before date scheduled for Substantial Completion inspections.

# END OF SECTION

### 02513	BITUMINOUS CONCRETE PAVEMENT

### PART 1 – GENERAL

1.01	DESCRIPTION

A. 	Provide hot bituminous pavement for driveways, parking lots and walkways as shown and specified.  The work includes the construction of hot bituminous paved sidewalks, in accordance with the most recent edition of the State of Maine Department of Transportation Standard Specifications Highways and Bridges.

B.	Related:

1.	Section 02100:  Site Preparation

2.	Section 02200:  Earthwork

3.	Section 02700:  Concrete Pavement

1.02	REFERENCES

A.	State of Maine Department of Transportation Standard Specifications Highways and Bridges, most recent edition, hereafter designated as MDOT Specifications.

1.03	QUALITY ASSURANCE

A.	Perform work in accordance with MDOT Section 403.

B.	Mixing Plant:  Conform to MDOT Section 401.

C.	Obtain materials from same source throughout.

1.04	REGULATORY REQUIREMENTS

A.	Conform to applicable standards for paving work on public property.

1.05	TESTS

A.	Submit proposed mix design of each class of mix for review prior to commencement of work.

B.	All testing shall be performed by a qualified testing laboratory approved by the Owner or Landscape Architect and at the Contractor’s expense in accordance with MDOT Standards.

C.	Testing organization shall have a minimum of 5 years experience in conducting tests and evaluations of bituminous concrete materials and design.

1.06	SUBMITTALS

A.	Submit product data.

B.	Submit manufacturers’ instructions.

C.	Submit test results from testing organization.

1.07	ENVIRONMENTAL REQUIREMENTS

A.	Weather and seasonal limitations as required by Section 401.07 of the MDOT Specifications shall apply to this Section.

#### PART 2 - PRODUCTS

2.01	MATERIALS

A.	Bituminous Paved sidewalks shall meet the requirements of MDOT Section 403.

#### PART 3 - EXECUTION

3.01	INSPECTION

A.	Verify compacted base and subbase material is dry and ready to support paving and imposed loads.

B.	Verify gradients and elevations of base are correct and that radius of all curves and dimensions of walks are in conformance with drawings.

C.	Beginning of installation means acceptance of substrate.

3.02	PREPARATION AND PLACEMENT

A.	Prepare and place plant mix hot bituminous pavement in accordance with MDOT Specifications Section 401 and Section 608.04.

3.03	TOLERANCES

A.	Flatness:  Conform to requirements of MDOT Section 401.20.

B.	Compacted Scheduled Thickness:  Conform to requirements of MDOT Section 401.17.

C.	Variation from True Elevation:  Conform to requirements of MDOT Section 403.

3.04	PROTECTION

A.	Immediately after placement, protect pavement from mechanical injury for 5 days.

###### END OF SECTION

02800	GRANITE STEPS, BENCHES & CURB WALLS

PART 1 - GENERAL

1.01	DESCRIPTION

A.	Provide all granite for steps, benches and curb walls.

B.	Provide all necessary fasteners, mortar, sealants, cleaners, etc., to install granite improvements as shown on the drawings.

C.	Related work:

1.	Section 02200:  Earthwork.

2.	Section 02700:  Concrete Pavement

1.02	SUBMITTALS

A.	Shop Drawings:  Provide shop drawings for approval prior to ordering materials.

1.	Shop Drawings shall show all granite pieces, identified by number and related to their locations in the construction drawings.

2.	All drawings shall include dimensions of all granite units; and shall indicate details, fasteners connections, joints, finishes, and adjacent work.

B.	Samples: Deliver to job site a representative granite sample (min. 12” x 12” for each element in the design.  (i.e.: step, bench leg, bench top, and curb wall)   and Owner will review for color, texture, finish, and general condition.

- Samples that do not meet approval shall be replaced until approval is met.

1.03		QUALITY ASSURANCE

A.	Single Source: Obtain all granite from a single source throughout the project.

B.	Qualifications:  All granite shall be installed by a single installer that has primary training and a specialty business in stone masonry.

C.	Schedule and hold a pre-installation meeting with the Landscape Architect at the site, at a mutually agreeable time, to review the methods and materials included in this section.

1.04		DELIVERY, STORAGE AND HANDLING

- All granite shall be packed, transported and stored in materials and methods that insure against damage, discoloration or other undesirable changes in the quality of the materials from mechanical, weather or other forces.

PART 2	PRODUCTS

2.01	GRANITE

A.	Granite:  All granite components shall be Freshwater Pearl (darker color ranges) from Freshwater Stone Inc. or approved equal.  All granite for benches, curb walls and steps must be from the same source.

B.	All granite shall be free of cracks, seams, or starts that might impair the structural integrity or function of the stone.  Inherent color variations characteristic of the source quarry will be acceptable.

C.	All granite surfaces that will be exposed after installation shall be sawn and thermal finished unless indicated otherwise on the drawings.

2.02	ACCESSORIES

A.	Anchors:  Attach all granite components with min. 3/8” dia stainless steel pins and waterproof epoxy grout as indicated on the drawings.

- Mortar:  all methods of measuring, mixing and handling of mortars shall be incompliance with ASTM C270 – Standard Specification for Unit Masonry.

PART 3 - EXECUTION

3.01	FABRICATION

A.	Fabrication:  Create granite components to dimensions and finishes indicated on the drawings.

B.	Tolerances: All granite steps, benches and curb walls shall be of the specified dimensions within 1/8 of the nominal dimension shown.

C.	Ease all edges 1/8”.

3.02	INSTALLATION

A.	Prior to installation clean all surfaces in compliance with recommendations of manufacturer of the mortar to be used.

B.	Prior to installation, inspect and verify grades of all surfaces upon which granite is to be installed.  Adjust grades as necessary before installing granite.

C.	Place units at the locations and grades indicated on the drawings so that they are plumb, level, and in alignment on prepared base.  Use care in moving units to avoid scratching, chipping, or gouging the surfaces.

D.	Joints: All joints shall be square and true, to the dimensions indicated on the drawings.

E.	Drill, sink and grout SS pins as indicated on the drawings to secure granite components to each other, and to their footings and/or base.

F.	Clean all granite surfaces with an organic, biodegradable solution to remove traces of mortar, grout, soil or other materials.

END OF SECTION

2900 - PLANTING

PART 1	GENERAL

1.01	SUMMARY

A.	This Section includes the following;

1.	Trees / Shrubs

2.	Groundcover.

3.	Topsoil and soil amendments.

4.	Fertilizers and mulches.

5.	Stakes and guys.

B.	Excavation, filling and rough grading required to establish elevations shown on drawings are not specified in this section.  Refer to Section 02300 - Earthwork.

C.	Lawns are specified in Section 02920.

1.02	SUBMITTALS

A.	Submit certificates of inspection as required by governmental authorities.  Submit manufacturer's or vendors certified analysis for soil amendments and fertilizer materials.  Submit other data substantiating that materials comply with specified requirements.

B.	Submit proposed planting schedule, indicating dates for each type of landscape work during normal seasons for such work in area of site.  Correlate with specified maintenance periods to provide maintenance from date of substantial completion.  Once accepted, revise dates only as approved in writing, after documentation of reasons for delays.

C.	Submit typewritten instructions recommending procedures to be established by Owner for maintenance of landscape work for one full year.  Submit prior to expiration of required maintenance periods.

D.	Submit reports of chemical and mechanical analysis of topsoil.

1.03	QUALITY ASSURANCE

A.	Landscape work shall be performed by a single firm specializing in landscape work.

B.	Testing Agency Qualifications:  To qualify for acceptance, an independent testing agency must demonstrate to Landscape Architect's satisfaction, based on evaluation of agency-submitted criteria conforming to ASTM E 699, that it has the experience and capability to satisfactorily conduct the testing indicated without delaying the Work.

C.	Ship landscape materials with certificates of inspection required by governing authorities.  Comply with regulations applicable to landscape materials.

D.	Do not make substitutions.  If specified landscape material is not obtainable, submit proof of non-availability to Landscape Architect, together with proposal for use of equivalent material.

E.	Package standard products with manufacturer's certified analysis.  For other materials, provide analysis by recognized laboratory made in accordance with methods established by the Association of Official Agricultural Chemists, wherever applicable.

F.	Provide trees, shrubs and plants of quantity, size, genus, species and variety shown on the drawings in compliance with ANSI Z60.1 "American Standard for Nursery Stock".  Provide healthy, vigorous stock, grown in a recognized nursery in accordance with good horticultural practice and free of disease, insects, eggs, larvae and defects such as knots, sun scald, injuries, abrasions, or disfigurement.

G.	Label each tree and shrub with securely attached waterproof tag bearing legible designation of botanical and common name.

H.	Where formal arrangements or consecutive order of trees or shrubs are shown, select stock for uniform height and spread, and label with number to assure symmetry in planting.

I.	The Landscape Architect may inspect trees and shrubs either at place of growth or at site before planting, for compliance with requirements for genus, species, variety, size and quality.  Landscape Architect retains right to further inspect trees and shrubs for size and condition of balls and root systems insects, injuries and latent defects, and to reject unsatisfactory or defective material at any time during progress of work.  Remove rejected trees or shrubs immediately from project site.

J.	Measure trees and shrubs according to ANSI Z60.1 with branches and trunks or canes in their normal position.  Do not prune to obtain required sizes.  Take caliper measurements 6" (150 mm) above ground for trees up to 4" (100-mm) caliper size, and 12" (300 mm) above ground for larger sizes.  Measure main body of tree or shrub for height and spread; do not measure branches or roots tip-to-tip.

1.04	DELIVERY, STORAGE AND HANDLING

A.	Deliver packaged materials in containers showing weight, analysis and name of manufacturer.  Protect materials from deterioration during delivery, and while stored at site.

B.	Provide freshly dug trees and shrubs.  Do not prune plants prior to delivery unless otherwise approved by Landscape Architect. Do not bend or tie trees or shrubs in such manner as to damage bark, break branches or destroy natural shape. Provide protective covering during delivery.  Do not drop balled and burlapped stock during delivery.

C.	Immediately after digging bare-root stock, pack root system in wet straw, hay, or other suitable material to keep root system moist until planting.

D.	Handle balled and burlapped stock by the root ball.

E.	Deliver trees and shrubs after preparations for planting have been completed and plant immediately.  If planting is delayed more than 6 hours after delivery, set trees and shrubs in shade, protect from weather and mechanical damage, and keep roots moist by covering with mulch, burlap or other acceptable means of retaining moisture.

F.	Do not remove container grown stock from containers until planting time.

1.05	PROJECT CONDITIONS

A.	Proceed with and complete landscape work as rapidly as portions of site become available, working within seasonal limitations for each kind of landscape work required.

B.	Determine location of underground utilities and perform work in a manner which will avoid possible damage.  Hand excavate, as required.

C.	When conditions detrimental to plant growth are encountered, such as rubble fill, adverse drainage conditions, or obstructions, notify Landscape Architect before planting.

D.	Plant trees and shrubs after final grades are established and prior to planting of lawns, unless otherwise acceptable to Landscape Architect. If planting of trees and shrubs occurs after lawn work, protect lawn areas and promptly repair damage to lawns resulting from planting operations.

1.06	SPECIAL PROJECT WARRANTY

A.	Warranty trees and shrubs, for a period of one year after date of substantial completion, against defects including death and unsatisfactory growth, except for defects resulting from neglect by Owner, abuse or damage by others, or unusual phenomena or incidents which are beyond landscape installer's control.

B.	Remove and replace trees, shrubs, or other plants found to be dead or in unhealthy condition during warranty period.  Make replacements during growth season following end of warranty period.  Replace trees and shrubs which are in doubtful condition at end of warranty period; unless, in opinion of Landscape Architect, it is advisable to extend warranty period for a full growing season.

C.	Another inspection will be conducted at end of extended warranty period to determine acceptance or rejection.  Only one replacement per tree, shrub or plant will be required at end of warranty period, except for losses or replacements due to failure to comply with specified requirements.

D.	This warranty shall not deprive the Owner of other rights or remedies that the Owner may have under other provisions of the Contract Documents and is in addition to and runs concurrent with other warranties made by the Contractor under requirements of the Contract Documents.

## PART 2 - PRODUCTS

2.01	TOPSOIL

A.	Topsoil for landscape work is not available at site and shall be furnished as specified.

B.	Provide new topsoil which is fertile, friable, natural loam, surface soil, reasonably free of subsoil, clay lumps, brush, weeds and other litter, and free of roots, stumps, stones larger than 2" in any dimension, and other extraneous or toxic matter harmful to plant growth.

C.	Obtain topsoil from local sources or from areas having similar soil characteristics to that found at project site.  Obtain topsoil only from naturally, well drained sites where topsoil occurs in a depth of not less than 4"; do not obtain from bogs or marshes.

D.	Perform chemical and mechanical analysis on topsoil.  Topsoil shall contain at least 5% organic matter determined by loss of ignition of moisture-free samples dried in accordance with the current method of the Association of Official Agricultural Chemists.  The acidity range shall be pH 6.5 to pH 7.0 inclusive.  The chemical analysis shall state the percentages of nitrogen, phosphoric acid and potash.   The mechanical analysis of the soil shall be:

E.

PASSING                   RETAINED ON        PERCENTAGE

1" Screen                                                100%

1" Screen                   1/4" screen            (Gravel not more than)   3%

1/4" Screen               #100 USS sieve       (Course, medium &   fine sand)  40-60%

2.02	SOIL AMENDMENTS

A.	Lime:  ASTM C 60-2, Class T, agricultural limestone containing a minimum of 80% calcium carbonate equivalent, with a minimum 99 percent passing a No. 8 (2.36 mm) sieve and a minimum 75% passing a No. 60 (250 micrometer) sieve.  Provide lime in the form of dolomitic limestone.

B.	Aluminum Sulfate:  Commercial grade, unadulterated.

C.	Sand:  Clean, washed sand, free of toxic materials.

D.	Perlite:  Horticultural perlite, soil amendment grade.

E.	Peat Humus:  Finely divided or granular texture, with a pH range of 6 to 7.5, composed of partially decomposed peat moss (other than sphagnum), peat humus, or reed-sedge peat.

F.	Peat Humus:  for acid-tolerant trees and shrubs, provide peat moss, with a pH range of 3.2 to 4.5, coarse fibrous texture, medium-divided sphagnum peat moss or reed-sedge peat.

G.	Manure:  Well rotted, unleached stable or cattle manure containing not more than 25% by volume of straw, sawdust or other bedding materials and containing no chemicals or ingredients harmful to plants.

2.03	FERTILIZER

A.	Bonemeal:  Commercial, raw, finely ground; 4% nitrogen and 20% phosphoric acid.

B.	Superphosphate:  Soluble mixture of treated minerals; 20% available phosphoric acid.

C.	Commercial Fertilizer:  Complete fertilizer of neutral character, with some elements derived from organic sources and containing not less than 5% total nitrogen, 10% available phosphoric acid and 5% soluble potash.

2.04	MULCHES

A.	Organic mulch free from deleterious materials and suitable for top dressing of trees, shrubs or plants and consisting of finely ground bark, dark in color.  Submit 5 gallon sample for approval.

B.	Peat Mulch:  Provide peat moss in natural, shredded, or granulated form, of fine texture, with a pH range of 4 to 6 and a water absorbing capacity of 1100 to 2000%.

2.05	PLANT MATERIALS

A.	Deciduous Trees:  Provide trees of a minimum height and caliper scheduled or shown and with branching configuration recommended by ANSI Z60.1 for type and species required.  Provide single stem trees except where special forms are shown or listed.

1.	Provide only balled and burlapped deciduous trees.

B.	Deciduous Shrubs:  Provide shrubs of the height shown or listed and with not less than minimum number of canes required by ANSI Z60.1 for type and height of shrub required.

1.	Provide Container grown deciduous shrubs subject to specified limitations for container grown stock.

D.	Coniferous and Broadleafed Evergreens:  Provide evergreens of sizes shown or listed.  Dimensions indicate minimum spread for spreading and semi-spreading type evergreens and height for other types, such as globe, dwarf, cone, pyramidal, broad upright, and columnar.  Provide normal quality evergreens with well balanced form complying with requirements for other size relationships to the primary dimension shown.

1.	Provide only balled and burlapped evergreen trees.

2.	Provide Container grown deciduous shrubs subject to specified limitations for container grown stock.

2.06	MISCELLANEOUS LANDSCAPE MATERIALS

A.	Anti-Desiccant:  Emulsion type, film forming agent designed to permit transpiration but retard excessive loss of moisture from plants.  Deliver in manufacturer's fully identified containers and mix in accordance with manufacturer's instructions.

B.	Wrapping:  Tree wrap tape not less than 4" wide, designed to prevent bore damage and winter freezing.

C.	Stakes and Guys:  Provide stakes and deadmen of sound new hardwood, treated softwood, free of knot holes and other defects.  Provide wire ties and guys of 2 strand, twisted, pliable galvanized iron wire not lighter than 12 ga. with zinc coated turnbuckles.  Provide not less than 1/2" diameter rubber or plastic  hose, cut to required lengths and of uniform color, material and size to protect tree trunks from damage by wires.

## PART 3	EXECUTION

3.01	PREPARATION

A.	Layout individual tree, shrub and groundcover locations and areas for multiple plantings.  Stake locations and outline areas and secure Landscape Architect's acceptance before start of planting work.  Make minor adjustments as requested.  Notify the Landscape Architect at least five (5) working days prior to placement of all plantings.

B.	Preparation of planting soil:

1.	Before mixing, clean topsoil of roots, plants, sods, stones, clay lumps, and other extraneous materials harmful or toxic to plant growth.

2.	Planting mixture:

a.	Provide planting mixture consisting of 7 parts topsoil to 1 part humus.

b.	Incorporate ground limestone with the topsoil at the following rate per cubic yard of topsoil used in the planting mixture, depending on the pH value shown by the chemical analysis of the topsoil.

pH              Rate lbs.

4.0 to 4.5         7

4.5 to 5.0         5

5.0 to 5.5         3

5.5 to 6.0         1

over 6.0           0

c.	Incorporate 1 pound fertilizer per cubic yard of topsoil.

3.	For pit and trench type backfill, mix planting soil prior to backfilling and stockpile at site.

4.	For planting beds, mix planting soil either prior to planting or apply on surface of topsoil and mix thoroughly before planting.

a.	Mix lime with dry soil prior to mixing of fertilizer.

b.	Prevent lime from contacting roots of acid loving plants.

c.	Apply phosphoric acid fertilizer, other than that constituting a portion of complete fertilizers, directly to subgrade before applying planting soil and tilling.

C.	Preparation of planting beds:

1.	Loosen subgrade of planting bed areas to a minimum depth of 6" using a cultimulcher or similar equipment.  Remove stones over 1-1/2" in any dimension, sticks, stones, rubbish and other extraneous matter.

2.	Spread planting soil mixture to a minimum depth required to meet lines, grades and elevations shown, after light rolling and natural settlement.  Place approximately 1/2 of total amount of planting soil required.  Work into top of loosened subgrade to create a transition layer, then place remainder of the planting soil.

D.	Excavation for trees and shrubs, and groundcover beds.

1.	Excavate pits and beds with vertical sides and with bottom of excavation slightly raised at center for trees, to provide proper drainage.  Loosen hard subsoil in bottom of excavation.

a.	For balled and burlapped trees and shrubs, make excavations at least half again as wide as the ball diameter and equal to the ball depth, plus allowance for setting of ball on a 3" setting layer of planting soil mixture.

b.	For container grown stock, excavate as specified for balled and burlapped stock, adjusted to size of container width and depth.

2.	Dispose of subsoil removed from landscape excavations offsite, or as directed by the Owners’ Representative. Do not mix with planting soil or use as backfill without prior approval of the Landscape Architect.

3.	Fill excavations for trees and shrubs with water and allow to percolate out before planting.

3.02	PLANTING TREES, SHRUBS & GROUNDCOVER

A.	Set balled and burlapped stock on layer of compacted planting soil mixture, plumb and in center of pit or trench with top of ball at same elevation as adjacent finished landscape grades.  Remove burlap from sides of balls; retain on bottoms.  When set, place additional backfill around base and sides of ball, and work each layer to settle backfill and eliminate voids and air pockets.  When excavation is approximately 2/3 full, water the pits thoroughly before placing remainder of backfill.  Repeat watering until no more is absorbed.  Water again after placing final layer of backfill.

B.	Create an earthen saucer around each tree pit to allow for mulching, as shown on the drawings.

C.	Mulch all pits trenches and planted areas.  Provide not less than 4" thickness of mulch and work into top of backfill and finish level with adjacent finish grades.

D.	Apply anti-desiccant using power spray to provide an adequate film over trunks, branches, stems, twigs and foliage.

E.	If deciduous trees or shrubs are moved in full leaf, spray with anti-desiccant at nursery before moving and again 2 weeks after planting.

F.	Prune, thin out and shape trees and shrubs only as directed by the Landscape Architect. All pruning shall be completed in accordance with standard horticultural practices.  Prune trees to retain required height and spread.  Unless otherwise directed by the Landscape Architect, do not cut tree leaders, and remove only injured or dead branches from flowering trees, if any.  Prune shrubs to retain natural character.

G.	Plants damaged by unapproved or improper methods shall be removed and replaced with new plant material at the contractor’s expense.

H.	Wrap tree trunks of 2" caliper and larger.  Start at ground and cover trunk to height of first branches and securely attach.  Inspect tree trunks for injury, improper pruning and insect infestation and take corrective measures before wrapping.

I.	Guy and stake trees immediately after planting, as indicated.

3.03	MAINTENANCE

A.	Begin maintenance immediately after planting.

B.	Maintain trees, shrubs and other plants until final acceptance but in no case less than 90 days after Substantial Completion.

C.	Maintain trees, shrubs and other plants by cultivating and weeding as required for healthy growth.  Restore and maintain planting saucers. Tighten and repair stake and guy supports and reset trees and shrubs to proper grades or vertical position as required.  Restore or replace damaged wrappings.  Spray as required to keep trees and shrubs free of insects and disease.

3.04	CLEANUP AND PROTECTION

A.	During landscape work, keep pavements clean and work area in an orderly condition.

B.	Protect landscape work and materials against damage from landscape operations, operations by other contractors and trades and vandalism. Maintain protection during installation and maintenance periods.  Treat, repair or replace damaged landscape work as directed.

3.05	INSPECTION AND ACCEPTANCE

A.	When landscape work is 100% complete, including maintenance, Landscape Architect will, upon request, make an inspection to determine acceptability.  Landscape work may be inspected for acceptance in portions agreeable to Landscape Architect and the owner, provided work offered for inspection is complete, including maintenance.

B.	Where inspected landscape work does not comply with requirements, replace rejected work and continue specified maintenance until re-inspected by Landscape Architect and found to be acceptable.  Remove rejected plants and materials promptly from project site.

# END OF SECTION

02920 - LAWNS

PART 1	GENERAL

1.01	SUMMARY

A.	Seed all areas disturbed by the construction process.

1.02	DEFINITIONS

A.	Weeds:  Includes Dandelion, Jimsonweed, Quackgrass, Horsetail, Morning Glory, Rush Grass, Mustard, Lambsquarter, Chickweed, Cress, Crabgrass, Canadian Thistle, Nutgrass, Poison Oak, Blackberry, Tansy Ragwort, Bermuda Grass, Johnson Grass, Poison Ivy, Nut Sedge, Nimble Will, Blindweed, Bent Grass, Wild Garlic, Perennial Sorrel and Brome Grass.

1.03	QUALITY ASSURANCE

A.	Lawn work shall be performed by a single firm specializing in lawn work.

B.	Ship materials with certificates of inspection required by governing authorities.

C.	Package standard products with manufacturer's certified analysis.  For other materials, provide analysis by recognized laboratory made in accordance with methods established by the Association of Official Agricultural Chemists, wherever applicable.

1.04	SUBMITTALS

A.	Submit certificates of inspection as required by governmental authorities.   Submit manufacturer's or vendors certified analysis for soil amendments and fertilizer materials.  Submit other data substantiating that materials comply with specified requirements.

B.	Submit seed vendor's certified statement for each grass seed mixture required, stating botanical and common name, percentage by weight, and percentages of purity, germination, and weed seed for each grass seed species.

C.	Submit typewritten instructions recommending procedures to be established by Owner for maintenance of lawns for one full year.  Submit prior to expiration of required maintenance periods.

1.05	DELIVERY, STORAGE AND HANDLING

A.	Deliver packaged materials in containers showing weight, analysis and name of manufacturer.  Protect materials from deterioration during delivery, and while stored at site.

1.06	JOB CONDITIONS

A.	Lawns shall be seeded from March 15th to May 15th and from August 15th to October 1st, or as weather permits.

1.07	SPECIAL PROJECT WARRANTY

A.	Contractor shall warranty lawns for a period of one year following the grass establishment period.  At any time within the warranty period, replace any lawn which, for any reason has died or is in a dying condition or has failed to flourish to such a degree that its usefulness or appearance has been impaired.  The decision of the Landscape Architect as to the necessity of replacing any lawn shall be conclusive and binding upon the Contractor.  Replacement of lawn or individual part shall be made only once and thereafter the responsibility for such lawn shall lie with the Owner.

B.	During the warranty period, from time to time, inspect the watering and other maintenance practices carried on by the Owner and promptly report to the Owner any practices which are considered unsatisfactory and not good horticultural practices.  The failure of the Contractor to inspect or report shall be construed as an acceptance by him of the Owner's maintenance practices and he shall not thereafter claim that any defects which may later develop are the results of such practices.

C.	This warranty shall not deprive the Owner of other rights or remedies that the Owner may have under other provisions of the Contract Documents and is in addition to and runs concurrent with other warranties made by the Contractor under requirements of the Contract Documents.

PART 2 - PRODUCTS

2.01	TOPSOIL

A.	Topsoil for lawns is not available at site and shall be furnished as specified.

B.	Provide new topsoil which is fertile, friable, natural loam, surface soil, reasonably free of subsoil, clay lumps, brush, weeds and other litter, and free of roots, stumps, stones larger than 2" in any dimension, and other extraneous or toxic matter harmful to plant growth.

C.	Obtain topsoil from local sources or from areas having similar soil characteristics to that found at project site.  Obtain topsoil only from naturally, well drained sites where topsoil occurs in a depth of not less than 4"; or as shown on the drawings, which ever is greater.  Do not obtain from bogs or marshes.

D.	Perform chemical and mechanical analysis on topsoil.  Topsoil shall contain at least 3% organic matter determined by loss on ignition of moisture-free samples dried in accordance with the current method of the Association of Official Agricultural Chemists.  The acidity range shall be pH 6.5 to pH 7.0 inclusive.  The chemical analysis shall state the percentages of nitrogen, phosphoric acid and potash.  The mechanical analysis of the soil shall be:

PASSING            RETAINED ON             PERCENTAGE

1" screen                100%

1" screen              1/4" screen             (gravel not more than)      3%

1/4" screen         #100 USS sieve         (course, medium & fine sand)  40 - 60%

2.02	SOIL AMENDMENTS

A.	Lime:  Natural dolomitic limestone containing not less than 85% of total carbonates, with a minimum of 30% magnesium carbonates, ground so that not less than 90% passes a 20 mesh sieve and not less than 50% passes a 100 mesh sieve.

B.	Aluminum Sulfate:  Commercial grade.

C.	Straw Mulch:  Clean oat or wheat straw well seasoned before bailing, free from mature seed-bearing stalks or roots of prohibited or noxious weeds.

D.	Wood Cellulose Fiber Mulch:  Degradable green dyed wood cellulose fiber or 100% recycled long fiber pulp, free from weeds or other foreign matter toxic to seed germination and suitable for hydromulching.

E.	Tackifier:  Liquid concentrate diluted with water forming a transparent 3-dimensional film-like crust permeable to water and air and containing no agents toxic to seed germination.

F.	Asphaltic Emulsion Binder:  Refined petroleum asphalt emulsified in alkaline water without use of clay, starch, or like deleterious substances, and not more than 75% of saponifiable acids.  Binder shall be of a fluid consistency with no petroleum solvents or other diluting agents toxic to seed germination.

G.	Fertilizer:  Shall be granular, non-burning packaged material composed of not less than 50% organic slow acting, guaranteed analysis by manufacturer.  One quarter of the nitrogen shall be in the form of nitrates, one quarter in the form of ammonia salts and one half in the form of organic nitrogen.  Available phosphoric acid shall be derived from super-phosphate having a minimum guaranteed analysis of 20% available phosphate.  The potash shall be in the form of sulphate of phosphate.

1.	Type A:  Starter fertilizer containing 18% nitrogen, 24% phosphoric acid, and 6% potash by weight or similar approved composition.

2.	Type B:  Top dressing fertilizer containing 31% nitrogen, 3% phosphoric acid and 10% potash by weight or similar approved composition.

2.03	GRASS MATERIALS

A.	Grass Seed:  Provide fresh, clean, new crop seed complying with tolerance for purity and germination established by Official Seed Analysts of North America.  Provide seed mixture composed of grass species, proportions and minimum percentages of purity, germination, and maximum percentage of weed seed, as specified.

Proportion                                          Min.%    Min.%     Max.%

by Weight   Common Name               Germ.    Pure Sd. Weed Sd.

50%     Kentucky Bluegrass             80          85           0.50

30%     Chewings Red Fescue         85          98            0.50

10%     Perennial Ryegrass             90          98            0.50

10%     Redtop                                85          92            1.00

## PART 3 - EXECUTION

3.01	PREPARATION

A.	Preparation for planting lawns:

1.	Loosen subgrade of lawn areas to a minimum depth of 4".  Remove stones over 1-1/2" in any dimension, sticks, roots, rubbish and other extraneous matter.  Limit preparation to areas which will be planted promptly after preparation.

2.	Spread topsoil to 4" minimum depth or greater as required to meet lines, grades and elevations shown, after light rolling and natural settlement.  Add specified soil amendments and mix thoroughly into upper 4" of topsoil.

3.	Where lawns are to be planted in areas that have not been altered or disturbed by excavating, grading, or stripping operations, prepare soil for lawn planting as follows; till to a depth of not less than 6"; except under the canopy of existing trees, where soil shall be tilled by hand, to a depth of 1.5” to 2.0”; apply soil amendments and initial fertilizers as specified; remove high areas and fill in depressions; till soil to a homogeneous mixture of fine texture, free of lumps, clods, stones, roots and other extraneous matter.

4.	Prior to preparation of unchanged areas, remove existing grass, vegetation and turf.  Dispose of such material off site; do not turn over into soil being prepared for lawns.

B.	Fertilizer:  Shall be incorporated into hydroseeding procedure as required and indicated on the Drawings.  Where hydroseeding application is not used, use the following guidelines:

1.	Apply Type A fertilizer to indicated turf areas at the rate equal to 1.0 lb. of actual nitrogen per 1000 square feet.

2.	Apply fertilizers by mechanical rotary or drop type distributor and thoroughly and evenly incorporate with soil to a depth of 3" by discing or other approved method.  Fertilize areas inaccessible to power equipment with hand tools and incorporate into soil.

C.	Moisten prepared lawn areas before planting if soil is dry.  Water thoroughly and allow surface moisture to dry before planting lawns.  Do not create a muddy soil condition.

D.	Restore lawn areas to specified condition if eroded or otherwise disturbed after fine grading and prior to planting.

3.02	SEEDING NEW LAWNS

A.	Do not use wet seed or seed which is moldy or otherwise damaged in transit or storage.

B.	Sow seed using a spreader or seeding machine.  Do not seed when wind velocity exceeds 5 mph.  Distribute seed evenly over entire area by sowing equal quantity in 2 directions at right angles to each other.

C.	Sow not less than 4 pounds per 1,000 square feet.

D.	Rake seed lightly into top 1/8" of soil, roll lightly, and water with a fine spray.

E.	Protect seeded areas against erosion by spreading specified lawn mulch after completion of seeding operations.  Spread uniformly to form a continuous blanket not less than 1-1/2" loose measurement over seeded areas.

F.	Anchor mulch by spraying with asphalt emulsion at the rate of 10 to 13 gallons per 1000 sq. ft.  Take precautions to prevent damage or staining of construction or other plantings adjacent to mulched areas.

3.03	HYDROSEEDING NEW LAWNS

A.	As an alternate method to mechanical seeding, Contractor may hydroseed.

B.	Use a hydro-mulcher (sprayer) and apply mixtures at rates proportionally equivalent to mechanical seeding techniques.  Mix in accordance with hydro-mulcher manufacturer's recommendations.  Protect all buildings, paving, plantings and all non-seeded areas from liquid tackifier and asphaltic emulsion over-spray.  Adjust for soil conditions are necessary.

3.04	RECONDITIONING EXISTING LAWNS

A.	Recondition existing lawn areas damaged by Contractor's operations including storage of materials and equipment and movement of vehicles.  Recondition existing lawn areas where minor re-grading is required.

B.	Provide fertilizer, seed or sod and soil amendments as specified for new lawns and as required to provide a satisfactorily reconditioned lawn.  Provide new topsoil as required to fill low spots and meet new finish grades.

C.	Cultivate bare and compacted areas thoroughly to provide a satisfactory planting bed.

D.	Remove diseased and unsatisfactory lawn areas, do not bury into soil.  Remove topsoil containing foreign materials resulting from Contractor's operations including oil drippings, stone, gravel and other loose building materials.

E.	Where substantial lawn remains, but is thin, mow, rake, aerate if compacted, fill low spots, remove humps and cultivate soil, fertilize, and seed.  Remove weeds before seeding or, if extensive, apply selective chemical weed killers as required.  Apply a seedbed mulch, if required, to maintain moist condition.

F.	Water newly planted areas and keep moist until new grass is established.

3.06	MAINTENANCE

A.	Grass Establishment Period:  Period is defined as extending from completion of seeding work until grass has been mowed three times.

B.	The following work shall be performed during the grass establishment period:

1.	Watering, spot weeding, mowing, applications of herbicides, fungicides, insecticides, and reseeding until a full, uniform stand of grass free of weeds, undesirable grass species, disease and insects is achieved and accepted by the Landscape Architect.

2.	Water daily, or as needed to maintain adequate surface soil moisture for proper seed germination.  Continue daily watering for not less than 30 days.  Thereafter apply 1/2" of water twice weekly until acceptance.

3.	Repair, rework and reseed all rain-washed gullies and rutted areas or areas that do not germinate.

4.	Mow grass to a height of 2" when grass attains height of 3-1/2" or when growth tends to smother new seedlings, or as directed by the Landscape Architect.

5.	Apply Type B fertilizer to lawns approximately 30 days after seeding at a rate equal to 1.0 lbs. of actual nitrogen per 1000 square feet.  Apply with mechanical rotary or drop type distributor.  Thoroughly water into soil.

C.	At completion of grass establishment period, request Landscape Architect's inspection.  If a substantial area of grass has not become established (more than 25% of total new lawn area), rework areas as required and restart grass establishment period.  Upon successful completion of grass establishment period, notify Owner to begin maintaining lawns and provide written instructions for proper maintenance.

# END OF SECTION