| ##### FIELD REPORT #6-CONTINUED SRG Engineering, Inc. P. O. Box 925 Gray, ME 04039-0925 ### Tel:(207)-657-7323 Fax:(207)-657-7342 | Project No.:07-003 Date: March 28/29, 2007 Project Name: Mercy Hospital Project Location: Portland, Maine Weather Conditions: Sunny, low 40’s Contact Person(s): Steven Grant-SRG Eng. James Lanza-Gilbane Co. Butch-American Steel Mike Drew-QA Labs |  |
| --- | --- | --- |
| Discussion/Observations: Level 2 | C: Shawn Welty-KLMK Paul Stevens-SMRT Janusz Wszola (EOR)-SMRT Roger Domingo-SWC Cathy Streifel-SMRT James Lanza-Gilbane |  |
| 3/28/07 SRG Observations: (Level 2 Structural Steel): 1. Level 2 structural steel beam connections were observed to be in place with the following 2 exceptions: A. Low-end of double angle diagonal brace that spans between the edge beam on grid 1 and the adjacent beam located just “west” of line 1 (located between grids F and G) is missing gusset and related connection. Butch indicated this will be in place prior to placement of Level 2 concrete. ACTION REQUIRED: CONTRACTOR TO BE SURE MISSING CONNECTION IS PROVIDED PRIOR TO PLACEMENT OF LEVEL 2 CONCRETE. B. “South” end of W12x14 near grid F-3 is missing one bolt at lower hole. Beam is located about 3’-2” West of Grid 3. ACTION REQUIRED: CONTRACTOR TO PROVIDE MISSING BOLT OR WELD PER FABRICATOR’S CONNECTION DEISGN ENGINEER. |  |  |
| Diagrams: (Please see attached SRG Photos 1-32 and 1-19) |  |  |
| C: File |  | Signed: _________________________________ |

- 1 of 2-