SECTION 01560 - ENVIRONMENTAL PROTECTION

# PART 1 - GENERAL

1.01	DEFINITIONS OF CONTAMINANTS:

A.	 Sediment:  Soil and other debris that has been eroded and transported by runoff water.

B.	Solid Waste:  Rubbish, debris, garbage, and other discarded solid materials resulting from industrial, commercial, and agricultural operations, and from community activities.

C.	Rubbish:  A variety of combustible and noncombustible wastes such as paper, boxes, glass, crockery, metal, lumber, cans and bones.

D.	Debris:  Includes combustible and noncombustible wastes such as ashes, waste materials that result from construction or maintenance and repair work, leaves, and tree trimmings.

E.	Chemical Wastes:  Includes salts, acids, alkalies, herbicides, pesticides, and organic chemicals.

F.	Sewage:  Wastes characterized as domestic sanitary sewage.

G.	Garbage:  Refuse and scraps resulting from preparation, cooking, dispensing, and consumption of food.

H.	Oily Waste:  Includes petroleum products and bituminous materials.

  - ENVIRONMENTAL PROTECTION REQUIREMENTS:

A.	General:

1.	Provide and maintain during the life of the contract, environmental protection as defined herein.  Provide environmental protective measures as required to control pollution that develops during normal construction practice.  Provide also environmental protection measures required to correct conditions that develop during the construction of permanent or temporary environmental features associated with the project.  Comply with all federal, state, and local regulations pertaining to water, air, and noise pollution.

# PART 2 - PRODUCTS: NOT USED

# PART 3 - EXECUTION

  - PROTECTION OF NATURAL RESOURCES:

- General:

- The natural resources within the project boundaries and outside the limits of permanent work performed under this contract shall be preserved in their existing condition or restored to an equivalent or improved condition upon completion of the work.  Confine construction activities to areas defined by the work schedule, drawings, and specifications.

- Land Resources:

- Except in areas indicated to be cleared, do not remove, cut, deface, injure, or destroy trees or shrubs without special approval of the Owner.  Do not fasten or attach ropes, cables, or guys to any existing nearby trees for anchorages unless specifically authorized.  Where such special emergency use is authorized, the Contractor shall be responsible for any resultant damage.

- Protection:

- Protect existing trees which are to remain and which may be injured, bruised, defaced, or otherwise damaged by construction operators.  Remove displaced rocks from uncleared areas.  Protect monuments, markers and works of art.

- Repair and Restoration:

- Repair or restore to their original condition all trees or other landscape features scarred or damaged by the equipment operations.  Obtain approval of the repair or restoration from the Owner prior to its initiation.

- Temporary Construction:

1.	Obliterate all signs of temporary construction facilities such as haul roads, work areas, structures, foundations of temporary structures, stockpiles of excess or waste materials, and all other vestiges of construction.  Temporary roads, parking areas, and similar temporary use areas shall be graded in conformance with surrounding areas, tilled, and seeded. Include topsoil or nutriment during the seeding operation as necessary to establish a suitable stand of grass.

F.	Water Resources:

- Perform all work in such a manner that any adverse environmental impact on water resources is reduced to a level acceptable to the Owner.

- Oil Substances:

1.	Take special measures to prevent oily or hazardous substances from entering the ground, drainage areas or local bodies of water.  Surround all temporary fuel oil, petroleum, or liquid chemical storage tanks with a temporary berm of sufficient size and strength to contain the contents of the tanks in the event of content leakage or spillage.

- Fish and Wildlife Resources:

1.	During the performance of the work take such steps as required to prevent interference or disturbance to fish and wildlife.  Do not alter water flows or otherwise significantly disturb native habitat adjacent to the project area which are critical to fish and wildlife except as may be indicated or specified.

- Historical and Archaeological Resources:

1.	Carefully preserve and report immediately to the Owner all items having any apparent historical or archaeological interest which are discovered in the course of any construction activities.

3.02	EROSION AND SEDIMENT CONTROL MEASURES:

A.	Burn-off:

1.	Burn-off of ground cover is not permitted.

B.	Protection of Erodible Soils:

1.	All earthwork brought to final grade shall be immediately finished as indicated or specified. Protect immediately side slopes and back slopes upon completion of rough grading.  Plan and conduct all earthwork in such a manner as to minimize the duration of exposure of unprotected soils.

C.	Temporary Protection to Erodible Soils:

1.	Utilize the following methods to prevent erosion and control sedimentation.

D.	Mechanical Retardation and Control of Runoff:

1.	Mechanically retard and control the rate of runoff from the construction site.  This includes construction of diversion ditches, benches, and berms, to retard and divert runoff to protected drainage courses.

  - Vegetation and Mulch:

1.	Provide temporary protection on all side and back slopes as soon as rough grading is completed or sufficient soil is exposed to require protection to prevent erosion.  Such protection shall be by accelerated growth of permanent vegetation, temporary vegetation, mulching, or netting.  Stabilize slopes by hydro seeding, anchoring mulch in place, covering with anchored netting, sodding, or such contamination of these and other methods necessary for effective erosion control.

  - CONTROL AND DISPOSAL OF SOLID, CHEMICAL AND SANITARY WASTES:

- General:

1.	Pick up solid wastes and place in containers which are emptied on a regular schedule.  The preparation, cooking, and disposing of food are strictly prohibited on the project site.  Conduct handling and disposal of wastes to prevent contamination of the site and other areas.  On completion, leave areas clean and natural looking. Obliterate signs of temporary construction and activities incidental to construction of permanent work in place.

- Disposal of Rubbish, Garbage, and Debris:

1.	Dispose of rubbish, garbage and debris in accordance with the requirements specified herein.

- Sewage, Odor, and Pest Control:

- Dispose of sewage through connection to the installation sanitary sewage system.  Where such a system is not available, use chemical toilets or comparable effective units and periodically empty wastes into the installation sanitary sewerage system. Include provisions for pest control and elimination of odors.

- Chemical Wastes:

1.	Store chemical waste in corrosion resistant containers labeled to identify type of waste and date filled. Remove containers from the project site, and dispose of chemical waste in accordance with federal, state, and local regulations. For oil and hazardous material spills which may be large enough to violate federal, state, or local regulations, notify the MeARNG Environmental Specialist immediately - Telephone (207) 626-4395.

- Petroleum Products:

- Conduct fueling and lubricating of equipment and motor vehicles in a manner that affords the maximum protection against spills and evaporation.  Dispose of lubricants to be discarded and excess oil in accordance with approved procedures meeting federal, state and local regulations.

  - DUST CONTROL:

- General:

1.	Keep dust down at all times, including nonworking hours, weekends, and holidays. Sprinkle or treat with dust suppressors, the soil at the site, haul roads, and other areas disturbed by operations. Petroleum products will not be used as suppressors. No dry power brooming is permitted.  Instead use vacuuming, wet mopping, wet sweeping, or wet power brooming.  Air blowing is permitted only for cleaning of non-particulate debris, such as steel reinforcing bars.  No unnecessary shaking of bags is permitted where bagged cement, concrete mortar and plaster is used.

  - NOISE:

- General:

1.	When available, make the maximum use "low-noise-emission products" as certified by EPA. No blasting or use of explosives is permitted without written permission of the Contracting Officer and then only during designated times.

END OF SECTION 01560