SECTION 05210 - STEEL JOISTS

Copyright 1998, The American Institute of Architects (AIA)

This Section uses the term "Architect."  Change this term to match that used to identify the design professional as defined in the General and Supplementary Conditions.

Verify that Section titles referenced in this Section are correct for this Project's Specifications; Section titles may have changed.

- GENERAL

      - RELATED DOCUMENTS

        - Drawings and general provisions of the Contract, including General and Supplementary Conditions and Division 1 Specification Sections, apply to this Section.

      - SUMMARY

        - This Section includes the following:

Adjust list below to suit Project.

          - Open-web K-series steel joists.

          - KCS-type, open-web K-series steel joists.

          - Joist accessories.

        - Related Sections include the following:

List below only products and construction that the reader might expect to find in this Section but are specified elsewhere.

          - Division 4 Section "Unit Masonry Assemblies" for installing bearing plates in unit masonry.

Delete first subparagraph below if bearing plates are included in this Section.

          - Division 5 Section "Metal Fabrications" for furnishing steel bearing plates.

          - Division 9 Section "Painting" for prime painting.

      - PERFORMANCE REQUIREMENTS

        - Structural Performance: Provide connections capable of withstanding design loads within limits and under conditions indicated.

          - Design Loads: As specified on structural drawings.

Retain this Article, revised as necessary, if nonuniform, unequal, or special loading conditions invalidate SJI load or weight tables and if joist manufacturer will be required to design special joists.  Indicate locations of special joists on Drawings and identify them as "special."  SJI's "Standard Specifications Load Tables and Weight Tables for Steel Joists and Joist Girders" usually calls for loading diagrams, net uplift loads, and end moments, as applicable, to allow joist manufacturer to design special joists.

Retain paragraph above if indicating design loads for special joists on Drawings; retain paragraph and applicable subparagraphs below if including design loads here.  Revise requirements below to suit Project and add other performance and design criteria if applicable.

        - Design joists to withstand design loads with live load deflections no greater than the following:

Deflection limits in subparagraphs below are examples only.  Select deflection limits from below or insert other limits as appropriate for floor, roof, and ceiling materials.

          - Roof Joists:  Vertical deflection of 1/240 of the span.

Add other performance and design criteria that apply to special joists.  Examples may include stress level reduction for special joists that are part of a fire-resistance-rated assembly, joist extensions with nonuniformly distributed loads, or special joists with rigid end connections.

      - SUBMITTALS

        - Product Data:  For each type of joist, accessory, and product indicated.

        - Shop Drawings:  Show layout, mark, number, type, location, and spacings of joists.  Include joining and anchorage details, bracing, bridging, accessories; splice and connection locations and details; and attachments to other construction.

          - Submit a letter of certification from the material fabricator sealed by a professional engineer licensed to practice in the State of Maine attesting that all shop drawings were prepared under his direct supervision.

          - Submit design calculations from the material fabricator sealed by a professional engineer licensed to practice in the State of Maine for all joist types and associated connections.

Delete subparagraph below if bearing plates are specified in Division 5 Section "Metal Fabrications."

          - Indicate locations and details of anchorage devices and bearing plates to be embedded in other construction.

Retain subparagraph below if "Performance Requirements" Article is retained.  Delete or modify if Architect assumes or is required by law to assume design responsibility.

Retain first paragraph below if procedures for welder certification are retained in "Quality Assurance" Article.

        - Welding Certificates:  Copies of certificates for welding procedures and personnel.

        - Mill certificates signed by manufacturers of bolts certifying that their products comply with specified requirements.

        - Qualification Data:  For firms and persons specified in "Quality Assurance" Article to demonstrate their capabilities and experience.  Include lists of completed projects with project names and addresses, names and addresses of architects and owners, and other information specified.

Insert specific model code organization below or revise if report must be from another source.

      - QUALITY ASSURANCE

        - Manufacturer Qualifications:  A firm experienced in manufacturing joists similar to those indicated for this Project and with a record of successful in-service performance.

          - Manufacturer must be certified by SJI to manufacture joists complying with SJI standard specifications and load tables.

          - Assumes responsibility for engineering special joists to comply with performance requirements.  This responsibility includes preparation of Shop Drawings and comprehensive engineering analysis by a qualified professional engineer licensed to practice in the State of Maine.

Delete subparagraph above and below if manufacturer is not required to engage the services of a qualified professional engineer and if submission of a comprehensive engineering analysis is not retained in "Submittals" Article.

          - Professional Engineer Qualifications:  A professional engineer who is legally authorized to practice in the State of Maine and who is experienced in providing engineering services of the kind indicated.  Engineering services are defined as those performed for installations of joists that are similar to those indicated for this Project in material, design, and extent.

        - SJI Specifications:  Comply with SJI's "Standard Specifications Load Tables and Weight Tables for Steel Joists and Joist Girders" (hereafter, "Specifications"), applicable to types of joists indicated.

        - Steel Structures Painting Council - SSPC-PS 14.01 - Steel Joist Shop Paint System.

Delete below if no welding.  Retain "Welding Certificates" Paragraph in "Submittals" Article if below is retained.  AWS states that welding qualifications remain in effect indefinitely unless welding personnel have not welded for more than six months or there is a specific reason to question their ability.

        - Welding:  Qualify procedures and personnel according to AWS D1.1, "Structural Welding Code--Steel"; and AWS D1.3 "Structural Welding Code--Sheet Steel."

      - DELIVERY, STORAGE, AND HANDLING

        - Deliver, store, and handle joists as recommended in SJI's "Specifications."

        - Handle, transport, and store steel joists, steel joist substitutes, at the job site in a manner to prevent permanent distortion of any part or other damages affecting their structural integrity.  Replace damaged items that cannot be restored to like-new condition.  Store all items off the ground in a well-drained location protected from the weather and easily accessible for inspection and handling.

Delete this Article if steel bearing plates are specified in Division 5 Section "Metal Fabrications."

- PRODUCTS

      - MATERIALS

See SJI's "Specifications" for a current list of ASTM standards for steel.

        - Steel:  Comply with SJI's "Specifications" for chord and web members.

Delete paragraph below if bearing plates are specified in Division 5 Section "Metal Fabrications."

ASTM A 307 defines the term "studs" to include stud stock and threaded rods.

        - Welding Electrodes:  Comply with AWS standards.

All structural steel exposed in the building shall be sandblasted and shop painted with primer paint - Fabricator’s Standard Lead – and Chromate-Free, Non-Asphaltic, Rust-inhibiting Primer All structural steel exposed in the building shall be sandblasted and shop painted with primer paint - Fabricator’s Standard Lead – and Chromate-Free, Non-Asphaltic, Rust-inhibiting Primer, except that paint shall conform to AISC specifications and shall be suitable for top coating. (At areas where structural steel is scheduled to receive a finish coat, verify compatibility of primer)All structural steel exposed in the building shall be sandblasted and shop painted with primer paint - Fabricator’s Standard Lead – and Chromate-Free, Non-Asphaltic, Rust-inhibiting Primer, except that paint shall conform to AISC specifications and shall be suitable for top coating. (At areas where structural steel is scheduled to receive a finish coat, verify compatibility of primer).All structural steel exposed in the building shall be sandblasted and shop painted with primer paint - Fabricator’s Standard Lead – and Chromate-Free, Non-Asphaltic, Rust-inhibiting Primer, except that paint shall conform to AISC specifications and shall be suitable for top coating. (At areas where structural steel is scheduled to receive a finish coat, verify compatibility of primer).All structural steel exposed in the building shall be sandblasted and shop painted with primer paint - Fabricator’s Standard Lead – and Chromate-Free, Non-Asphaltic, Rust-inhibiting Primer, except that paint shall conform to AISC specifications and shall be suitable for top coating. (At areas where structural steel is scheduled to receive a finish coat, verify compatibility of primer).All structural steel exposed in the building shall be sandblasted and shop painted with primer paint - Fabricator’s Standard Lead – and Chromate-Free, Non-Asphaltic, Rust-inhibiting Primer, except that paint shall conform to AISC specifications and shall be suitable for top coating. (At areas where structural steel is scheduled to receive a finish coat, verify compatibility of primer).All structural steel exposed in the building shall be sandblasted and shop painted with primer paint - Fabricator’s Standard Lead – and Chromate-Free, Non-Asphaltic, Rust-inhibiting Primer, except that paint shall conform to AISC specifications and shall be suitable for top coating. (At areas where structural steel is scheduled to receive a finish coat, verify compatibility of primer).Delete this Article if priming is not required or is specified in other Sections.  Select one of two paragraphs below.  SJI's "Specifications" permits two types of red-oxide primers or other types of primers complying with these performance requirements.  Verify that primer is compatible with fireproofing if required.  See Evaluations.

Although FS TT-P-636 has been replaced with FS TT-P-664, SJI has not changed its Specifications.  Verify availability of primer below with manufacturers.  Verify that 420 g/L of VOCs permitted by FS TT-P-664 do not exceed jurisdiction limits.

Add proprietary or special primers if preferred.  Some joist manufacturers can shop apply these special primers; others will need to ship joists to a painting facility.

      - OPEN-WEB K-SERIES STEEL JOISTS

Retain this Article if Project uses open-web K-series steel joists.

        - Manufacture steel joists according to "Standard Specifications for Open Web Steel Joists, K-Series," in SJI's "Specifications," with steel-angle top- and bottom-chord members, underslung ends, and parallel top chord; of joist type indicated.

          - Joist Type:  K-series steel joists and KCS-type K-series steel joists.

        - Comply with AWS requirements and procedures for shop welding, appearance, quality of welds, and methods used in correcting welding work.

SJI advises proportionately reducing carrying capacity for reduction in chord area.

Delete paragraph above or below if not required.  SJI distinguishes between Type S and Type R joist extensions to K-series joists.  See Evaluations and SJI's "Specifications."

Retain paragraph above or below.  SJI's "Specifications" states that camber is optional with K-series steel joist manufacturer and lists camber that varies according to top-chord length.

        - Camber joists according to SJI's "Specifications."

SJI advises proportionately reducing carrying capacity for reduction in chord area.

SJI's "Specifications" lists long-span joist manufacturer's standard camber that varies according to top-chord length.

      - JOIST ACCESSORIES

Retain one of three paragraphs below.  Bridging refers to permanent bridging.

        - Bridging:  Schematically indicated.  Detail and fabricate according to SJI's "Specifications."

Retain subparagraph below with any paragraph selected above.

          - Furnish additional erection bridging if required.

Retain paragraph above or below if joists bear on masonry or concrete construction.  Delete above and retain below if bearing plates are specified in Division 5 Section "Metal Fabrications."

Retain paragraph below if ceiling will be attached to bottom of joists.  See Evaluations.

        - Supply miscellaneous accessories, including splice plates and bolts required by joist manufacturer to complete joist installation.

      - CLEANING AND SHOP PAINTING

Retain this Article if shop cleaning and priming are required.

        - Clean and prime steel joists steel joist substitutes and steel trusses in accordance with SSPC-PS 14.01, Steel Joist Shop Paint System, except that paint shall conform to SJI specifications and shall be suitable for top coating.  (At areas where steel joists, steel joist substitutes, and steel trusses are scheduled to receive a finish top coating, verify compatibility of primer.

          - Use Lead – and Chromate-Free, Non-Asphaltic, Rust-inhibiting Primer.

Retain paragraph below if priming is not permitted or is not permitted in selected locations.  Standard fabrication practice is to prime paint joists.

SSPC-Paint 15 requires the minimum paint film thickness in paragraph below.  Verify that joist manufacturer complies with this minimum.  Revise to a lesser thickness if permitted.

        - Apply one shop coat of primer to joists, joist accessories and trusses to be primed to provide a continuous, dry paint film not less than 1 mil thick.

Delete paragraph above and retain below if painting is specified in Division 9 Section "Painting."  Revise title of Division 9 Section if applicable.

- EXECUTION

      - EXAMINATION

        - Examine supporting substrates, embedded bearing plates, and abutting structural framing, with Installer present, for compliance with requirements for installation tolerances and other conditions affecting performance.

        - Proceed with installation only after unsatisfactory conditions have been corrected.

      - INSTALLATION

        - Do not install joists until supporting construction is in place and secured.

        - Install joists and accessories plumb, square, and true to line; securely fasten to supporting construction according to SJI's "Specifications," joist manufacturer's written recommendations, and requirements in this Section.

          - Before installation, splice joists delivered to Project site in more than one piece.

          - Space, adjust, and align joists accurately in location before permanently fastening.

          - Install temporary bracing and erection bridging, connections, and anchors to ensure that joists are stabilized during construction.

SJI's "Specifications" cautions that a rigid connection of bottom chord to column be made only after application of dead loads.  Revise subparagraph below to suit Project.  Add other limitations as required.

          - Delay rigidly connecting bottom-chord extensions to columns or supports until dead loads have been applied.

Retain paragraph below if field welding is required.

        - Field weld joists to supporting steel bearing plates.  Coordinate welding sequence and procedure with placement of joists.  Comply with AWS requirements and procedures for welding, appearance and quality of welds, and methods used in correcting welding work.

Retain welding and bolting as Contractor's option in paragraph above and below, if applicable.

        - Bolt joists to supporting steel framework using carbon-steel bolts as required by SJI “Specifications”..

Retain paragraph above or below if bolting is required.

        - Install and connect bridging concurrently with joist erection, before construction loads are applied.  Anchor ends of bridging lines at top and bottom chords if terminating at walls or beams.

      - REPAIRS AND PROTECTION

Retain galvanizing repair paint if repairs to galvanized bolts or bearing plates are required.

Retain paragraph and subparagraphs below if on-site paint repair is in this Section.  Touchup painting may be unnecessary if building is closed-in immediately and building in-service conditions require no permanent paint protection.

        - Touchup Painting:  After installation, promptly clean, prepare, and prime or reprime field connections, rust spots, and abraded surfaces of prime-painted joists and accessories, bearing plates and abutting structural steel.

          - Clean and prepare surfaces by hand-tool cleaning, SSPC-SP 2, or power-tool cleaning, SSPC-SP 3.

          - Apply a compatible primer of the same type as the shop primer used on adjacent surfaces.

Delete paragraph and subparagraphs above and retain paragraph below if touchup painting is required for Project but is not part of the Work of this Section.  Revise Division 9 Section title below if applicable.

        - Provide final protection and maintain conditions, in a manner acceptable to manufacturer and Installer, that ensure joists and accessories are without damage or deterioration at time of Substantial Completion.

END OF SECTION 05210