Copyright 2000, The American Institute of Architects (AIA)

SECTION 07115 - BITUMINOUS DAMPPROOFING

Verify that Section titles referenced in this Section are correct for this Project's Specifications; Section titles may have changed.

- GENERAL

      - SUMMARY

        - This Section includes cold-applied, emulsified-asphalt dampproofing applied to the following surfaces:

- Adjust list below to suit Project.

          - Exterior face of inner wythe of exterior masonry cavity walls.

- List below only products that the reader might expect to find in this Section but are specified elsewhere.

      - SUBMITTALS

        - Product Data:  For each type of product indicated.  Include recommendations for method of application, primer, number of coats, coverage or thickness, and protection course.

        - Materials containing asbestos fibers shall not be used.

      - QUALITY ASSURANCE

        - Source Limitations:  Obtain primary dampproofing materials and primers through one source from a single manufacturer.  Provide secondary materials recommended by manufacturer of primary materials.

      - PROJECT CONDITIONS

- Delete paragraph below if all applications are interior.

        - Weather Limitations:  Proceed with installation only when existing and forecasted weather conditions permit asphalt dampproofing to be performed according to manufacturers' written instructions.

- Delete below if no enclosed or interior applications are included in Project.

        - Ventilation:  Provide adequate ventilation during application of dampproofing in enclosed spaces.  Maintain ventilation until dampproofing has thoroughly cured.

- PRODUCTS

      - MANUFACTURERS

- See Editing Instruction No. 1 in the Evaluations for cautions about naming manufacturers and products.

        - Available Manufacturers:  Subject to compliance with requirements, manufacturers offering products that may be incorporated into the Work include, but are not limited to, the following:

- Retain above for nonproprietary or below for semiproprietary specification.  Refer to Division 1 Section "Product Requirements."

          - Cold-Applied, Emulsified-Asphalt Dampproofing:

            - Euclid

            - Karnak Corporation.

            - Meadows, W. R., Inc.

            - Sonneborn, Div. of ChemRex, Inc.

      - BITUMINOUS DAMPPROOFING

        - Trowel Grade:  ASTM D 1227, Type II, Class 1.

          - Available Products:

            - Sealmastic, Type 3; W. R. Meadows

            - Hydrocide 700; Sonneborn Building Products.

            - Dampproofing Asphalt Coatings Mastic; Euclid

            - Karnak 920; Karnac Chemical Corp.

        - Fibered Brush and Spray Coats:  ASTM D 1227, Type II, Class 1.

          - Available Products:

            - Sealmastic, Type 2; W. R. Meadows

            - Hydrocide 700B; Sonneborn Building Products.

            - Dampproofing Asphalt Coatings Semimastic; Euclid

            - Karnak 220; Karnac Chemical Corp.

- EXECUTION

      - EXAMINATION

        - Examine substrates, with Applicator present, for compliance with requirements for surface smoothness and other conditions affecting performance of work.

          - Begin dampproofing application only after substrate construction and penetrating work have been completed and unsatisfactory conditions have been corrected.

      - PREPARATION

        - Protection of Other Work:  Mask or otherwise protect adjoining exposed surfaces from being stained, spotted, or coated with dampproofing.  Prevent dampproofing materials from entering and clogging weep holes and drains.

        - Clean substrates of projections and substances detrimental to work; fill voids, seal joints, and apply bond breakers if any, as recommended by prime material manufacturer.

      - APPLICATION, GENERAL

        - Comply with manufacturer's written recommendations unless more stringent requirements are indicated or required by Project conditions to ensure satisfactory performance of dampproofing.

          - Apply additional coats if recommended by manufacturer or required to achieve coverages indicated.

          - Allow each coat of dampproofing to cure 24 hours before applying subsequent coats.

        - Apply dampproofing to provide continuous plane of protection on exterior face of inner wythe of exterior masonry cavity walls.

- Two subparagraphs below include some general requirements for providing continuity of plane of protection; for best results, supplement with details on Drawings.

          - Lap dampproofing at least 1/4 inch (6 mm) onto flashing, masonry reinforcement, veneer ties, and other items that penetrate inner wythe.

      - COLD-APPLIED, EMULSIFIED-ASPHALT DAMPPROOFING

- If retaining this Article, coordinate paragraphs below with applications required.

- Select paragraph above or first paragraph below if dampproofing is applied to unparged masonry foundation walls.  Trowel grade is better for rough exterior, below-grade surfaces and where thick film is required.

        - On Exterior Face of Inner Wythe of Cavity Walls: Apply one brush or spray coat at not less than 1.25 gal./100 sq. ft. (0.5 L/sq. m).

        - On structural steel  embedded in concrete or gravel : Apply one brush or spray coat at not less than 1.25 gal./100 sq. ft. (0.5 L/sq. m).

      - CLEANING

        - Remove dampproofing materials from surfaces not intended to receive dampproofing.

END OF SECTION 07115