## Memorandum

Department of Planning and Development

Planning Division

To:		Chair Lowry and Members of the Portland Planning Board

From:		Sarah Hopkins, Development Review Services Manager

Date:		October 21, 2005

Re:	October 25, 2005 Planning Board Workshop

#### Morrill’s Crossing Mixed-Use Development

Morrrill’s Crossing, LLC has requested a second Planning Board workshop to discuss its plans for the mixed-use project at Morrill’s Corner.  As the Board will recall, a conditional zone was approved for this project by the City Council earlier this year, after a substantial review by the Planning Board Community Development Committee and City Council.

At the last workshop with the Planning Board, the applicant went through the overall plan for the site, including the conditional rezoning, phasing and architecture.  The Planning Board asked that this second workshop be used to discuss the standards by which the project will be reviewed.  Associate Corporation Counsel, Penny Littell, has provided a memo to provide guidance on the authority of the Planning Board in regards to the conditional zoning and underlying zoning.

The third section of this memo, following the summary of the conditional rezoning, outlines the site plan and subdivision standards applicable to this development proposal.  Given the size and scope of the project, I propose to use the standards as a method to keep an inventory of the items submitted, questions raised, and information requested.

## II.	PROJECT BACKGROUND & DESCRIPTION

# Conditional Rezoning

Included as Attachment 1 is the executed conditional rezoning as adopted by the City Council.  The rezoning document and associated plans specify the various uses permitted on site, the phasing of development and the requirements for infrastructure improvements.

Highlights of the contract are summarized below.

- Permitted uses

  - retail

  - no fewer than ten apartments above Allen Ave. retail building

  - 18-24 townhouse units

  - up to 24 apartments to the rear of the site

  - trails, multi-purpose soccer field

  - boxing club with up to twelve events per year with attendance over 300. Parking management plan must be approved as part of site plan and updated annually.

- Phasing

  - Phase I:

- 10 apartments near Allen Ave

- all traffic improvements to Morrill’s Corner

- trail network

- loaming/seeding of future residential lot in rear

- Grocery store, all other retail, boxing club expansion

- multi-purpose field: if DEP permitting takes longer for the field, Packard has an additional year to construct the field. Field must be loamed/seeded in interim.

Note: No certificate of occupancy will be issued for any purpose until all improvements are completed.

  - Phase II

    - 18-24 townhouses, but no Certificate of Occupancy will be granted for grocery store until a building permit is issued for the first townhouse units.  Building permits for at least 18 townhouses must be issued within two years of commencement of phase II

  - Phase III

- Up to 24 apartments to rear of site

Note: If Phase III apartments are not constructed within seven years of final Certificate of Occupancy of Phase I, Packard must assign interest in parcel for future residential development to the City for $100.

Overall Project

Land Area						20.2 acres

Total square footage of total project			237,514

Total ground coverage of total project		156,136

Retail

Non-grocery					63,182

Grocery					71,445

Office

Proposed					15,462 (boxing)

Allowed by zoning				up to 25,000

Residential

Upstairs Allen Ave apartments		11

Townhouses					18-24

Rear apartments 				12

### SITE PLAN

# 1.	Traffic, Loading, and Circulation 	§14-526(a)(1)

Traffic

A Traffic Study was submitted by the applicant in preparation for the MDOT Traffic Permit Scoping Meeting. The meeting was held in September, at which time a number of questions were asked and items requested for further review.  The applicant has collected the data as requested and is currently analyzing the data for a later presentation to the Planning Board.

As discussed during the conditional rezoning, a major aspect of the traffic review includes the required roadway improvements to Morrill’s Corner.  Signalization and widening/striping for the roadway is envisioned.  Once the final traffic study is submitted, the City will work with the applicant to determine the detailed requirements of the roadway design and cross section.  There will be approximately 62 feet of right-of way along Forest Avenue to be divided into four lanes of traffic, sidewalks, and (bike) shoulders.  While travel lanes are a given at 11ft., the sidewalk/esplanade and shoulder width can be shifted.  Finding the appropriate division will be an important aspect of the review.

Access

Primary access to the site will be from Allen Avenue at a signalized driveway.  Paul White Tile will be reconfigured as indicated conceptually in the layout plans in order to provide an aligned four-way intersection at the entrance drive.

A secondary, one-way access will be provided along Morrill Street to offer neighbors an opportunity to shop at the development without driving out onto Allen Avenue.  Neighbors wishing to turn left onto Allen Ave may choose to go through the development via Morrill Street in order to turn at the proposed signal.

The applicant has also proposed an access drive from Allen Avenue Apartments onto the main entrance drive for the same purpose: to allow residents an opportunity to make left hand turns at the new signal.  Additional details on the driveway and amendment to the Allen Avenue Apartments will be needed.

Morrill Street, aside from the one-way entrance, also provides an opportunity to provide additional access to the development.  Planning Board members expressed interest in looking into other methods of access – either primary or emergency –into the development from Morrill Street.

##### Loading

Retail loading areas are provided to the rear of the supermarket and proposed retail buildings.  Due to the two-sided design of the Allen Avenue retail building, loading will most likely occur from the parking lot side of the building. Items will be brought in by hand-trucks.

An access lane for emergency access and deliveries is shown around the second retail building next to Bruno’s, between the railroad tracks and building.

The supermarket includes a loading area to the rear with four enclosed truck bays, a smaller loading platform and associated maneuvering room.  The four loading docks are designed to “enclose” the rear of the truck while it is unloaded.  Stop & Shop will be providing information on the details of this construction as well as noise readings of similar facilities during a delivery.  Such data will help inform the Board on the impacts of the use and whether restrictions may be required as to hours of operation or the idling of refrigeration trucks.

Pedestrian Circulation

A sidewalk/trail loop is shown around the perimeter of the site.  A pedestrian connection is shown to Magnolia Street, an unimproved paper street.  Pedestrian access is also anticipated from the neighborhood form Morrill Street.  The level of required improvement of Morrill Street will have to be determined as part of the Planning Board’s review.  Details and legal rights will have to be resolved as to any paper street improvement.

The Morrill’s Corner improvement plan indicates sidewalks along all frontages of Forest and Allen Avenue. As discussed above, the dimensions of sidewalks, esplanades and (bike) shoulders will have to be determined as the review continues.

Intermodal Transportation

A METRO bus stop is shown on the outgoing side of the entry drive to provide transit service within the development.  The applicant is working with METRO to confirm the location of the site and specifications for the pull-out.

# Parking								§14-526(a)(2)

666 parking spaces are proposed for the development.  As a development of over 50,000 sq ft, the Planning Board will determine the parking requirement based on information submitted by the applicant.

As part of the conditional rezoning, the applicant is required to submit a parking management plan for the boxing club and update it annually.  A parking plan is included with the Traffic Study and will be addressed at a later workshop.

##### Boxing Club Parking Management

One issue fore the Board to consider is the location of parking for the Boxing Club.  Condition # 5d of the conditional rezoning states that off-site parking shall be provided with shuttle bus property to the facility. (See bottom of page 4 of zoning document.)

Since the drafting of that requirement, the City Council did require parking to the rear of the site to serve both the boxing facility and recreation field.  The applicant has stated that this rear parking, in addition to parking located furthest away from the supermarket, will be adequate to serve the boxing club functions.

# Health and Safety						§14-526(a)(3)

# Value / Utility of Neighboring Structures			§14-526(a)(4)

# Sewers, Stormdrains, Water				§14-526(a)(5)

The applicant has submitted a utility plan which outlines the size and location of utilities including sanitary sewer, stormdrains, gas, water, underground electric and telephone.

The applicant is working with the City’s reviewing engineer, Steve Bushey of DeLuca Hoffman, and Public Works to refine and finalize the stormwater management plan.  The stormwater plan will be presented at a subsequent workshop.

# On-Site Landscaping					14-526(a)(6)

At the Planning Board’s request, the Planning Office is working to contract with an outside landscape architecture firm to provide a peer review of the landscaping plan.

# Significant Existing Vegetation  				§14-526(a)(7)

Vegetation on the site consists of scrubby growth and trees.  There will be preserved areas around the perimeter of the site and around the recreational field and detention pond.  For the most part, however, the applicant proposed to buffer the project with the installation of new plant material and fencing.

# Drainage and Erosion Control  				§14-526(a)(8)

Details submitted with the site plan include temporary and permanent erosion control.  Erosion control will be addressed during a subsequent workshop with stormwater management.

A portion of the site, known as the “Burt property” will be subject to environmental mitigation due to environmental pollution located on site.

# Exterior Lighting  						§14-526(a)(9)

A lighting plan is included with the site plans.  At the last workshop, the applicant mentioned that a waiver will be needed from the lighting standards.  Additional specifications will be needed to review the waiver request.

# Fire Prevention  						§14-526(a)(10)

Fire access to the site will be provided from Allen Avenue.  The Fire Department is currently reviewing the plans to determine the location of secondary access via Morrill or any other neighborhood street.

# Consistency with Off-Premises City Infrastructure  	§14-526(a)(11)

The proposal is located adjacent to and on top of several paper streets. Rocky Hill, a paper street running parallel below Princeton Street will have to be vacated.  Licenses or easements may be needed for proposed improvements within Morrill and Magnolia Street.

The plans show Morrill Street as providing important access to the site from the neighborhood.  The Fire Department is reviewing the plans to determine whether additional length will be required along Morrill Street to provide emergency access to the rear apartments and recreational field.

12. PRUD/Multi-family Standards				§14-526(a)(14)

# 13.	Environmental Impact					§14-526(a)(20)

Stormwater

The applicant has submitted a stormwater management study that is currently under review by Public Works and our reviewing engineer, Steve Bushey.  Based on recent discussions with Public Works, the stormwater management plan may be amended to reduce the size of the detention basin to the rear of the site.  Updated plans will be provided and reviewed prior to a subsequent engineering workshop.

Wetlands

While standing water is indicated on the plans, no wetlands have been mapped.

Stream Protection

There are no streams indicated on the plans.

DEP Site Location Review

The Department of Environmental Protection will be conducting the Site Location of Development review.

# Signage							§14-526(a)(22-23)

The architecture indicates consistent signage “fields” for each tenant.  Signage permits will be required as prior to construction.  All signage will be required to meet the B-2 requirements through zoning.

15.	B2 Standards							§14-526(a)(27)

The parcel was rezoned by the City Council as a conditional B-2 Zone.  Unless otherwise specified in the conditional rezoning document, the B-2 standards will apply.  A copy of the Site plan and Design Standards relative to B-2 projects is attached.

II.	SUBDIVISION

There are a number of land/property divisions proposed as part of the development:

- 11 apartments above the Allen Avenue retail building;

- 22 townhouse units on a separate parcel to be owned by a condominium association on the property along Princeton Street; and

- 12 apartments to the rear of the site.

The standards for subdivision review are listed below.

# Water and Air Pollution						§14-497(a)(1)

The proposed project is not anticipated to have any adverse effect on water or air quality.  The Department of Environmental Protection will be reviewing the plans under requirements for Site Location of Development review.

# Water Capacity and Supply						§14-497(a)(2-3)

The applicant will need to submit a capacity letter from the Portland Water District.

# Soil Erosion								§14-497(a)(4)

Detail submitted with the site plan include temporary and permanent erosion control.  Erosion control will be addressed during a subsequent workshop with stormwater management.

A portion of the site, known as the “Burt property” will be subject to an environmental mitigation due to environmental pollution located on site.

# Traffic									§14-497(a)(5)

The traffic study is currently being completed by the applicant. We anticipate presenting the traffic study during a December workshop.

# Sanitary and Stormwater Capacity					§14-497(a)(6)

The applicant has presented a sanitary sewer infrastructure capacity letter (See Site Plan Book).

# Sewage and Solid Waste Disposal					§14-497(a)(7)

Solid waste for the Allen Avenue retail/apartment building will be collected in a central utility room.

The site plan narrative describes the supermarket as having a 35-yard container for recyclable materials and solid waste to be replaced every 7-10 days.  It is not clear where the containers will be located—inside or outside—and whether restrictions on hours of trash collection should be required.

All waste will be contracted and hauled privately.

# Scenic Beauty								§14-497(a)(8)

The applicant is proposing to construct a recreational multi-purpose field for public access.  The Parks and Recreation Department is currently reviewing the plans and will provide specifications to the applicant.  An item to be determined will be whether the field is owned by the applicant with public access easements granted to the City or whether the City will own the field outright.

# Comprehensive Plan Conformance					§14-497(a)(9)

The proposed project was recently approved as a conditional zone. In doing so, the City Council found the development to conform to the goals and policies of the Comprehensive Plan.

# Financial and Technical Capability				§14-497(a)(10)

Included as Attachment C of the Site Plan Book is a letter submitted by Stop & Shop attesting to ample financial capability to construct the project.

# Water Bodies Protection						§14-497(a)(11)

# The project is located in the Fall Brook watershed.  A requirement of the conditional rezoning was that Packard “contribute up to $100,000 to the City’s Fall Brook Combined Sewer Overflow project, as determined by the Public Works Authority.

# Groundwater Quality						§14-497(a)(12)

# This item will be addressed during stormwater management review.

# Flood Zone								§14-497(a)(13)

# Staff has consulted current flood insurance maps and determined that the proposed project is not located within any flood prone area.

# Wetlands								§14-497(a)(14)

# Rivers, Streams, Brooks						§14-497(a)(15)

# Subdivision Technical and Design Standards			§14-498_____

The final design of Forest and Allen Avenues will be determined once the final traffic study has been submitted.  Sidewalk, esplanade, driving lanes, utility locations and shoulder width will be determined at that time.

###### The Applicant has requested a waiver of parking space dimension as required in the Technical Standards.  Stop and Shop has found that by using a double striping of parking stall lines, a 9 x 18 parking stall with 24ft wide aisles is sufficient.  The Traffic Engineer is currently reviewing the plans.

## Other Items

  - Boxing Club Size Boxing Club: Conditional Rezoning exhibit indicated a “10,000 sq ft expansion”.  The present plans show an expanded footprint of 10,000sq ft with two additional floors: one open mezzanine and one office.  A memo from Marge Schmuckal is included as Attachment 2.

  - Snow Storage

  - Milliken Brook Watershed Study

  - Street Vacations

  - Neighborhood Meeting

Attachments

- Rezoning Document

- Memo from Zoning Administrator

- Memo from Penny Littell

- Zoning Parcel Map

- Site Plans

- B-2 Technical Standards

- Correspondence from Neighbors

PLEASE BRING THE SITE PLAN APPLICATION NOTEBOOK TO THE WORKSHOP.