SECTION 013100 - PROJECT MANAGEMENT AND COORDINATION

PART 1 - GENERAL

1.1	DESCRIPTION OF WORK

A.	This section specifies general administrative procedures and performance required, including but not necessarily limited to, the following:

1.	Coordination and meetings.

2.	Administrative/supervisory personnel.

3.	Limitations for use of site.

4.	Special reports.

5.	Tradespeople and workmanship standards.

6.	Inspections, tests and reports.

7.	General installation provisions.

8.	Overtime work, suspension of work

9.	Cutting and patching.

10.	Cleaning and protection.

11.	Environmental Protection.

12.	   Work restrictions.

1.2	COORDINATION AND MEETINGS

A.	General:  Prepare and distribute to each entity performing work at project site, a written memorandum of instructions on required coordination activities, including required notices, reports and attendance at meetings.  Prepare similar memorandum for separate government contractors where interfacing of work is required.

B.	Coordination drawings:  Refer to clause "Shop Drawings, Coordination Drawings, and Schedules," of "Contract Clauses," for general requirements.

1.	Work by Separate Entities:  Where work by separate entities requires off-site fabrication of products and materials which must be accurately interfaced and closely intermeshed to produce required results, prepare coordination drawings to indicate how work shown by separate shop drawings will be interfaced, intermeshed, and sequenced for installation.  Comply with submittal requirements of "Submittals" section.

2.	Work installed without prior approval:  Any work installed prior to a required approval of coordination drawings shall be at the Contractor's risk.  Subsequent relocations required to avoid interferences shall be made without additional expense to the Government. In case interference develops, the Contracting Officer will decide which work shall be relocated, regardless of which was installed first.

C.	Coordination of Trades:  Coordinate work of different trades so that interference between mechanical, electrical, architectural, and structural work, including existing services, will be avoided and within limits indicated the maximum practical space for operation, repair, removal, and testing of equipment is provided.

D.	Preconstruction Meeting:  The Government will conduct a meeting to discuss administrative procedures with the contractor which will be scheduled to occur between the Contract Award and the Notice to Proceed.

- Weekly Coordination Meeting:  In addition to specific coordination meetings for each major element of work, and regular project meetings for other purposes (as indicated elsewhere in contract documents), hold weekly general project coordination meetings at regularly scheduled times which are convenient for everyone involved.  Request representation (at each meeting) by every entity currently involved in coordination or planning for work of the entire project.  Conduct meetings in a manner which will resolve coordination problems.  Record results of meetings and distribute copies to everyone in attendance and to others affected by decisions or actions resulting from each meeting.

1.	Review work schedule including when work is being done in which rooms for the upcoming three weeks at this meeting.  Coordinate with Government necessary staffing needs for this period.

1.3	ADMINISTRATIVE/SUPERVISORY PERSONNEL

A.	General:  In addition to a General Superintendent and other administrative and supervisory personnel required for performance of the work, provide specific coordinating personnel as specified herein.

B.	Environmental Control Officer:  The Contractor shall designate an "Environmental Control Officer" whose duties shall include the responsibility for enforcing the environmental protection provisions of these specifications including safety and health; the requirements of the Occupational Safety and Health Act; and other applicable Federal, state and local standards. As outlined in the applicable sections of this specification, submit for approval by the Contracting Officer the proposed security plan, housecleaning plan, demolition program, and safety and health plan.  After approval by the Contracting Officer, the implementation and enforcement of these programs shall become the responsibility of the Environmental Control Officer.  Any changes in the programs must be requested by the Contractor through the Environmental Control Officer for written concurrence by the Contracting Officer. NOTE: The duties of the Environmental Control Officer may be combined with the duties of another individual associated with this project who is also qualified to perform the duties of the Environmental Control Officer.

C.	Project Coordinator:  Provide a Project Coordinator, who is experienced in administration and supervision of building construction including mechanical and electrical work, and who is hereby authorized to act as the general coordinator of interfaces between units of work.  For purpose of this provision, "interface" is defined to include the scheduling and sequencing of work, sharing of access to work spaces, installations, protection of each other's work, cutting and patching, tolerances, cleaning, selections for compatibility, preparation of coordination drawings, inspections, tests, and temporary facilities and services. NOTE; The duties of the Project Coordinator may be combined with the duties of another individual associated with this project who is also qualified to perform the duties of Project Coordinator.

D.	Submittal of Staff Names, Duties:  Within 15 days of contract date, submit a listing of Contractor's principal staff assignments and consultants, naming persons and listing their addresses and telephone numbers.

1.4	LIMITATIONS FOR USE OF SITE

- General:  In addition to site utilization limitations and requirements shown on drawings, and indicated by other contract documents, administer allocation of available space equitably among entities needing access and space, so as to produce best overall efficiency in performance of total work of project.  Schedule deliveries so as to minimize space and time requirements for storage of materials and equipment on site.

  - See  specification section “Summary of Work” for specific work hour limitations.

B.	Occupancy of Premises: With the exception of the spaces being renovated, the Building will be fully and publically occupied at all times during performance of work under the contract.

C.	Parking Facilities:  Except as noted or specified, and except for one Contractor vehicle, there will be no parking available at site during normal working hours. The Contractor and his employees shall make their own arrangements for vehicle parking.

D.	Loading Dock Facilities:  The Government will need to coordinate all use of the loading dock. Coordinate need to access dock with Contracting Officer and provide a minimum of 48 hour notice for any use.

E.	Materials on the Site:  Unless otherwise noted or specified, materials and equipment that are removed and not reused under this contract become the property of the Contractor, and salvage value shall be reflected in the bid.

F.	Storage or Sale of Equipment on Site:  Storage or sale of excess salvageable material on the site is not permitted.  Remove from site.

- Areas abutting the project are suspect to significant security requirements. Provide Contracting Officer 7 days notification prior to any access to these areas including the Jetport passenger gate area. See Section 13200 for schedule documentation for added requirements

1.5	SPECIAL REPORTS

A.	General:  Except as otherwise indicated, submit special reports to the Contracting Officer within one day of occurrence requiring special report, with copy to others affected by occurrence.

B.	Reporting Accidents:  Refer to Section, "Governmental Safety Requirements," 013526.

C.	Reporting Unusual Events:  When an event of unusual and significant nature occurs at site (examples:  visit by news media; extraordinary weather phenomenon), prepare and submit a special report listing chain of events, persons participating, response by Contractor's personnel, evaluation of results or effects, and similar pertinent information.  When such events are known or predictable in advance, advise the Contracting Officer in advance at earliest possible date.

1.6	TRADESPERSONS AND WORKMANSHIP STANDARDS

A.	General:  Instigate and maintain procedures to ensure that persons performing work at site are skilled and knowledgeable in methods and craftsmanship needed to produce required quality-levels for workmanship in completed work.  Remove and replace work which does not comply with workmanship standards as specified and as recognized in the construction industry for applications indicated.  Remove and replace other work damaged or deteriorated by faulty workmanship or its replacement.

B.	Availability of Tradespersons:  At each progress or coordination meeting, review availability of tradespersons and projected needs to accomplish work as scheduled.  Require each entity employing personnel to report on events which might affect progress of work. Where possible, consider alternatives and take actions to avoid disputes and delays.

1.7	INSPECTIONS, TESTS AND REPORTS

A.	General:  Required inspection and testing services are intended to assist in determination of probable compliances of work with requirements, but do not relieve Contractor of responsibility for those compliances, or for general fulfillment of requirements of contract documents.  Specified inspections and tests are not intended to limit Contractor's quality control program.  Afford reasonable access to agencies performing tests and inspections.

B.	Qualification of Testing Agencies:  Except as otherwise indicated and except where manufacturer's testing facilities are indicated as acceptable, engage independent testing laboratories specializing in required services and acceptable to the Contracting Officer.

C.	Reports:  Refer to Section, "Submittal Procedures" 013300.

1.8	OVERTIME WORK, UTILITY SHUTDOWN, SUSPENSION OF WORK

A.	Overtime Work:

1.	Work which is required to be performed outside of normal working hours of the building occupants.

B.	Schedule shutdown of a utility at least one week in advance with the Contracting Officer or his designated representative.

PART 2 - PRODUCTS (Not Applicable)

PART 3 - EXECUTION

3.1	GENERAL INSTALLATION PROVISIONS

A.	Installer's Inspection of Conditions:  Require Installer of each major unit of work to inspect substrate to receive work, and conditions under which work will be performed, and to report (in writing to Contractor) unsatisfactory conditions.  Do not proceed with the work until unsatisfactory conditions have been corrected in a manner acceptable to Installer.

B.	Manufacturer's Instructions:  Where installations include manufactured products, comply with manufacturer's applicable instructions and recommendations for installation, to extent these are more explicit or more stringent than requirements indicated in contract documents.

C.	Inspect each item of materials or equipment immediately prior to installation, and reject damaged and defective items.

D.	Install work during conditions of temperature, humidity, exposure, forecasted weather, and status of project completion which will ensure best possible results for each unit of work, in coordination with entire work. Isolate each unit of work from noncompatible work, as required to prevent deterioration.

E.	Coordinate enclosure (closing-in) of work with required inspections and tests, so as to minimize necessity of uncovering work for that purpose.

F.	Mounting Heights:  Where mounting heights are not indicated, mount individual units of work at industry-recognized standard mounting heights for applications indicated.  Refer questionable mounting height choices to the Contracting Officer for final decision.

3.2	CLEANING AND PROTECTION

A.	General:  During handling and installation of work at project site clean and protect work in progress and adjoining work on a basis of perpetual maintenance. Apply suitable protective covering on newly installed work where reasonably required to ensure freedom from damage or deterioration at time of substantial completion; otherwise, clean and perform maintenance on newly installed work as frequently as necessary through remainder of construction period.

B.	Limiting Exposures of Work:  To extent possible through reasonable control and protection methods, supervise performance of work in a manner and by means which will ensure that none of the work, whether completed or in progress, will be subjected to harmful, dangerous, damaging, or otherwise deleterious exposures during construction period.  Such exposures include (where applicable, but not by way of limitation) static loading, dynamic loading, internal pressures, external pressures, high or low temperatures, thermal shock, high or low humidity, air contamination or pollution, water, solvents, chemicals, light, puncture, abrasion, heavy traffic, soiling, bacteria, insect infestation, combustion, electrical current, unusual wear, misuse, incompatible interface, destructive testing, misalignment, unprotected storage, improper shipping/ handling, theft and vandalism.

3.3	ENVIRONMENTAL PROTECTION

A.	Solid, Liquid, and Gaseous Contaminants:  Contractor shall be responsible for the proper disposal of all solid, liquid, and gaseous contaminants in accordance with all local codes and regulations, together with the following requirements:

1.	Discharge gaseous contaminants so that they will be sufficiently diluted with fresh air to reduce the toxicity to an acceptable level.

2.	Liquid contaminants may, subject to local utility standards, be diluted with water to a level of quality acceptable in the local sewer system, or shall be contained in approved vessels for disposal at approved sites.

B.	Disposal of Refuse:  Remove refuse resulting from construction operations from the site. Remove material in hampers in a fashion that does not disrupt daily operations of building as directed by Contracting Officer.

C.	Trucking:

1.	Load all trucks leaving the site with loose debris in a manner that will prevent dropping of materials on streets.  Fasten suitable tarpaulins over the load before they enter surrounding paved streets.  Trucks bringing loose materials over paved streets to the site shall be similarly loaded and covered.

2.	The Contractor and all subcontractors on or delivering to the site shall conform to all local regulations regarding load limits.

D.	Construction Site Maintenance:

1.	Store all supplies and equipment on project site so as to preclude mechanical and climatic damage. Maintain site in a neat and orderly manner.

2.	Contractor shall be responsible for maintaining the temporary structures and construction enclosure in good repair and visually pleasant

E.	Noise Control:

1.	Comply with all applicable state and local laws, ordinances, and regulations relative to noise control.

2.	Equipment to be employed on this site shall not produce a noise level exceeding the following limits in Db(A) at a distance of 50 feet from the equipment under test.

3.	Equipment:

- Materials Handling

- Truck 91

- Stationary

1)	pumps 76

2)	generators 78

3)	compressors 81

- Impact

1)	pneumatic tools 86

- Other

1)	saws 78

2)	vibrators 76

  - WORK RESTRICTIONS

A	Access:

  - The elevators must be available to the Jetport operations at all times.

  - Stairwells must be kept passable between occupied floors and building exits.

- Housekeeping:

  - Keep active stairwells broom clean daily.

  - Provide carpet floor mat at active stairwell landings on occupied floors.

- Safety:

  - Restrict application of products requiring respirator protection to unoccupied hours.

  - Fire Alarm, standpipe, all existing life safety components, and emergency power shall remain active throughout construction.

- Noise:

  - Prohibit any construction activity during occupied hours that generates more than 75 decibels ambient noise in occupied space.

- Power and Lighting:

  - Provide one week advance notice to Government for power outages.

  - Perform power outages that will affect occupants as directed by Contracting Officer.

END OF SECTION 013100