SECTION 013300 – SUBMITTAL PROCEDURES

PART 1 - GENERAL

1.1	SUMMARY

A.	The types of submittal requirements specified in this section include shop drawings, product data, samples, certificates of conformance or compliance, certified test or inspection reports, and miscellaneous work-related submittals.  Individual submittal requirements are specified in applicable sections for each unit of work.

B.	Requirements for record documents, progress and final photographs, and operation and maintenance manuals are covered in the "Project Close-out" section.

1.2	DEFINITIONS

A.	Work-related submittals of this section are categorized for convenience as follows:

1.	Shop drawings include specially-prepared technical data for this project, including drawings, diagrams, performance curves, data sheets, schedules, templates, patterns, reports, calculations, instructions, measurements and similar information not in standard printed form for general application to a range of similar projects.

2.	Product data include standard printed information on materials, products and systems; not specially-prepared for this project, other than the designation of selections from among available choices printed therein.

3.	Samples include both fabricated and unfabricated physical examples of materials, products and units of work; both as complete units and as smaller portions of units of work; either for limited visual inspection or (where indicated) for more detailed testing and analysis.  Mock-ups are a special form of samples, which are too large or otherwise inconvenient for handling in specified manner for transmittal of sample submittals.

4.	Certificates of conformance or compliance are documents attesting that a product complies with a specified standard.

5.	Certified test (or inspection) reports are documents attesting that a product meets a specified level of performance or quality when a prototype specimen is tested or inspected in accordance with a specified procedure, and consist of a certified statement by the product supplier or Contractor accompanied by a complete report of the inspection or test.

6.	Miscellaneous submittals related directly to the work (non-administrative) include warranties, maintenance agreements, workmanship bonds, project photographs, survey data and reports, physical work records, copies of industry standards, record drawings, field measurement data, operating and maintenance materials, overrun stock, and similar information, devices and materials applicable to the work and not processed as shop drawings, product data samples, certificates of conformance or compliance, or certified test reports.

7.	Secure FTP Site: AMEC maintains an FTP site that has been cleared by GSA as secure. Use of this site will be expected as part of the transmission of documents being submitted for approval. AMEC will provide access to one Contractor personnel upon their receipt of clearance documentation.

1.3	GENERAL SUBMITTAL REQUIREMENTS

A.	General:

1.	All submittals shall be made to the Contracting Officer or to an individual designated by the Contracting Officer, or as outlined and agreed on by the GSA during the PreConstruction Meeting.

2.	Only the Contracting Officer or an individual designated by him in writing can approve or disapprove submittals.  Deviations and variations from the contract requirements contained in the submittal can be approved only by the Contracting Officer or an individual delegated such authority in writing by the Contracting Officer.

3.	Failure on the part of the Contractor to indicate approval on submittals prior to submission to Contracting Officer will result in their being returned to the Contractor without being acted upon.

4.	No delays in construction occasioned by the Contractor's failure to submit material for approval in accordance with the approved schedule will be excused.

- It is the intent of this project to process submittals in a “paperless” fashion through the use of a  Owner approved secure FTP site. This will be reviewed and coordinated at the PreConstruction meeting.

B.	Scheduling:  Submit schedule of submissions for approval by the Contracting Officer, within 7 calendar days after notice to proceed is received, unless otherwise stated.

- All required submittals shall be submitted not later than 2 months prior to Contract Completion Date.

1.	If any required submittals are not made within the stipulated time frame, the Contractor’s monthly progress payments shall be reduced by 10% each and every month that the submittals remain outstanding. This with-holding will be released upon receipt of the outstanding submittals.

D.	Coordination and Sequencing:  Submittals shall be generated and submitted on a specification by specification basis, and all submittals required in a specific specification section shall be submitted together as one package. Each required submittal within the specification section will be numbered independently to allow the processing of individual items. Submittals not included but required at the project completion will be identified. Coordinate the preparation and processing of submittals with performance of the work so that work will not be delayed by submittals.  Coordinate and sequence different categories of submittals for same work, and for interfacing units of work, so that one will not be delayed for coordination of the Contracting Officer's review with another.

E.	Preparation of Submittals:  Provide permanent marking on each submittal to identify project, date, Contractor, subcontractor, submittal name and similar information to distinguish it from other submittals.

F.	Show Contractor's executed review and approval marking and provide space for the Contracting Officer's action marking.  Package each submittal appropriately for transmittal and handling.

- Submittals which are received from sources other than through Contractor's office will be returned without action.

- Submittals which are received but have not been reviewed and stamped  by the Contractor will be returned without review.

G.	Transmittal Form:  Shall  be on Contractor Letterhead and shall identify the Specification Section of each submittal.

1.4	SPECIFIC-CATEGORY SUBMITTAL REQUIREMENTS

A.	General:  Except as otherwise indicated in individual work sections, comply with requirements specified herein for each indicated category of submittal.  Provide and process intermediate submittals, where required between initial and final, similar to initial submittals.

B.	Shop Drawings:  Refer to clause "Shop Drawings, Coordination Drawings, and Schedules" of the Contract Clauses.  Provide newly-prepared information, on reproducible sheets, with graphic information at accurate scale (except as otherwise indicated), with name of preparer indicated (firm name).  Show dimensions and note which are based on field measurement.  Identify materials and products in the work shown.  Indicate compliance with standards, and special coordination requirements.  Do not allow shop drawing copies that have not been reviewed or have been disapproved to be used in connection with the work.

1.	Submit for Review:  Provide in PDF format and post to the secure FTP site.

2.	The Contractor shall be responsible for reproduction and distribution of reviewed shop drawings to his subcontractors.

- Product Data:  Collect required data into one submittal for each unit of work or system; and mark each copy to show which choices and options are applicable to project.  Include manufacturer's standard printed recommendations for application and use, compliance with standards, application of labels and seals, notation of field measurements which have been checked, and special coordination requirements.  Maintain one set of product data (for each submittal) at project site, available for reference.

1.	Submittals:  Contractor shall not submit product data, or allow its use on the project, until he has confirmed compliance with requirements of contract documents.  Submittal is for information and record, unless otherwise indicated.  Initial submittal is final submittal unless returned by the Contracting Officer, marked with an action which indicates an observed noncompliance.  Provide in PDF format and post to the secure FTP site.

a.	Installer's Copy:  Do not proceed with installation of materials, products or systems until final copy of applicable product data is in possession of Installer.

- Samples:  (Refer to clause "Samples" of Contract Clauses.) Provide units identical with final condition of proposed materials or products for the work.  Include "range" samples (not less than 3 units) where unavoidable variations must be expected, and describe or identify variations between units of each set.  Provide full set of optional samples on project site where the Contracting Officer's selection is required.  Prepare samples to match the Government-furnished sample where so indicated.  Include information with each sample to show generic description, source or product name and manufacturer, limitations, and compliance with standards.

1.	Submittal:  At Contractor's option, provide preliminary submittal of a single set of samples for review and action.  Otherwise, initial submittal is final submittal unless returned with action which requires resubmittal.  Submit 3 sets of samples in final submittal; one set will be returned.

2.	Quality Control Set:  Maintain returned final set of samples at project site, in suitable condition and available for quality control comparisons.

E.	Certificates of Conformance or Compliance:  Follow same procedure as for product data.  Where feasible, and/or where required by other sections of specification indicate compliance with the specified standard by means of a label on the container, or on an inconspicuous place on the product.

F.	Certified Test and Inspection Reports:  Process each as either "shop drawing" or "product data", depending upon whether report is uniquely prepared for project or a standard publication of workmanship control testing at point of production; process accordingly.

1.	Report shall include a description of the prototype specimen tested or inspected which is sufficiently descriptive to ensure positive identification of the product by an inspector when delivered and/or installed.

2.	The report shall be accompanied by a notarized statement from the supplier of the product certifying that the prototype is identical in all respects to the product proposed for the project.

- Where feasible and/or where required by other sections of the specification, indicate compliance with the specified performance or quality by means of a label on the container or on an inconspicuous place on the product.  The label shall refer to the test or inspection report and include the date of the report.

G.	Warranties (Guarantees):  In addition to copies desired for Contractor's use, furnish  in PDF format and post to the secure FTP site, except furnish additional (conformed) copies where required for maintenance manuals.

H.	Standards/Manufacturer's Recommendations:  Where copy submittal is indicated, and except where specified integrally with "Product Data" submittal, submit in PDF format and post to the secure FTP site.  Where workmanship at project site and elsewhere is governed by standard, furnish additional copies to fabricators, installers and others involved in performance of the work. Installation of the item will not be allowed to proceed until the information is received.  Failure to furnish the information can be cause for rejection of the material.

I.	Close-out Submittals:  Refer to individual work sections and to "close-out" section for specific requirements on submittal of close-out information, materials, tools and similar items.

J.	General Distribution:  Provide additional distribution of submittals (not included in foregoing copy submittal requirements) to subcontractors, suppliers, fabricators, installers, and others as necessary for proper performance of the work.

1.5	ACTION ON SUBMITTALS

A.	Contracting Officer's Action:  Where action and return is required or requested, the Contracting Officer, or the Contracting officer’s designated representative, will review each submittal, mark with Action, and where possible return within 2 weeks of receipt.  Where submittal must be held for coordination, Contractor will be so advised.

1.	Final Unrestricted Release:  Work may proceed, provided it complies with contract documents, when submittal is returned marked “No Exceptions Taken”.

2.	Final-But-Restricted Release:  Work may proceed, provided it complies with notations and corrections on submittal and with contract documents, when submittal is returned marked "Approved as Noted."

3.	Resubmittal Required, Restricted Release:  Work may proceed provided it complies with notations and corrections and any additional information requested on the submittal and with contract documents when submittal is returned marked “Approved as Noted, Resubmittal Required”.  Resubmittal is required to make the corrections and provide requested information noted.

4.	Disapproved, Resubmit:  Do not proceed with work.  Revise submittal in accordance with notations thereon, and resubmit without delay to obtain a different action marking.  Do not allow submittals marked "Disapproved, Resubmit" (or unmarked submittals where a marking is required) to be used in connection with performance of the work.

5.	Other Action:  Where submittal is returned for other reasons, with the Contracting Officer's explanation included, it will be marked "Action Not Required."

PART 2 - PRODUCTS (Not Applicable)

PART 3 - EXECUTION (Not Applicable)

END OF SECTION 013300