SECTION 017329 - CUTTING AND PATCHING

PART 1 - GENERAL

1.1	DESCRIPTION OF REQUIREMENTS

A.	Definition:  "Cutting and patching" includes cutting into existing construction to provide for the installation or performance of other work and subsequent fitting and patching required to restore surfaces to their original condition.

1.	Cutting and patching is performed for coordination of the work, to uncover work for access or inspection, to obtain samples for testing, to permit alterations or installation of products and materials, in existing conditions and for other similar purposes.

2.	Cutting and patching performed during the manufacture of products, or during the initial fabrication, is not considered to be "cutting and patching" under this definition.  Drilling of holes to install fasteners and similar operations are also not considered to be "cutting and patching".

3.	"Demolition" and "Selective Demolition" are recognized as related-but-separate categories of work, which may or may not require cutting and patching as defined in this section; refer to "Selective Demolition" sections of Division 1.

B.	Refer to other sections of these specifications for specific cutting and patching requirements and limitations applicable to individual units of work.

1.2	QUALITY ASSURANCE

A.	Requirements for Structural Work:  Do not cut and patch structural work in a manner that would result in a reduction of load-carrying capacity or of 1oad-deflection ratio.  Prior to such work, obtain approval of project's structural engineer.

B.	Before cutting and patching the following categories of work, obtain the Contracting Officer's approval to proceed.

1.	Structural steel and/or concrete.

2.	Miscellaneous structural metals, including lintels, equipment supports, stair systems and similar categories of work.

3.	Noise and vibration control elements and systems.

- Where removals leave holes and damaged surfaces exposed in the finished work, patch and repair these holes and damaged surfaces to match adjacent finished surfaces.  Where new work is to be applied to existing surfaces, perform removals and patching in a manner to produce surfaces suitable for receiving new work.  Finished surfaces of patched area shall be flush with  the adjacent existing surface and shall match the existing adjacent surface texture and finish as closely as possible.

1.3	SUBMITTALS

A.	Procedural Proposal for Cutting and Patching:  Where prior approval of cutting and patching is required, submit proposed procedures for this work well in advance of the time work will be performed and request approval to proceed. Include the following information, as applicable, in the submittal:

1.	Describe nature of the work and how it is to be performed, indicating why cutting and patching cannot be avoided. Describe anticipated results of the work in terms of changes to work, including structural, operational and visual changes as well as other significant elements.

2.	List products to be used when work is expected to be performed.

3.	Give dates when work is expected to be performed.

4.	List utilities that will be disturbed or otherwise be affected by work, including those that will be relocated and those that will be out of service temporarily. Include how long utility service will be disrupted.

5.	When cutting, modifying, and patching of structural work is required to facilitate the installation of work required in this contract, notify the Contracting Officer immediately. Do not cut or modify the structural element without written instructions from the Contracting Officer. The Contractor shall follow the Contracting Officer’s direction.

PART 2 - PRODUCTS

2.1	MATERIALS

A.	General:  Except as otherwise indicated, or as directed by the Contracting Officer, use materials for cutting and patching that are identical to adjacent materials.  If identical materials are not available, or cannot be used, use materials that match adjacent surfaces to the fullest extent possible with regard to visual effect. Use materials for cutting and patching that will result in equal-or-better performance characteristics.

1.	The use of a trade name and suppliers name and address is to indicate a possible source of the product.  Products of the same type from other sources shall not be excluded provided they possess like physical and functional characteristics.

PART 3 - EXECUTION

3.1	INSPECTION

A.	Before cutting, examine the surfaces to be cut and patched and the conditions under which the work is to be performed.  If unsafe or otherwise unsatisfactory conditions are encountered, notify the Contracting Officer.  Corrective action shall be taken before proceeding with the work.

3.2	PREPARATION

A.	Temporary Support:  To prevent failure provide temporary support of work to be cut as required.

B.	Protection:  Protect other work during cutting and patching to prevent damage.  Provide protection from adverse weather conditions of that part of the project that may be exposed during cutting and patching operations.

1.	Avoid interference with use of adjoining areas or interruption of free passage to adjoining areas.

3.3	PERFORMANCE

A.	General:  Employ skilled workmen to perform cutting and patching work. Except as otherwise indicated or as approved by the Contracting Officer, proceed with cutting and patching at the earliest feasible time and complete work without delay.

B.	Cutting:  Cut the work using methods that are least likely to damage work to be retained or adjoining work.  Where possible review proposed procedures with the original installer; comply with original installer's recommendations.

1.	In general, where cutting is required use hand or small power tools designed for sawing or grinding, not hammering and chopping. Cut through concrete and masonry using a cutting machine such as a carborundum saw or core drill to insure a neat hole.

2.	Cut holes and slots neatly to size required with minimum disturbance of adjacent work.  To avoid marring existing finished surfaces, cut or drill from the exposed or finished side into concealed surfaces.  Temporarily cover openings when not in use.

C.	Patching: Patch with seams which are durable and as invisible as possible.  Comply with specified tolerances for the work.

1.	Where feasible, inspect and test patched areas to demonstrate integrity of work.

2.	Restore exposed finishes of patched areas and where necessary extend finish restoration into retained adjoining work in a manner which will eliminate evidence of patching and refinishing.

3.	Where removal of walls or partitions extends from one finished area into another finished area, patch and repair floor and wall surfaces in the new space to provide an even surface of uniform color and appearance.  If necessary to achieve uniform color and appearance, remove floor and wall coverings and replace with new materials.

a.	Where patch occurs in a smooth painted surface, extend final paint coat over entire unbroken surface containing patch, after patched area has received prime and base coat.

4.	Patch or repair existing ceilings as necessary to provide an even plane surface of uniform appearance.

3.4	CLEANING

A.	Thoroughly clean areas and spaces where work is performed or used as access to work.  Completely remove paint, mortar, oils, putty and items of similar nature which are not part of the final patch or refinished surface.  Thoroughly clean piping, conduit and similar features before painting or other finishing is applied in accordance with other trade sections.

END OF SECTION 017329