# SECTION 017419 - CONSTRUCTION WASTE MANAGEMENT AND DISPOSAL

Note:  This is not an official LBNL specification section.  It is intended to serve as a guideline in the development of LBNL sections on this topic.

PART 1 - GENERAL

1.1	WASTE MANAGEMENT GOALS

[New construction generates  approximately 4 lbs of waste per square foot of construction.

New construction waste is easier to recycle than demolition waste due to the relative ease of separating the materials. Construction occurs in phases which makes it easier to keep scraps separate.  Demolition can generate over 70lbs. of waste per square ft. of demolition]

[Typical amounts of waste as a % of total waste for commercial construction

- Wood						20-30%*

- Concrete and Block				10-20%

- Drywall						5-10%

- Cardboard					5-10%

- Steel from decking, rebar			1-8%

- Brick						1-5%

- Crates and Pallets				1-5%

- Expanded Poly Styrene				3% range

* For wood frame or partially wood framed building.]

A.	GENERAL WASTE MANAGEMENT GOALS:

1. 	This Project shall generate the least amount of waste possible.

2.	The Contractor shall employ processes which ensure the generation of as little waste as possible and shall avoid the generation of waste due to the following:

a.	Over-packaging

b.	Error

c.	Poor planning, layout

d.	Over ordering

e.	Breakage

f.	Mishandling

g.	Contamination

h.	Damage from weather

3.	Of the inevitable waste that is generated, as many of the waste materials as economically feasible shall be reused, salvaged, or recycled.

4.	Waste disposal in landfills shall be minimized to the greatest extent possible.

B. 	WASTE Diversion Goals:

1.	The following waste categories, at a minimum, shall be diverted from landfill:

- Inerts (concrete, asphalt, masonry)

- Clean dimensional  wood, palette wood

- Engineered wood products: Plywood, Particle board, I Joists, etc.

- Cardboard, paper, packaging

- Metals from banding, ductwork, framing, piping and repair. Including steel, iron, galvanized sheet steel, stainless steel, aluminum, copper, zinc, lead, brass and bronze.

- Paint

- Plastics: ABS, PVC

- Beverage containers

- Insulation

- Gypsum board

  - RELATED SECTIONS

- All technical sections.

1.3	DEFINITIONS

A.	CLASS III LANDFILL:  A landfill that accepts non-hazardous resources such as household, commercial, and industrial waste resulting form construction, remodeling, repair, and demolition operations.

B.	CLEAN:  Untreated and unpainted; uncontaminated with adhesives, oils, solvents, mastics, and like products.

C.	CONSTRUCTION AND DEMOLITION WASTE:  Includes all non-hazardous resources resulting from construction, remodeling, alterations, repair and demolition operations.

D.	DISMANTLE:  The process of parting out a building in such a way as to preserve the usefulness of its materials and components.

E.	DISPOSAL:  acceptance of solid wastes at a legally operating facility for the purpose of landfilling.  Includes Class III landfills and inert fills.

F.	INERT FILL:  A facility that can legally accept inert waste such as asphalt and concrete exclusively for the purpose of disposal.

G.	INERT SOLIDS/INERT WASTE:  Non-liquid solid resources including, but not limited to, soil and concrete, that does not contain hazardous waste or soluble pollutants at concentrations in excess of water-quality objectives established by a regional Water Board and does not contain significant quantities of decomposable solid resources.

H.	MIXED DEBRIS:  Loads that include commingled recyclable and non-recyclable materials generated at the construction site.

I.	MIXED DEBRIS RECYCLING FACILITY:  A solid resources processing facility that accepts loads of mixed construction and demolition debris for the purpose of recovering re-usable and recyclable materials and disposing non-recyclable materials.

J.	PERMITTED WASTE HAULER:  A company that holds a valid permit from the State of Connecticut to collect and transport solid wastes from individuals or businesses for the purpose of recycling or disposal.

K.	RECYCLING:  The process of sorting, cleansing, treating, and reconstituting materials for the purpose of using the altered form in the manufacture of a new product.  Recycling does not include burning, incinerating or thermally destroying solid waste.

1.	On-site Recycling:  Materials that are sorted and processed on site for use in an altered stated in the Work, i.e. concrete crushed for use as a sub-base in paving.

2.  	Off-site Recycling:  Materials hauled to a location and used in an altered form in the manufacture of a new product.

L.	RECYCLING FACILITY:  An operation that can legally accept materials for the purpose of processing the materials into an altered form for the manufacture of a new product.  Depending on the types of materials accepted and operating procedures, a recycling facility may or may not be required to have a Solid Waste Facilities permit from the City of Harford.

M.	RE-USE:  Materials that are recovered for use in the same form, on- or off-site.

N.	RETURN:  To give back reusable items or unused products to vendors for credit.

O.	SALVAGE:  To remove waste materials from the site for resale or re-use by a third party.

P.	SOURCE-SEPARATED MATERIALS:  Materials that are sorted by type at the Site for the purpose of reuse and recycling.

Q.	SOLID WASTE:  Materials that have been designated as non-recyclable and are discarded for the purposes of disposal.

R.	TRANSFER STATION:  A facility that can legally accept solid wasted for the purpose of temporarily storing the materials for re-loading onto other trucks and transporting them to a landfill for disposal, or recovering some materials for re-use or recycling.

1.4	SUBMITTALS

A.	WASTE MANAGEMENT PLAN:

Prior to any waste removal, the Contractor shall submit to the Contracting Officer a Waste Management Plan.  The Plan shall contain the following:

1.	Analysis of the estimated job site waste to be generated, including types and quantities. Provide written description method for removal of waste from renovation area to outside of Jetport.

2.	Proposed Alternatives to Landfilling:  Contractor shall prepare a list of each material proposed to be salvaged, re-used, or recycled during the course of the Project.

3.	Methods handling of materials to be recycled

a.	On site:

i.	Materials separation

ii.	Materials storage

ii.	Materials protection, where applicable.

b.	Off site: Provide name of mixed debris recycling facility; include list of materials to be recycled.

4.	Materials Handling Procedures:  A description of the means to be employed in recycling the above materials consistent with requirements for acceptance by designated facilities.

5.	Landfill options: The name of the landfill(s) where trash will be disposed of.

6.	Meetings:  Contractor shall conduct Construction Waste Management meetings. Meetings shall include the Subcontractor, the Contracting Officer and the Building Manager. At a minimum, waste management goals and issues shall be discussed at the following meetings:

a.	Pre-construction meeting.

b.	Regular job-site meetings.

7.	Transportation:  A description of the means of transportation of the recyclable materials (whether materials will be site-separated and self-hauled to designated centers, or whether mixed materials will be collected by a waste hauler and removed from the site) and destination of materials

B. 	REPORTS

1.	The Contractor shall submit regularly, a Waste Management Progress Report. The report shall contain the following information:

a.	The amount (in tons or cubic yards) of material landfilled from the Project, the identity of the landfill, the total amount of tipping fees paid at the landfill, and the total disposal cost. Include legible copies of manifests, weight tickets, receipts, and invoices.  Manifests shall be from recycles and/or disposal site operators that can legally accept the materials for the purpose of reuse, recycling or disposal.

b.	For each material recycled, reused, or salvaged from the Project, provide the following:

i. 	Amount (in tons or cubic yards)

ii. 	Date removed from the job site

iii.	Receiving party

1.5	QUALITY ASSURANCE

A.	Comply with EPA Executive Order 12873, 1993.

PART 2 - PRODUCTS:  Not Used

PART 3 - EXECUTION:  Not Used

END OF SECTION 07419