Copyright 2002 and 2005 by The American Institute of Architects (AIA)

Exclusively published and distributed by Architectural Computer Services, Inc. (ARCOM) for the AIA

SECTION 024119 - SELECTIVE STRUCTURE DEMOLITION

This Section uses the term "Architect."  Change this term to match that used to identify the design professional as defined in the General and Supplementary Conditions.

Verify that Section titles referenced in this Section are correct for this Project's Specifications; Section titles may have changed.

- GENERAL

      - RELATED DOCUMENTS

        - Drawings and general provisions of the Contract, including General and Supplementary Conditions and other Division 01 Specification Sections, apply to this Section.

      - SUMMARY

        - This Section includes the following:

Adjust list below to suit Project.

          - Demolition and removal of selected portions of building or structure.

        - Related Sections include the following:

List below only procedures that the reader might expect to find in this Section but are specified elsewhere.

          - Division 01 Section "Summary" for use of premises and Owner-occupancy requirements.

          - Division 01 Section “Project Management and Coordination” for use of premises and daily Owner-occupancy requirements.

          - Division 01 Section "Temporary Facilities and Controls" for temporary construction and environmental-protection measures for selective demolition operations.

          - Division 01 Section "Cutting and Patching" for cutting and patching procedures.

          - Division 01 Section "Construction Waste Management and Disposal" for disposal of demolished materials.

      - DEFINITIONS

Retain definitions that remain after this Section has been edited.

        - Remove:  Detach items from existing construction and legally dispose of them off-site, unless indicated to be removed and salvaged or removed and reinstalled.

Salvage in first paragraph below may add cost to Project; verify with Owner.

        - Remove and Reinstall:  Detach items from existing construction, prepare them for reuse, and reinstall them where indicated.

        - Existing to Remain:  Existing items of construction that are not to be removed and that are not otherwise indicated to be removed, removed and salvaged, or removed and reinstalled.

      - QUALITY ASSURANCE

        - Demolition Firm Qualifications:  An experienced firm that has specialized in demolition work similar in material and extent to that indicated for this Project.

        - Regulatory Requirements:  Comply with governing EPA notification regulations before beginning selective demolition.  Comply with hauling and disposal regulations of authorities having jurisdiction.

        - Standards:  Comply with ANSI A10.6 and NFPA 241.

Delete paragraph below if Work of this Section is not extensive or complex enough to justify a predemolition conference.  If retaining, coordinate with other Division 01 Sections.

        - Predemolition Conference:  Conduct conference at Project site to comply with requirements in Division 01 Section "Project Management and Coordination."

Retain paragraph above if requirements in other Division 01 Sections are adequate for Project.  Retain paragraph below if additional requirements are necessary; include information about conference.  Identify specific participants not mentioned in other Division 01 Sections.

Insert agenda items below not listed in Division 01 Section "Project Management and Coordination."

      - PROJECT CONDITIONS

Retain, revise, or delete this Article to suit Project.  Add other limitations if necessary, such as when adjacent floors will be occupied.

        - Owner will occupy building during selective demolition.  Conduct selective demolition so Owner's operations will not be disrupted.

          - Comply with requirements specified in Division 01 Section "Summary."

Review first paragraph and subparagraph below and revise if necessary.  In subparagraph, include list of items that will be removed by Owner.

Retain one of first three paragraphs and associated subparagraphs below, or remove all references to asbestos.  Expand scope of this Article to include PCBs or other materials if required.  Coordinate statements with the General and Supplementary Conditions.

Retain first paragraph and subparagraphs below and delete two paragraphs and associated subparagraphs above if asbestos abatement is part of Work of this Contract.  Consult a professional liability insurance carrier for current recommendations.  Indemnification and a waiver of claims may be required from Owner as a condition for providing services related to asbestos abatement.

        - Hazardous Materials:  Hazardous materials are present in construction to be selectively demolished.

          - Hazardous material remediation is specified elsewhere in the Contract Documents.

          - Do not disturb hazardous materials or items suspected of containing hazardous materials except under procedures specified elsewhere in the Contract Documents.

        - Storage or sale of removed items or materials on-site is not permitted.

        - Utility Service:  Maintain existing utilities indicated to remain in service and protect them against damage during selective demolition operations.

          - Maintain fire-protection facilities in service during selective demolition operations.

      - WARRANTY

Delete this Article if no existing warranties would be affected.

        - Existing Warranties:  Remove, replace, patch, and repair materials and surfaces cut or damaged during selective demolition, by methods and with materials so as not to void existing warranties.

- PRODUCTS (Not Used)

- EXECUTION

      - EXAMINATION

        - Verify that utilities have been disconnected and capped.

        - Survey existing conditions and correlate with requirements indicated to determine extent of selective demolition required.

        - Inventory and record the condition of items to be removed and reinstalled.

        - When unanticipated mechanical, electrical, or structural elements that conflict with intended function or design are encountered, investigate and measure the nature and extent of conflict.  Promptly submit a written report to Contracting Officer.

Usually retain first paragraph below.  OSHA regulations require an engineering survey before selective demolition begins.

        - Survey of Existing Conditions:  Record existing conditions by use of preconstruction photographs.

Requirement in subparagraph below is for treatment of historic facilities.

          - Before selective demolition or removal of existing building elements that will be reproduced or duplicated in final Work, make permanent record of measurements, materials, and construction details required to make exact reproduction.

        - Perform surveys as the Work progresses to detect hazards resulting from selective demolition activities.

      - UTILITY SERVICES AND MECHANICAL/ELECTRICAL SYSTEMS

        - Existing Services/Systems:  Maintain services/systems indicated to remain and protect them against damage during selective demolition operations.

          - Comply with requirements for existing services/systems interruptions specified in Division 01 Section "Summary."

Delete paragraph and subparagraphs below if not applicable or if Owner arranges and performs work.

        - Service/System Requirements:  Locate, identify, disconnect, and seal or cap off indicated utility services and mechanical/electrical systems serving areas to be selectively demolished.

          - Government will arrange to shut off indicated services/systems when requested by Contractor.

Delete subparagraph above or first subparagraph below to suit Project.

      - PREPARATION

        - Site Access and Temporary Controls:  Conduct selective demolition and debris-removal operations to ensure minimum interference with roads, streets, walks, walkways, and other adjacent occupied and used facilities.

          - Comply with requirements for access and protection specified in Division 01 Section "Temporary Facilities and Controls."

Revise paragraph and subparagraph above and first paragraph and subparagraphs below to suit Project.  Delete all if adequately covered in Division 01 Section "Temporary Facilities and Controls."

        - Temporary Facilities:  Provide temporary barricades and other protection required to prevent injury to people and damage to adjacent buildings and facilities to remain.

          - Provide protection to ensure safe passage of people around selective demolition area and to and from occupied portions of building.

          - Protect walls, ceilings, floors, and other existing finish work that are to remain or that are exposed during selective demolition operations.

          - Cover and protect furniture, furnishings, and equipment that have not been removed.

          - Comply with requirements for temporary enclosures, dust control, heating, and cooling specified in specification sections “Asbestos Removal” and “Lead Paint Removal”.

      - SELECTIVE DEMOLITION, GENERAL

        - General:  Demolish and remove existing construction only to the extent as indicated.  Use methods required to complete the Work within limitations of governing regulations and as follows:

          - Neatly cut openings and holes plumb, square, and true to dimensions required.  Use cutting methods least likely to damage construction to remain or adjoining construction.  Use hand tools or small power tools designed for sawing or grinding, not hammering and chopping, to minimize disturbance of adjacent surfaces.  Temporarily cover openings to remain.

          - Cut or drill from the exposed or finished side into concealed surfaces to avoid marring existing finished surfaces.

          - Do not use cutting torches until work area is cleared of flammable materials.  At concealed spaces, such as duct and pipe interiors, verify condition and contents of hidden space before starting flame-cutting operations.  Maintain fire watch and portable fire-suppression devices during flame-cutting operations.

          - Maintain adequate ventilation when using cutting torches.

          - Remove decayed, vermin-infested, or otherwise dangerous or unsuitable materials and promptly dispose of off-site.

          - Dispose of demolished items and materials promptly.

        - Removed and Reinstalled Items:

          - Clean and repair items to functional condition adequate for intended reuse.  Finish equipment to match new equipment.

          - Pack or crate items after cleaning and repairing.  Identify contents of containers.

          - Protect items from damage during transport and storage.

          - Reinstall items in locations indicated.  Comply with installation requirements for new materials and equipment.  Provide connections, supports, and miscellaneous materials necessary to make item functional for use indicated.

In paragraph below, option for cleaning existing items to remain before reinstalling adds cost.  Revise to suit Project.

        - Existing Items to Remain:  Protect construction indicated to remain against damage and soiling during selective demolition.  When permitted, items may be removed to a suitable, protected storage location during selective demolition, cleaned, and reinstalled in their original locations after selective demolition operations are complete.

        - PCB Light Fixtures - All light ballasts that do not have a clear “NO PCB’s” label on them are to be considered PCB containing and if are to be removed, disposed of as PCB containing material. All bulbs are to be packaged and disposed as a mercury (Hg) containing waste, if they are to be removed.

          - Review with Contracting Officer the quantity and location of all suspect light fixtures and proceed based on direction received.

If needed, insert requirements for other materials, products, equipment, and services.

      - DISPOSAL OF DEMOLISHED MATERIALS

Add other specific disposal, cleanup, or removal requirements to suit Project.

        - General:  Dispose of all materials containing asbestos and lead paint as directed in the specification sections “Asbestos Removal” and “Lead Paint Removal”.  Items or materials indicated to be reinstalled or otherwise indicated are to remain Owner's property. Remove demolished materials daily from Project site and legally dispose of them.

          - Do not allow demolished materials to accumulate on-site.

          - Remove and transport debris in a manner that will prevent spillage on adjacent surfaces and areas.

Coordinate first subparagraph below with use of elevators, stairs, or building entries permitted by building manager.

          - Remove debris from elevated portions of building by chute, hoist, or other device that will convey debris to grade level in a controlled descent.

          - Comply with requirements specified in Division 01 Section "Construction Waste Management and Disposal."

        - Burning:  Do not burn demolished materials.

Retain paragraph above or below; burning is usually not permitted.

Retain paragraph below if disposal is permitted on Owner's property.  Revise below, if applicable, to indicate limits on type of materials that may be disposed of on-site.

Delete paragraph above and retain paragraph below when disposal off Owner's property is required.  Add specific requirements for off-site disposal to suit Project.

        - Disposal:  Transport demolished materials off Owner's property and legally dispose of them.

      - CLEANING

        - Clean adjacent structures and improvements of dust, dirt, and debris caused by selective demolition operations.  Return adjacent areas to condition existing before selective demolition operations began.

END OF SECTION 024119