Copyright 2003 and 2005 by The American Institute of Architects (AIA)

Exclusively published and distributed by Architectural Computer Services, Inc. (ARCOM) for the AIA

SECTION 064023 - INTERIOR ARCHITECTURAL WOODWORK

This Section uses the term "Architect."  Change this term to match that used to identify the design professional as defined in the General and Supplementary Conditions.

Verify that Section titles referenced in this Section are correct for this Project's Specifications; Section titles may have changed.

- GENERAL

      - SUMMARY

        - This Section includes the following:

Adjust list below to suit Project.

          - Plastic-laminate cabinets.

          - Plastic-laminate countertops.

        - Interior architectural woodwork includes wood furring, blocking, shims, and hanging strips unless concealed within other construction before woodwork installation.

      - SUBMITTALS

        - Product Data:  For cabinet hardware and accessories and finishing materials and processes.

        - Shop Drawings:  Show location of each item, dimensioned plans and elevations, large-scale details, attachment devices, and other components.

First paragraph and subparagraphs below assume manufacturer's standard-size Samples are acceptable.  Revise to suit Project.

        - Samples:

          - Plastic-laminates, for each type, color, pattern, and surface finish.

        - Sustainable Submittals:

Retain subparagraph and associated subparagraphs below if low-emitting materials are required for LEED-NC or LEED-CI Credit EQ 4.4.

          - Product Data:

            - For each composite-wood product used, documentation indicating that the bonding agent contains no urea formaldehyde.

            - For each adhesive used, documentation indicating that the adhesive contains no urea formaldehyde.

Retain subparagraph and associated subparagraph below and "Forest Certification" Paragraph in "Quality Assurance" Article if wood products are required to be certified for LEED-NC or LEED-CI Credit MR 7.

          - Certificates:  Chain-of-custody certificates certifying that products specified to be made from certified wood comply with forest certification requirements.  Include evidence that mill is certified for chain of custody by an FSC-accredited certification body.

            - Include statement indicating costs for each certified wood product.

If retaining paragraph below, also retain paragraph in "Quality Assurance" Article that requires compliance with selected quality standard and certification program.  See Evaluations.

        - Woodwork Quality Standard Compliance Certificates:  AWI Quality Certification Program certificates.

      - QUALITY ASSURANCE

Delete first paragraph below if Installer is not required to be woodwork fabricator.

        - Installer Qualifications:  Fabricator of woodwork.

        - Quality Standard:  Unless otherwise indicated, comply with AWI's "Architectural Woodwork Quality Standards."

Consider retaining applicable subparagraph below.  Consider deleting certification for installation if installation may not be performed by woodworking firm.  See Evaluations.

          - Provide AWI Quality Certification Program labels and certificates for woodwork, including installation.

Paragraph below may be retained to specify interior architectural woodwork made from certified wood for LEED-NC or LEED-CI Credit MR 7, which requires that a minimum of 50 percent of wood-based materials be certified.

        - Forest Certification:  Provide interior architectural woodwork produced from wood obtained from forests certified by an FSC-accredited certification body to comply with FSC STD-01-001, "FSC Principles and Criteria for Forest Stewardship."

      - PROJECT CONDITIONS

        - Environmental Limitations:  Do not deliver or install woodwork until building is enclosed, wet work is complete, and HVAC system is operating and maintaining temperature and relative humidity at occupancy levels during the remainder of the construction period.

- PRODUCTS

      - WOODWORK FABRICATORS

Delete this Article if fabricators are not preapproved.

        - Fabricators:  Subject to compliance with requirements, provide interior architectural woodwork by one of the following:

Insert list of preapproved firms.  Refer to AWI's or WI's member list for names of woodworking firms that could potentially be included.

      - MATERIALS

        - Wood Products:

Retain first subparagraph below if recycled content is required for LEED-NC or LEED-CI Credits MR 4.1 and MR 4.2.

          - Hardboard:  AHA A135.4.

If retaining subparagraph below and low-emitting materials are required for LEED-NC or LEED-CI Credit EQ 4.4, retain option prohibiting urea formaldehyde.  See Evaluations.  SierraPine's "Medex," "Medex NC," and "Medite II" and Weyerhaeuser's "Premier Plus" fiberboard are made without urea formaldehyde.

If retaining subparagraph below and low-emitting materials are required for LEED-NC or LEED-CI Credit EQ 4.4, retain option prohibiting urea formaldehyde.  See Evaluations.

          - Veneer-Faced Panel Products (Hardwood Plywood):  HPVA HP-1, made with adhesive containing no urea formaldehyde.

First paragraph below describes "Permalam," frequently called "melamine." If retaining, also retain "Medium-Density Fiberboard" Subparagraph and applicable "Particleboard" subparagraphs in "Wood Products" Paragraph above.

        - High-Pressure Decorative Laminate:  NEMA LD 3, grades as indicated or, if not indicated, as required by woodwork quality standard.

      - CABINET HARDWARE AND ACCESSORIES

        - General:  Provide cabinet hardware and accessory materials associated with architectural woodwork, except for items specified in Division 08 Section "Door Hardware (Scheduled by Describing Products)."

        - Butt Hinges:  2-3/4-inch (70-mm), 5-knuckle steel hinges made from 0.095-inch- (2.4-mm-) thick metal, and as follows:

          - Semiconcealed Hinges for Flush Doors:  BHMA A156.9, B01361.

          - Semiconcealed Hinges for Overlay Doors:  BHMA A156.9, B01521.

        - Back-Mounted Pulls:  BHMA A156.9, B02011.

        - Catches:  Ball friction catches, BHMA A156.9, B03013.

        - Drawer Slides:  BHMA A156.9, B05091.

          - Heavy Duty (Grade 1HD-100 and Grade 1HD-200):  Side mounted; full-extension type; zinc-plated steel ball-bearing slides.

Grades in three subparagraphs below correspond to the following initial load test requirements:  Grade 2:  30 lbf (13.3 kg); Grade 1:  50 lbf (22.2 kg); Grade 1HD-100:  100 lbf (44.5 kg); Grade 1HD-200:  200 lbf (90 kg).

        - Exposed Hardware Finishes:  For exposed hardware, provide finish that complies with BHMA A156.18 for BHMA finish number indicated.

          - Satin Stainless Steel:  BHMA 630.

      - MISCELLANEOUS MATERIALS

Delete first paragraph below if specified in a carpentry section.

        - Furring, Blocking, Shims, and Hanging Strips:  Softwood or hardwood lumber, kiln-dried to less than 15 percent moisture content.

Retain paragraph below if low-emitting materials are required for LEED-NC or LEED-CI Credit EQ 4.4.  See Evaluations.

        - Adhesives, General:  Do not use adhesives that contain urea formaldehyde.

      - FABRICATION

        - General:  Complete fabrication to maximum extent possible before shipment to Project site.  Where necessary for fitting at site, provide allowance for scribing, trimming, and fitting.

          - Interior Woodwork Grade - AWI:  Premium.

          - Shop cut openings to maximum extent possible.  Sand edges of cutouts to remove splinters and burrs.  Seal edges of openings in countertops with a coat of varnish.

          - Panel-Matching Method:  No matching between panels is required.  Select and arrange panels for similarity of grain pattern and color between adjacent panels.

Retain subparagraph above for Economy grade; retain subparagraph below for Custom or Premium grade.  Options below are listed in order of increasing cost and last option only applies to Premium grade.

        - Plastic-Laminate Cabinets:

          - AWI Type of Cabinet Construction:  Flush overlay.

Retain subparagraph above if AWI standard is referenced.  If WI standard is referenced, delete above and select one WI requirement from each of three subparagraphs below.

Delete first subparagraph below if reveal overlay construction is not used.

          - Reveal Dimension:  1/2 inch (13 mm).

          - Laminate Cladding for Exposed Surfaces:  High-pressure decorative laminate as follows:

Retain grade designations from options in four subparagraphs below.  Grade HGS is 1.2 mm thick, Grades HGL and HGP are 1.0 mm thick, and Grade VGS is 0.7 mm thick.  AWI and WI standards require minimum thickness of 0.028 inch (0.7 mm) regardless of surface type.

            - Horizontal Surfaces Other Than Tops:  Grade HGS.

            - Postformed Surfaces:  Grade HGP.

            - Vertical Surfaces:  Grade HGS.

            - Edges:  PVC tape, 0.018-inch (0.460-mm) minimum thickness, matching laminate in color, pattern, and finish.

Delete first three subparagraphs below for woodworker's option from choices in standard.  See Editing Instruction No. 2 in the Evaluations.

          - Materials for Semiexposed Surfaces Other Than Drawer Bodies:  High-pressure decorative laminate, Grade VGS.

          - Drawer Sides and Backs:  Solid-hardwood lumber.

          - Drawer Bottoms:  Hardwood plywood.

Retain and edit subparagraph above or below.

          - Colors, Patterns, and Finishes:  As selected by Contracting Officer from laminate manufacturer's full range.

Delete subparagraph below if not desired.  Dust panels are not required by standards.

          - Provide dust panels of 1/4-inch (6.4-mm) plywood or tempered hardboard above compartments and drawers, unless located directly under tops.

        - Plastic-Laminate Countertops:

Retain first option in first subparagraph below for flat countertops; retain second for postformed countertops.

          - High-Pressure Decorative Laminate Grade:  HGS.

Retain and edit subparagraph above or first subparagraph below.

          - Colors, Patterns, and Finishes:  As selected by Contracting Officer from laminate manufacturer's full range.

          - Edge Treatment:  Same as laminate cladding on horizontal surfaces.

          - Core Material at Sinks:  exterior-grade plywood.

- EXECUTION

      - INSTALLATION

        - Before installation, condition woodwork to average prevailing humidity conditions in installation areas.  Examine shop-fabricated work for completion and complete work as required, including removal of packing and backpriming.

Revise first paragraph below if installation grade is different from fabrication grade.

        - Grade:  Install woodwork to comply with requirements for the same grade specified in Part 2 for fabrication of type of woodwork involved.

        - Install woodwork level, plumb, true, and straight to a tolerance of 1/8 inch in 96 inches (3 mm in 2400 mm).  Shim as required with concealed shims.

        - Scribe and cut woodwork to fit adjoining work, refinish cut surfaces, and repair damaged finish at cuts.

        - Anchor woodwork to anchors or blocking built in or directly attached to substrates.  Secure with countersunk, concealed fasteners and blind nailing as required for complete installation.  Use fine finishing nails for exposed fastening, countersunk and filled flush with woodwork and matching final finish if transparent finish is indicated.

Delete any of five paragraphs and associated subparagraphs below if not applicable.

        - Cabinets:  Install without distortion so doors and drawers fit openings properly and are accurately aligned.  Adjust hardware to center doors and drawers in openings and to provide unencumbered operation.

Revise requirements in subparagraph below as necessary for seismic restraint of cabinets.  Delete if hanging cleats are used and are detailed on Drawings.

          - Fasten wall cabinets through back, near top and bottom, at ends and not more than 16 inches (400 mm) o.c. with No. 10 wafer-head sheet metal screws through metal backing or metal framing behind wall finish.

        - Countertops:  Anchor securely by screwing through corner blocks of base cabinets or other supports into underside of countertop.  Calk space between backsplash and wall with sealant specified in Division 07 Section "Joint Sealants."

END OF SECTION 064023