Copyright 2003, The American Institute of Architects (AIA)

Exclusively published and distributed by Architectural Computer Services, Inc. (ARCOM) for the AIA

SECTION 087100 - DOOR HARDWARE

This Section uses the term "Architect."  Change this term to match that used to identify the design professional as defined in the General and Supplementary Conditions.

Verify that Section titles referenced in this Section are correct for this Project's Specifications; Section titles may have changed.

See Editing Instruction No. 2 in the Evaluations before editing this Section.

- GENERAL

      - RELATED DOCUMENTS

        - Drawings and general provisions of the Contract, including General and Supplementary Conditions and Division 1 Specification Sections, apply to this Section.

      - SUMMARY

        - This Section includes the following:

Adjust list below to suit Project.

          - Commercial door hardware for the following:

            - Swinging doors.

            - Other doors to the extent indicated.

          - Cylinders for doors specified in other Sections.

        - Related Sections include the following:

List below only products and construction that the reader might expect to find in this Section but are specified elsewhere.

          - Division 8 Section "Hollow Metal Doors and Frames."

          - Division 8 Section "Flush Wood Doors."

      - SUBMITTALS

        - Product Data:  Include construction and installation details, material descriptions, dimensions of individual components and profiles, and finishes.

Retain paragraph and subparagraphs below for electrified door hardware.

Delete subparagraph and associated subparagraphs below if electrified door hardware is not complex (e.g., only electromagnetic locks or electric strikes are required).

Select options in first subparagraph below to suit Project or delete if not required.

Retain one of first two paragraphs below.  First is usually for plastic-laminate or acrylic armor, kick, mop, and stretcher plates.

Delete paragraph above if colors and other characteristics are preselected and specified or scheduled.  Retain one of two paragraphs and subparagraph below with or without above.

        - Samples for Verification:  Submit minimum 2-by-4-inch (51-by-102-mm) plate Samples of each type of finish required, except primed finish.

Retain paragraph and subparagraph below for electrified door hardware.

Coordinate paragraph below with qualification requirements in Division 1 Section "Quality Requirements" and as supplemented in "Quality Assurance" Article.

Retain first paragraph below if retaining "Accessibility Requirements" paragraphs in Part 2.

        - Maintenance Data:  For each type of door hardware to include in maintenance manuals.  Include final hardware and keying schedule.

        - Warranty:  Special warranty specified in this Section.

        - Other Action Submittals:

          - Door Hardware Sets:  Prepared by or under the supervision of Installer, detailing fabrication and assembly of door hardware, as well as procedures and diagrams.  Coordinate the final door hardware sets with doors, frames, and related work to ensure proper size, thickness, hand, function, and finish of door hardware.

Retain one of first two subparagraphs below.

            - Format:  Use same scheduling sequence and format and use same door numbers as in the Contract Documents.

            - Content:  Include the following information:

              - Identification number, location, hand, fire rating, and material of each door and frame.

              - Type, style, function, size, quantity, and finish of each door hardware item.  Include description and function of each lockset and exit device.

              - Complete designations of every item required for each door or opening including name and manufacturer.

              - Fastenings and other pertinent information.

              - Location of each door hardware set, cross-referenced to Drawings, both on floor plans and in door and frame schedule.

              - Explanation of abbreviations, symbols, and codes contained in schedule.

              - Mounting locations for door hardware.

              - Door and frame sizes and materials.

Retain subparagraph below for electrified door hardware.

Edit first subparagraph below for type of hardware and function.

              - List of related door devices specified in other Sections for each door and frame.

            - Submittal Sequence:  Submit the final door hardware sets at earliest possible date, particularly where approval of the door hardware sets must precede fabrication of other work that is critical in Project construction schedule.  Include Product Data, Samples, Shop Drawings of other work affected by door hardware, and other information essential to the coordinated review of the door hardware sets.

Delete subparagraph above or below, or both if early approval is not critical.  Sequence below fragments and extends review process.

Retain "Installer Qualifications" Paragraph in "Quality Assurance" Article with subparagraph below.

          - Keying Schedule:  Prepared by or under the supervision of Installer, detailing Owner's final keying instructions for locks.  Include schematic keying diagram and index each key set to unique door designations.

      - QUALITY ASSURANCE

        - Installer Qualifications:  An employer of workers trained and approved by lock manufacturer.

          - Installer's responsibilities include supplying and installing door hardware and providing a qualified Architectural Hardware Consultant available during the course of the Work to consult with Contractor, Architect, and Owner about door hardware and keying.

          - Scheduling Responsibility:  Preparation of door hardware and keying schedules.

Delete subparagraph below if not required.

        - Architectural Hardware Consultant Qualifications:  A person who is currently certified by DHI as an Architectural Hardware Consultant and who is experienced in providing consulting services for door hardware installations that are comparable in material, design, and extent to that indicated for this Project.

Delete subparagraph below if not required.

        - Source Limitations:  Obtain each type and variety of door hardware from a single manufacturer, unless otherwise indicated.

        - Fire-Rated Door Assemblies: Assemblies complying with NFPA 80 that are listed and labeled by a testing and inspection agency acceptable to authorities having jurisdiction, for fire ratings indicated, based on testing according to NFPA 252.

Delete subparagraph below if not required.

Delete first paragraph and subparagraph below if not applicable.

Delete subparagraph below if retaining UBC standard in paragraph above; UBC standard requires positive-pressure testing.  Second option below is based on positive-pressure testing requirement in the International Building Code.

Delete paragraph and subparagraphs below if keying is not extensive or complex enough to justify a keying conference.  Identify additional participants not mentioned.

        - Keying Conference:  Conduct conference at Project site to comply with requirements in Division 1 Section "Project Management and Coordination."  In addition to Owner, Contractor, and Architect, conference participants shall also include Installer's Architectural Hardware Consultant.  Incorporate keying conference decisions into final keying schedule after reviewing door hardware keying system including, but not limited to, the following:

          - Function of building, flow of traffic, purpose of each area, degree of security required, and plans for future expansion.

          - Preliminary key system schematic diagram.

          - Requirements for key control system.

          - Address for delivery of keys.

Insert agenda items below not listed in Division 1 Section "Project Management and Coordination."

      - DELIVERY, STORAGE, AND HANDLING

        - Inventory door hardware on receipt and provide secure lock-up for door hardware delivered to Project site.

        - Tag each item or package separately with identification related to the final door hardware sets, and include basic installation instructions, templates, and necessary fasteners with each item or package.

Delete paragraph above or paragraph and subparagraph below, or revise to suit Project.  Retain below for high-security cylinders.

        - Deliver keys and permanent cores to Owner by registered mail or overnight package service or in person.

      - COORDINATION

Delete first paragraph below only if all door hardware preparation and installation will be performed at Project site.

        - Templates:  Distribute door hardware templates for doors, frames, and other work specified to be factory prepared for installing door hardware.  Check Shop Drawings of other work to confirm that adequate provisions are made for locating and installing door hardware to comply with indicated requirements.

        - Existing Openings:  Where new hardware components are scheduled for application to existing construction or where modifications to existing door hardware are required, field verify existing conditions and coordinate installation of door hardware to suit opening conditions and to provide for proper operation.

      - WARRANTY

When warranties are required, verify with Owner's counsel that special warranties stated in this Article are not less than remedies available to Owner under prevailing local laws.  Coordinate with Division 1 Section "Product Requirements."

        - Special Warranty:  Manufacturer's standard form in which manufacturer agrees to repair or replace components of door hardware that fail in materials or workmanship within specified warranty period.

          - Failures include, but are not limited to, the following:

            - Structural failures including excessive deflection, cracking, or breakage.

            - Faulty operation of operators and door hardware.

            - Deterioration of metals, metal finishes, and other materials beyond normal weathering and use.

Verify available warranties for units and components and insert number in five subparagraphs below.

          - Warranty Period:  Three years from date of Substantial Completion, except as follows:

            - Electromagnetic Locks:  Five years from date of Substantial Completion.

Delete both subparagraphs below if no closers.  Verify with manufacturers selected.

            - Manual Closers:  10 years from date of Substantial Completion.

            - Exit Devices: Two years from date of Substantial Completion.

      - MAINTENANCE SERVICE

        - Maintenance Tools and Instructions:  Furnish a complete set of specialized tools and maintenance instructions as needed for Owner's continued adjustment, maintenance, and removal and replacement of door hardware.

- PRODUCTS

      - SCHEDULED DOOR HARDWARE

Delete this Article if door hardware allowance is used or if no door hardware sets.

See Editing Instruction No. 1 in the Evaluations for cautions about naming manufacturers and products.

        - General:  Provide door hardware for each door to comply with requirements in this Section and door hardware sets indicated in Part 3 "Door Hardware Sets" Article.

Retain manufacturers' or BHMA designations in first subparagraph below to coordinate with those indicated in Part 3 "Door Hardware Sets" Article.

          - Door Hardware Sets:  Provide quantity, item, size, finish or color indicated, and products equivalent in function and comparable in quality to named products.

          - Sequence of Operation:  Provide electrified door hardware function, sequence of operation, and interface with other building control systems indicated.

        - Designations:  Requirements for design, grade, function, finish, size, and other distinctive qualities of each type of door hardware are indicated in this Section.  Products are identified by using door hardware designations, as follows:

Retain one of two subparagraphs below.

          - Named Manufacturers' Products:  Manufacturer and product designation are listed for each door hardware type required for the purpose of establishing minimum requirements.

Retain subparagraph below for a nonproprietary specification based on referencing BHMA standards, with or without naming available manufacturers.

Edit paragraph and subparagraphs below with other Part 2 articles in which manufacturers are named.  See Division 1 Section "Product Requirements" for an explanation of the terms "Available Manufacturers" and "Manufacturers" and the effect these terms have on "Comparable Product" and "Product Substitution" requirements.

        - In other Part 2 articles where titles below introduce lists, the following requirements apply to product selection:

          - Available Manufacturers:  Subject to compliance with requirements, provide products by one of the manufacturers specified.

See Editing Instruction No. 3 in the Evaluations for selecting door hardware products.

Coordinate remaining Part 2 articles with Part 3 "Door Hardware Sets" Article.  Delete door hardware products not required.

      - HINGES, GENERAL

Delete paragraph and subparagraphs below if only continuous geared hinges are required or if quantity is included in door hardware sets.

        - Quantity:  Provide the following, unless otherwise indicated:

          - Two Hinges:  For doors with heights up to 60 inches (1524 mm).

          - Three Hinges:  For doors with heights 61 to 90 inches (1549 to 2286 mm).

          - Four Hinges:  For doors with heights 91 to 120 inches (2311 to 3048 mm).

Insert sizes, including hinge height and metal thickness, only if they are not included in door hardware sets.

        - Template Requirements:  Except for hinges and pivots to be installed entirely (both leaves) into wood doors and frames, provide only template-produced units.

Delete first paragraph and subparagraphs below if requirements are indicated in door hardware sets.

        - Hinge Weight:  Unless otherwise indicated, provide the following:

          - Doors with Closers:  Antifriction-bearing hinges.

          - Interior Doors:  Heavy-weight hinges.

        - Hinge Base Metal:  Unless otherwise indicated, provide the following:

          - Interior Hinges:  Stainless steel, with stainless-steel pin.

          - Hinges for Fire-Rated Assemblies:  Stainless steel, with stainless-steel pin.

Delete paragraph and subparagraphs below if only continuous geared hinges are required.  Retain subparagraphs to suit Project.

        - Hinge Options:  Where indicated in door hardware sets or on Drawings:

Hospital tips in first subparagraph below are used in locations where additional cleanliness and safety are required.

          - Nonremovable Pins:  Provide set screw in hinge barrel that, when tightened into a groove in hinge pin, prevents removal of pin while door is closed; for all public access doors with locks.

          - Corners:  To match existing door hinges.

          - 180 Degree hinge.

        - Fasteners:  Comply with the following:

          - Machine Screws:  For metal frames.  Install into drilled and tapped holes.

          - Wood Screws:  For wood doors.

          - Threaded-to-the-Head Wood Screws:  For fire-rated wood doors.

          - Screws:  Phillips flat-head; wood screws for wood doors and frames.  Finish screw heads to match surface of hinges.

      - HINGES

BHMA standards in two paragraphs below are used as basis for comparing products even if manufacturers' products are named in door hardware sets.

        - Butts and Hinges:  BHMA A156.1.  Listed under Category A in BHMA's "Certified Product Directory."

Paragraph below is only for hollow-metal doors.

        - Template Hinge Dimensions:  BHMA A156.7.

        - Available Manufacturers:

- Baldwin Hardware Corporation (BH).

- Bommer Industries, Inc. (BI).

- Cal-Royal Products, Inc. (CCRP).

- Hagar Companies (HAG).

- Lawrence Brothers, Inc. (LB).

- McKinney Products company; an ASSA ABLOY Group Company (MCK).

- PBB, Inc. (PBB).

- Stanley commercial hardware; Div. of the Stanley Works (STW).

Coordinate paragraph and list below with Part 2 "Scheduled Door Hardware" Article.  Retain "Available" for nonproprietary and delete for semiproprietary specifications.

      - LOCKS AND LATCHES, GENERAL

Generally retain first option in first paragraph below for private projects; retain third option for Federal, State, and local government projects.  Second option is often referenced by building codes.  Verify requirements with authorities having jurisdiction.

        - Accessibility Requirements:  Comply with the U.S. Architectural & Transportation Barriers Compliance Board's "Americans with Disabilities Act (ADA), Accessibility Guidelines for Buildings and Facilities (ADAAG)." and FED-STD-795, "Uniform Federal Accessibility Standards.”

          - Provide operating devices that do not require tight grasping, pinching, or twisting of the wrist and that operate with a force of not more than 5 lbf (22 N).

        - Latches and Locks for Means of Egress Doors:  Comply with NFPA 101.  Latches shall not require more than 15 lbf (67 N) to release the latch.  Locks shall not require use of a key, tool, or special knowledge for operation.

Revise first paragraph and subparagraphs below if more than one type of lock trim is needed.

        - Lock Trim:

Most levers in first subparagraph below are cast.

          - Levers:  Cast.

Most knobs in first subparagraph below are wrought.

          - Escutcheons (Roses):  Wrought.

          - Dummy Trim:  Match lever lock trim and escutcheons.

Retain first option in subparagraph below if BHMA standards are used to specify door hardware and lock design is indicated on Drawings.

        - Lock Throw:  Comply with testing requirements for length of bolts required for labeled fire doors, and as follows:

          - Bored Locks:  Minimum 1/2-inch (13-mm) latchbolt throw.

          - Mortise Locks:  Minimum 3/4-inch (19-mm) latchbolt throw.

          - Deadbolts:  Minimum 1-inch (25-mm) bolt throw.

        - Backset:  2-3/4 inches (70 mm), unless otherwise indicated.

        - Strikes:  Manufacturer's standard strike with strike box for each latchbolt or lock bolt, with curved lip extended to protect frame, finished to match door hardware set, and as follows:

Retain subparagraphs below as basis for comparing products even if manufacturers' products are named in door hardware sets.

          - Strikes for Bored Locks and Latches:  BHMA A156.2.

          - Strikes for Mortise Locks and Latches:  BHMA A156.13.

          - Strikes for Auxiliary Deadlocks:  BHMA A156.5.

Delete subparagraphs below if not required.

          - Flat-Lip Strikes:  For locks with three-piece antifriction latchbolts, as recommended by manufacturer.

          - Extra-Long-Lip Strikes:  For locks used on frames with applied wood casing trim.

          - Aluminum-Frame Strike Box:  Manufacturer's special strike box fabricated for aluminum framing.

      - MECHANICAL LOCKS AND LATCHES

        - Lock Functions:  Function numbers and descriptions indicated in door hardware sets comply with the following:

          - Bored Locks:  BHMA A156.2.

          - Mortise Locks:  BHMA A156.13.

BHMA standards in three paragraphs below are used as basis for comparing products even if manufacturers' products are named in door hardware sets.  Delete grade options if grade is indicated in door hardware sets.  Insert function requirements with each set if an allowance is used to specify door hardware.

        - Bored Locks:  BHMA A156.2, Grade 1.

Coordinate subparagraph and list below with Part 2 "Scheduled Door Hardware" Article.  Retain "Available" for nonproprietary and delete for semiproprietary specifications.

          - Provide product indicated in Part 3 “Door Hardware Sets” by the following manufacturer:

            - Marks USA.

        - Mortise Locks:  Stamped steel case with steel or brass parts; BHMA A156.13, Grade 1.

Coordinate subparagraph and list below with Part 2 "Scheduled Door Hardware" Article.  Retain "Available" for nonproprietary and delete for semiproprietary specifications.

          - Provide product indicated in Part 3 “Door Hardware Sets” by the following manufacturer:

            - Marks USA.

      - ELECTRIC STRIKES

        - Electric Strikes: BHMA A156.31; Grade 1; with faceplate to suit lock and frame.

          - Available Manufacturers:

            - Adams Rite Manufacturing Co.; an ASSA ABLOY Group Company.

            - Dortronics Systems, Inc.

            - DynaLock Corp.

            - Folger Adam Electric Door Controls; an ASSA ABLOY Group Company.

            - HES, Inc.; an ASSA ABLOY Group Company.

            - Rutherford Controls Int’l. Corp.

            - Security Door Controls.

            - Trine Access Technology.

            - Von Duprin; an Ingersoll-Rand company.

Retain first subparagraph below if retaining card reader option in subparagraph above.

Retain paragraph below if retaining card reader option above.

Coordinate paragraph and list below with Part 2 "Scheduled Door Hardware" Article.  Retain "Available" for nonproprietary and delete for semiproprietary specifications.

Flush bolts in paragraph below are for inactive leaf of a pair of doors.  BHMA standard is used as basis for comparing products even if manufacturers' products are named in door hardware sets.  Delete options if grade is indicated in door hardware sets.

      - EXIT DEVICES

BHMA standard in paragraph below is used as basis for comparing products even if manufacturers' products are named in door hardware sets.  Delete grade options if grade is indicated in door hardware sets.

        - Exit Devices:  BHMA A156.3, Grade 1.

Generally retain first option in first paragraph below for private projects; retain third option for Federal, State, and local government projects.  Second option is often referenced by building codes.  Verify requirements with authorities having jurisdiction.

        - Accessibility Requirements:  Where handles, pulls, latches, locks, and other operating devices are indicated to comply with accessibility requirements, comply with the U.S. Architectural & Transportation Barriers Compliance Board's "Americans with Disabilities Act (ADA), Accessibility Guidelines for Buildings and Facilities (ADAAG)." and FED-STD-795, "Uniform Federal Accessibility Standards."

          - Provide operating devices that do not require tight grasping, pinching, or twisting of the wrist and that operate with a force of not more than 5 lbf (22 N).

        - Exit Devices for Means of Egress Doors:  Comply with NFPA 101.  Exit devices shall not require more than 15 lbf (67 N) to release the latch.  Locks shall not require use of a key, tool, or special knowledge for operation.

Retain one of two paragraphs below, or both.  Only fire exit devices may be used for fire doors.  NFPA 80 distinguishes between panic exit hardware and fire exit hardware.  See Evaluations.

        - Panic Exit Devices:  Listed and labeled by a testing and inspecting agency acceptable to authorities having jurisdiction, for panic protection, based on testing according to UL 305.

UL test in paragraph below includes operational test of 100,000 cycles.  BHMA A156.3 requires 250,000 cycles for Grade 1 and 100,000 cycles for Grade 2.

        - Fire Exit Devices:  Devices complying with NFPA 80 that are listed and labeled by a testing and inspecting agency acceptable to authorities having jurisdiction, for fire and panic protection, based on testing according to UL 305 and NFPA 252.

Delete first two paragraphs below if removable mullions are specified with door frames in other Division 8 Sections.

        - Dummy Push Bar:  Nonfunctioning push bar matching functional push bar.

          - Operation:  Rigid.

Retain paragraph and subparagraph below if exit device levers, knobs, and pulls are required to match lockset and latchset designs.  Matching locksets to exit device trim and levers limits the design options available; coordinate with lock design selected in "Locks and Latches, General" Article.

Electronic exit bars in paragraph below can be used only with electrified locks.

Coordinate paragraph and list below with Part 2 "Scheduled Door Hardware" Article.  Retain "Available" for nonproprietary and delete for semiproprietary specifications.

        - Available Manufacturers:

          - Corbin Russwin Architectural Hardware; an ASSA ABLOY Group company (CR).

          - Precision Hardware, Inc. (PH).

          - SARGENT Manufacturing Company; an ASSA ABLOY Group company (SGT).

          - Yale Commercial Locks and Hardware; an ASSA ABLOY Group company (YAL).

      - LOCK CYLINDERS

BHMA standards in two paragraphs and associated subparagraphs below are used as basis for comparing products even if manufacturers' products are named in door hardware sets.  Delete options if grade is indicated in door hardware sets.

        - Standard Lock Cylinders:  BHMA A156.5, Grade 1.

Revise first paragraph below to specify manufacturers' proprietary cylinder systems.

        - Cylinders:  Manufacturer's standard tumbler type, constructed from brass or bronze, stainless steel, or nickel silver, and complying with the following:

First option in subparagraph below is not available for high-security cylinders.

          - Number of Pins:  Six.

Select applicable types from three subparagraphs below.

          - Mortise Type:  Threaded cylinders with rings and straight- or clover-type cam.

          - Rim Type:  Cylinders with back plate, flat-type vertical or horizontal tailpiece, and raised trim ring.

          - Bored-Lock Type:  Cylinders with tailpieces to suit locks.

Retain subparagraph below with any of three subparagraphs above; high-security grade is not available with interchangeable core.

            - High-Security Grade:  BHMA A156.5, Grade 1A, listed and labeled as complying with pick- and drill-resistant testing requirements in UL 437 (Suffix A).

        - Permanent Cores:  Manufacturer's standard; finish face to match lockset; complying with the following:

Retain one of two subparagraphs below.  Second describes tumbler proprietary to cylinder manufacturer.

          - Removable Cores:  Core insert, removable by use of a special key; for use only with core manufacturer's cylinder and door hardware.

        - Construction Keying:  Comply with the following:

Retain one of first two subparagraphs below.

          - Construction Master Keys:  Provide cylinders with feature that permits voiding of construction keys without cylinder removal.  Provide 10 construction master keys.

          - Construction Cores:  Provide construction cores that are replaceable by permanent cores.  Provide 10 construction master keys.

            - Replace construction cores with permanent cores as directed by Contracting Officer.

Retain subparagraph above or below.

            - Furnish permanent cores to Owner for installation.

        - Manufacturer:  Medeco Security Locks, Inc; an ASSA ABLOY Group Company.

          - Provide product indicated in Part 3 “Door Hardware Sets.”

Retain paragraph above or paragraph and list below.  Above significantly reduces competition and may result in higher costs.  Coordinate below with Part 2 "Scheduled Door Hardware" Article.  Retain "Available" for nonproprietary and delete for semiproprietary specifications.

      - KEYING

        - Keying System:  Factory registered, complying with guidelines in BHMA A156.28, Appendix A.  Incorporate decisions made in keying conference, and as follows:

Retain one of four subparagraphs below.

Retain both subparagraphs below if required.

          - Existing System:  Master key or grand master key locks to Owner's existing system.

          - Key Pyramid:

- Building Master Key – Open all doors in the building.

- Tenant Sub-Master – Open all doors in tenant space only.

- Individual Keys – Open each individual’s office.

          - (Entry into Tenant space) Doors must be keyed alike and be on the Building Master and Sub-Master.

          - (Private Offices) Doors are to be keyed individually and be on the Tenant Sub-Master. Provide two keys for each door.

        - Keys:  Nickel silver.

Delete first subparagraph and associated subparagraph below if key does not require special marking.

          - Stamping:  Permanently inscribe each key with a visual key control number and include the following notation:

            - Notation:  Information to be furnished by Contracting Officer.

          - Quantity:  In addition to one extra key blank for each lock, provide the following:

Retain subparagraphs below that correspond to type of keying system selected above.

            - Cylinder Change Keys:  Three.

            - Master Keys:  Five.

            - Grand Master Keys:  Five.

            - Great-Grand Master Keys:  Five.

      - OPERATING TRIM

BHMA standard in first paragraph below is used as basis for comparing products even if manufacturers' products are named in door hardware sets.  Delete option if naming manufacturers' products.

        - Standard:  BHMA A156.6.

        - Materials:  Fabricate from brass or stainless steel, unless otherwise indicated.

Coordinate paragraph and list below with Part 2 "Scheduled Door Hardware" Article.  Retain "Available" for nonproprietary and delete for semiproprietary specifications.

        - Available Manufacturers:

          - Burns Manufacturing Incorporated (BM).

          - Don-Jo Mfg., Inc. (DJO).

          - Forms + Surfaces (FS).

          - Hager Companies (HAG).

          - Hiawatha, Inc. (HIA).

          - IVES Hardware; an Ingersoll-Rand Company (IVS).

          - Rockwood Manufacturing Company (RM).

          - Trimco (TBM).

      - CLOSERS

Generally retain first option in first paragraph below for private projects; retain third option for Federal, State, and local government projects.  Second option is often referenced by building codes.  Verify requirements with authorities having jurisdiction.

        - Accessibility Requirements:  Where handles, pulls, latches, locks, and other operating devices are indicated to comply with accessibility requirements, comply with the U.S. Architectural & Transportation Barriers Compliance Board's "Americans with Disabilities Act (ADA), Accessibility Guidelines for Buildings and Facilities (ADAAG)." and FED-STD-795, "Uniform Federal Accessibility Standards."

          - Comply with the following maximum opening-force requirements:

            - Interior, Non-Fire-Rated Hinged Doors:  5 lbf (22.2 N) applied perpendicular to door.

        - Door Closers for Means of Egress Doors:  Comply with NFPA 101.  Door closers shall not require more than 30 lbf (133 N) to set door in motion and not more than 15 lbf (67 N) to open door to minimum required width.

Delete first paragraph below if not required.

Retain first paragraph below if needed.

        - Size of Units:  Unless otherwise indicated, comply with manufacturer's written recommendations for size of door closers depending on size of door, exposure to weather, and anticipated frequency of use.  Provide factory-sized closers, adjustable to meet field conditions and requirements for opening force.

BHMA standards in first three paragraphs below are used as basis for comparing products even if manufacturers' products are named in door hardware sets.  Delete grade options if grade is indicated in door hardware sets.

        - Surface Closers:  BHMA A156.4, Grade 1.

Coordinate subparagraph and list below with Part 2 "Scheduled Door Hardware" Article.  Retain "Available" for nonproprietary and delete for semiproprietary specifications.

          - Provide the following product or equal:

            - LCN Closers; an Ingersoll-Rand Company (LCN); 4040 Series, unless otherwise noted in Hardware Sets.

        - Coordinators:  BHMA A156.3.

      - DOOR GASKETING

        - Door Gasketing: BHMA A156.22; air leakage not to exceed 0.50 cfm per foot of crack length for gasketing other than for smoke control, as tested according to ASTM E 283; with resilient or flexible seal strips that are easily replaceable and readily available from stocks maintained by manufacturer.

        - Door Bottom Gasketing: Surface mounted or semi-mortised automatic door bottom.

          - Available Manufacturers:

            - Hager Companies.

            - National Guard Products.

            - Pemko Manufacturing Co.; an ASSA ABLOY Group company.

            - Reese Enterprises, Inc.

            - Sealeze; a unit of Jason Incorporated.

            - Zero International.

      - THRESHOLDS

        - Thresholds: BHMA A156.21; fabricated to full width of opening indicated.

          - Available Manufacturers:

            - Hager Companies.

            - National Guard Products.

            - Pemko Manufacturing Co.; an ASSA ABLOY Group Company.

            - Reese Enterprises, Inc.

            - Sealeze; a unit of Jason Incorporated.

            - Zero International.

      - PROTECTIVE TRIM UNITS

ADA's "Accessibility Guidelines" Appendix recommends that protection plates be sized 2 inches (51 mm) less than door width.

        - Size:  1-1/2 inches (38 mm) less than door width on push side and 1/2 inch (13 mm) less than door width on pull side, by height specified in door hardware sets.

        - Fasteners:  Manufacturer's standard machine or self-tapping screws.

BHMA standard in two paragraphs below is used as basis for comparing products even if manufacturers' products are named in door hardware sets.

        - Metal Protective Trim Units:  BHMA A156.6; beveled top and 2 sides; fabricated from the following material:

          - Material:  0.050-inch- (1.3-mm-) thick stainless steel.

Coordinate subparagraph and list below with Part 2 "Scheduled Door Hardware" Article.  Retain "Available" for nonproprietary and delete for semiproprietary specifications.

          - Available Manufacturers:

            - Baldwin Hardware Corporation (BH).

            - Burns Manufacturing Incorporated (BM).

            - Don-Jo Mfg., Inc. (DJO).

            - Hager Companies (HAG).

            - IPC Door and Wall Protection Systems, Inc.; Div. of InPro Corporation (IPC).

            - IVES Hardware; an Ingersoll-Rand Company (IVS).

            - Rockwood Manufacturing Company (RM).

            - Trimco (TBM).

      - STOPS

BHMA standards in this Article are used as basis for comparing products even if manufacturers' products are named in door hardware sets.  Delete grade options if grade is indicated in door hardware sets.

        - Stops and Bumpers:  BHMA A156.16, Grade 1.

          - Provide floor stops for doors unless wall or other type stops are scheduled or indicated.  Do not mount floor stops where they will impede traffic.  Where floor stops are not appropriate, provide wall stops.

        - Combination Floor and Wall Stops and Holders:  BHMA A156.8, Grade 1.

Delete paragraph below if silencers are specified with metal door frames in another Division 8 Section.

        - Silencers for Metal Door Frames:  BHMA A156.16, Grade 1; neoprene or rubber, minimum diameter 1/2 inch (13 mm); fabricated for drilled-in application to frame.

Coordinate paragraph and list below with Part 2 "Scheduled Door Hardware" Article.  Retain "Available" for nonproprietary and delete for semiproprietary specifications.

        - Available Manufacturers:

          - Architectural Builders Hardware Mfg., Inc. (ABH).

          - Baldwin Hardware Corporation (BH).

          - Glynn-Johnson; an Ingersoll-Rand Company (GJ).

          - IVES Hardware; an Ingersoll-Rand Company (IVS).

          - SARGENT Manufacturing Company; an ASSA ABLOY Group company (SGT).

          - Stanley Commercial Hardware; Div. of The Stanley Works (STH).

      - MISCELLANEOUS DOOR HARDWARE

        - Boxed Power Supplies:  Modular unit in NEMA ICS 6, Type 4 enclosure; filtered and regulated; voltage rating and type matching requirements of door hardware served; and listed and labeled for use with fire alarm systems.

BHMA standard in paragraph below is used as basis for comparing products even if manufacturers' products are named in door hardware sets.  Delete options if grade is indicated in door hardware sets.

        - Auxiliary Hardware:  BHMA A156.16, Grade 1.

Coordinate subparagraph and list below with Part 2 "Scheduled Door Hardware" Article.  Retain "Available" for nonproprietary and delete for semiproprietary specifications.

          - Available Manufacturers:

            - Baldwin Hardware Corporation (BH).

            - Cal-Royal Products, Inc. (CRP).

            - Don-Jo Mfg., Inc. (DJO).

            - Hager Companies (HAG).

            - Lawrence Brothers, Inc. (LB).

            - Rockwood Manufacturing Company (RM).

            - Stanley Commercial Hardware; Div. of The Stanley Works (STH).

            - Trimco (TBM).

        - Hinge Filler Plate

- 10 GA minimum with finish to match frame.

        - Strike Cutout Filler Plate

- 10 GA minimum with finish to match frame.

      - FABRICATION

Delete first paragraph and subparagraph below if cost is prime consideration or if visibility of manufacturer's name on items is not objectionable.

        - Manufacturer's Nameplate:  Do not provide products that have manufacturer's name or trade name displayed in a visible location except in conjunction with required fire-rated labels and as otherwise approved by Contracting Officer.

          - Manufacturer's identification is permitted on rim of lock cylinders only.

        - Base Metals:  Produce door hardware units of base metal, fabricated by forming method indicated, using manufacturer's standard metal alloy, composition, temper, and hardness.  Furnish metals of a quality equal to or greater than that of specified door hardware units and BHMA A156.18.  Do not furnish manufacturer's standard materials or forming methods if different from specified standard.

        - Fasteners:  Provide door hardware manufactured to comply with published templates generally prepared for machine, wood, and sheet metal screws.  Provide screws according to commercially recognized industry standards for application intended, except aluminum fasteners are not permitted.  Provide Phillips flat-head screws with finished heads to match surface of door hardware, unless otherwise indicated.

Delete or revise subparagraph below for low-cost work and where visual quality is unimportant.  Coordinate with Division 8 Section "Flush Wood Doors."

          - Concealed Fasteners:  For door hardware units that are exposed when door is closed, except for units already specified with concealed fasteners.  Do not use through bolts for installation where bolt head or nut on opposite face is exposed unless it is the only means of securely attaching the door hardware.  Where through bolts are used on hollow door and frame construction, provide sleeves for each through bolt.

NFPA 80 requires locks, latches, surface-mounted top and bottom bolts, and fire exit hardware to be secured with machine screws or through bolts.

          - Fasteners for Wood Doors:  Comply with requirements in DHI WDHS.2, "Recommended Fasteners for Wood Doors."

      - FINISHES

        - Standard:  BHMA A156.18, 26D Satin Chromium Finish unless indicated otherwise.

        - Protect mechanical finishes on exposed surfaces from damage by applying a strippable, temporary protective covering before shipping.

        - Appearance of Finished Work:  Variations in appearance of abutting or adjacent pieces are acceptable if they are within one-half of the range of approved Samples.  Noticeable variations in the same piece are not acceptable.  Variations in appearance of other components are acceptable if they are within the range of approved Samples and are assembled or installed to minimize contrast.

- EXECUTION

      - EXAMINATION

        - Examine doors and frames, with Installer present, for compliance with requirements for installation tolerances, labeled fire door assembly construction, wall and floor construction, and other conditions affecting performance.

        - Proceed with installation only after unsatisfactory conditions have been corrected.

      - PREPARATION

NFPA 80 restricts on-site door preparation to that required for surface-applied door hardware, function holes for mortise locks, holes for labeled viewers, undercutting of doors, and protection plates.

        - Steel Doors and Frames:  Comply with DHI A115 Series.

          - Surface-Applied Door Hardware:  Drill and tap doors and frames according to ANSI A250.6.

        - Wood Doors:  Comply with DHI A115-W Series.

      - INSTALLATION

        - Mounting Heights:  Mount door hardware units at heights indicated as follows unless otherwise indicated or required to comply with governing regulations.

Retain subparagraphs below to suit Project.

          - Wood Doors:  DHI WDHS.3, "Recommended Locations for Architectural Hardware for Wood Flush Doors."

          - Standard Steel Doors and Frames: ANSI/SDI A250.8.

        - Install each door hardware item to comply with manufacturer's written instructions.  Where cutting and fitting are required to install door hardware onto or into surfaces that are later to be painted or finished in another way, coordinate removal, storage, and reinstallation of surface protective trim units with finishing work specified in Division 9 Sections.  Do not install surface-mounted items until finishes have been completed on substrates involved.

          - Set units level, plumb, and true to line and location.  Adjust and reinforce attachment substrates as necessary for proper installation and operation.

          - Drill and countersink units that are not factory prepared for anchorage fasteners.  Space fasteners and anchors according to industry standards.

        - Key Control System:  Tag keys and place them on markers and hooks in key control system cabinet, as determined by final keying schedule.

Delete first paragraph and subparagraphs below if electrified door hardware is not required.  Verify acceptable location for power supplies with authorities having jurisdiction.

      - ADJUSTING

        - Initial Adjustment:  Adjust and check each operating item of door hardware and each door to ensure proper operation or function of every unit.  Replace units that cannot be adjusted to operate as intended.  Adjust door control devices to compensate for final operation of heating and ventilating equipment and to comply with referenced accessibility requirements.

          - Spring Hinges:  Adjust to achieve positive latching when door is allowed to close freely from an open position of 30 degrees.

Subparagraph below is from ADA's "Accessibility Guidelines."

          - Door Closers:  Unless otherwise required by authorities having jurisdiction, adjust sweep period so that, from an open position of 70 degrees, the door will take at least 3 seconds to move to a point 3 inches (75 mm) from the latch, measured to the leading edge of the door.

Requirements in paragraph below increase cost but are recommended as a good investment on substantial projects even though they may be difficult to monitor.

        - Occupancy Adjustment:  Approximately six months after date of Substantial Completion, Installer's Architectural Hardware Consultant shall examine and readjust, including adjusting operating forces, each item of door hardware as necessary to ensure function of doors and door hardware.

      - CLEANING AND PROTECTION

        - Clean adjacent surfaces soiled by door hardware installation.

        - Clean operating items as necessary to restore proper function and finish.

        - Provide final protection and maintain conditions that ensure that door hardware is without damage or deterioration at time of Substantial Completion.

      - DEMONSTRATION

Possibly delete this Article for small projects.

        - Engage a factory-authorized service representative to train Owner's maintenance personnel to adjust, operate, and maintain door hardware and door hardware finishes.  Refer to Division 1 Section "Demonstration and Training."

      - DOOR HARDWARE SETS

        - Door Hardware Set No. 1

- NOT USED.

        - Door Hardware Set No. 2

- Door No. 101, 107

          - 3 hinges.

          - 1 cylindrical lockset with lever handle: MARKS 195-RF/26D-F21.

          - 1 interchangeable core cylinder: Medeco UL 437 M-3.

          - 1 electric strike.

          - 1 door contact.

          - 1 closer: BHMA C02021

          - 1 lock guard.

          - 1 floor bumper.

          - 3 silencers.

          - 1 threshold.

          - 1 automatic door bottom.

          - Perimeter gasketing.

          - Kickplate, 8 inches high – push side of door leaf.

        - Door Hardware Set No. 3

- Door No. 102, 106, 104, 105, 108, 111, 113

          - 3 hinges.

          - 1 cylindrical lockset with lever handle: MARKS 195-RF/26D-F21.

          - 1 interchangeable core cylinder: Medeco UL 437 M-3.

          - 1 electric strike.

          - 1 door contact.

          - 1 closer: BHMA C02011

          - 1 floor bumper.

          - 3 silencers.

          - 1 threshold.

          - 1 automatic door bottom.

          - Perimeter gasketing.

          - Kickplate, 8 inches high – push side of door leaf.

        - Door Hardware Set No. 4

- Door No. 110

          - 3 non-removable hinges.

          - 1 cylindrical lockset with lever handle: MARKS 5-CP-92-FS/32D-G3.

          - 1 interchangeable core cylinder: Medeco UL 437 M-3, #32-0200 housing

          - 1 electric strike.

          - 1 door contact.

          - 1 closer: BHMA C02011

          - 1 lock guard.

          - 1 floor bumper.

          - 3 silencers.

          - 1 threshold.

          - 1 automatic door bottom.

          - Perimeter gasketing.

          - Kickplate, 8 inches high – push side of door leaf.

        - Door Hardware Set No. 5

- Door No. 109, 112

          - 3 non-removable hinges.

          - 1 cylindrical lockset with lever handle: MARKS 5-CP-92-FS/32D-G3.

          - 1 interchangeable core cylinder: Medeco UL 437 M-3, #32-0200 housing

          - 1 exit device.

          - 1 electric strike.

          - 1 door contact.

          - 1 closer: BHMA C02021

          - 1 lock guard.

          - 1 floor bumper.

          - 3 silencers.

          - 1 threshold.

          - 1 automatic door bottom.

          - Perimeter gasketing.

          - Kickplate, 8 inches high – push side of door leaf.

        - Door Hardware Set No. 6

- Door No. 114

          - 3 non-removable hinges.

          - 1 exit device (exit only).

          - 1 electric strike.

          - 1 door contact.

          - 1 closer: BHMA C02021

          - 1 lock guard.

          - 1 floor bumper.

          - 3 silencers.

          - 1 threshold.

          - 1 automatic door bottom.

          - Perimeter gasketing.

          - Kickplate, 8 inches high – push side of door leaf.

        - Door Hardware Set No. 7

- Door No. 103

          - 3 non-removable hinges.

          - 1 door contact.

- Balance of hardware existing.

END OF SECTION 087100