Copyright 2008 by The American Institute of Architects (AIA)

Exclusively published and distributed by Architectural Computer Services, Inc. (ARCOM) for the AIA

SECTION 088720 – ARCHITECTURAL WINDOW FILM

Revise this Section by deleting and inserting text to meet Project-specific requirements.

This Section uses the term "Architect."  Change this term to match that used to identify the design professional as defined in the General and Supplementary Conditions.

Verify that Section titles referenced in this Section are correct for this Project's Specifications; Section titles may have changed.

- GENERAL

      - RELATED DOCUMENTS

Retain or delete this article in all Sections of Project Manual.

        - Drawings and general provisions of the Contract, including General and Supplementary Conditions and Division 01 Specification Sections, apply to this Section.

      - SUMMARY

        - Section includes the following decorative glass for interior applications:

Decorative glass may be a combination of descriptions.

          - Decorative film overlay for existing windows.

      - PERFORMANCE REQUIREMENTS

        - Fire Performance: Surface burning characteristics when tested in accordance ASTM E 84:

Retain this article if laminated glass is installed with glazing sealants or if glass with decorative film overlay in contact with glazing sealants is specified.  Adhesion and compatibility testing is essential and usually performed by sealant manufacturer.  Tests require many Samples, and some tests require four weeks to complete.  If retaining, also retain "Preconstruction Adhesion and Compatibility Test Reports" Paragraph in "Submittals" Article.

      - SUBMITTALS

First six paragraphs below are defined in Division 01 Section "Submittal Procedures" as "action submittals."

        - Product Data: Manufacturer’s data sheets on each product to be used, including:

          - Preparation instructions and recommendations.

          - Storage and handling requirements and recommendations.

          - Installation methods.

        - Samples for Color Selection:  Manufacturer's standard sample sets showing the full range of colors available for each type of product indicated.

        - Samples for Verification:  12-inch square samples of each glazing film, of each product color specified.

Revise list below to coordinate with products retained.

      - QUALITY ASSURANCE

If retaining first paragraph below, verify, with prospective installers, that they can comply with certification requirements referenced.

        - Installer Qualifications:  Engage an experienced installer certified, licensed, or otherwise qualified by film manufacturer as having the necessary experience, staff, and training to install manufacturer's products according to specified requirements.

        - Mockups:  Provide a mock-up for evaluation of surface preparation techniques and application workmanship.

          - Build mockups in the location and of the size indicated or, if not indicated, as directed by Architect.

          - Do not proceed with remaining work until workmanship, color and sheen are approved by Architect.

Retain subparagraph below if mockups are installed as part of building rather than erected separately and the intention is to make an exception to the default requirement in Division 01 Section "Quality Requirements" for demolishing and removing mockups when directed unless otherwise indicated.

          - Approved mockups may become part of the completed Work if undisturbed at time of Substantial Completion.

Retain paragraph below if Work of this Section is extensive or complex enough to justify a preinstallation conference.

      - DELIVERY, STORAGE, AND HANDLING

        - Protect glazing films according to manufacturer's written instructions and as needed to prevent damage condensation, temperature changes, direct exposure to sun, or other causes.

      - PROJECT CONDITIONS

        - Environmental Limitations:  Environmental Limitations:  Do not proceed with film installation when ambient and substrate temperature conditions are outside limits permitted by manufacturer and when glass substrates are wet from frost, condensation, or other causes.

      - WARRANTY

When warranties are required, verify with Owner's counsel that special warranties stated in this article are not less than remedies available to Owner under prevailing local laws.

Special warranty in this article is an example only.  See the Evaluations in Division 08 Section "Glazing" for warranty discussion.

        - At project closeout, provide to Owner or Owners Representative an executed current copy of the manufacturer’s standard limited warranty against manufacturing defect, outlining its terms, conditions, and exclusions from coverage.

- PRODUCTS

Retain paragraph below for glass with PVC film overlay that simulates an acid-etched or sandblasted appearance.

        - Architectural Window Film.  Use single or multi-layered decorative film products applied to interior (project side) glass surfaces to achieve desired level of translucence.

See Editing Instruction No. 1 in the Evaluations for cautions about naming manufacturers and products.  See Division 01 Section "Product Requirements."

          - Products:  Subject to compliance with requirements, provide one of the following:

Before retaining products in list below, verify opaqueness, appearance, and simulation quality.

            - Llumar Designer Window Film.

            - 3M Fasara.

            - 3M Crystal Window Finishes.

      - GLAZING FILM ACCESSORIES

Retain first paragraph below if decorative glass is glazed.  Revise by importing text from Division 08 Section "Glazing" if Project does not include that Section but glazing of decorative glass is required.

        - General: Provide products complying with requirements of glazing film manufacturer for application indicated, and with a proven record of compatibility with surfaces contacted in installation.

        - Adhesive: Pressure Sensitive acrylic adhesive system.

        - Cleaners, Primers, and Sealers: Types recommended by glazing film manufacturer.

- EXECUTION

      - EXAMINATIO

        - Examine glass and surrounding adjacent surfaces for conditions affecting installation.

          - Report conditions that may adversely affect installation.  In report, include description of any glass that is broken, chipped, cracked, abraded, or damaged in any way.

        - Proceed with installation only after unsatisfactory conditions have been corrected.

        - Beginning of installation means acceptance of conditions.

Retain, revise, or delete this article to suit Project.

      - PREPARATION

        - Comply with manufacturer's written instructions for surface preparation.

        - Immediately before beginning installation of films, clean glass surfaces of substances that could impair glazing film's bond, including mold, mildew, oil, grease, dirt and other foreign materials.

        - Protect window frames and surrounding conditions from damage during installation.

      - INSTALLATION

        - General:  Comply with glazing film manufacturers' written installation instructions applicable to products and applications indicated, except where more stringent requirements apply.

          - Install film continuously, but not necessarily in one continuous length.  Install with no gaps or overlaps.

          - If seamed, install with no gaps or overlaps.  Install seams vertical and plumb.  No horizontal seams allowed.

          - Do not remove release liner from film until just before each piece of film is cut and ready for installation.

          - Install film with mounting solution and custom cut to the glass with neat, square comers and edges to within 1/8 inch of the window frame.

          - Remove air bubbles, wrinkles, blisters, and other defects.

        - After installation, view film from a distance of 10 feet against a bright uniform sky or background.  Film shall appear uniform in appearance with no visible streaks, banding, thin spots or pinholes.

          - If installed film does not meet these criteria, remove and replace with new film.

      - CLEANING

        - Remove excess mounting solution at finished seams, perimeter edges, and adjacent surfaces.

        - Use cleaning methods recommended by glazing film manufacturer.

Retain, revise, or delete this article to suit Project.

END OF SECTION 088720