Copyright 2004 by The American Institute of Architects (AIA)

Exclusively published and distributed by Architectural Computer Services, Inc. (ARCOM) for the AIA

SECTION 092216 - NON-STRUCTURAL METAL FRAMING

Verify that Section titles referenced in this Section are correct for this Project's Specifications; Section titles may have changed.

- GENERAL

      - RELATED DOCUMENTS

        - Drawings and general provisions of the Contract, including General and Supplementary Conditions and Division 1 Specification Sections, apply to this Section.

      - SUMMARY

        - This Section includes non-load-bearing steel framing members for the following applications:

Adjust list below to suit Project.

          - Interior framing systems (e.g., supports for partition walls, framed soffits, furring, etc.).

          - Interior supplementary framing and blocking to support fixtures, equipment services, heavy trim, grab bars, toilet accessories, furnishings, or similar construction.

List below only products and construction that the reader might expect to find in this Section but are specified elsewhere.

Delete first subparagraph below if no Z-shaped furring.

      - SUBMITTALS

        - Product Data:  For each type of product indicated.

      - QUALITY ASSURANCE

Retain paragraph below where framing is part of fire-resistance-rated assemblies.  Indicate design designations of specific assemblies on Drawings.

Retain paragraph below where framing is part of STC-rated assemblies.  Indicate design designations of specific assemblies on Drawings.

        - STC-Rated Assemblies:  For STC-rated assemblies, provide materials and construction identical to those tested in assembly indicated according to ASTM E 90 and classified according to ASTM E 413 by an independent testing agency.

- PRODUCTS

      - NON-STRUCTURAL METAL FRAMING, GENERAL

Retain first paragraph below if recycled content is required for LEED-NC or LEED-CI Credits MR 4.1 and MR 4.2.  USGBC allows a default value of 25 percent to be used for steel without documentation; higher percentages can be claimed if they are supported by appropriate documentation.  The Steel Recycling Institute indicates that steel sheet is made by the basic oxygen furnace method, which typically has 23 percent postconsumer recycled content and 1.5 percent preconsumer recycled content.

        - Recycled Content of Steel Products:  Provide products with average recycled content of steel products such that postconsumer recycled content plus one-half of preconsumer recycled content is not less than 25 percent.

        - Framing Members, General:  Comply with ASTM C 754 for conditions indicated.

          - Steel Sheet Components:  Comply with ASTM C 645 requirements for metal, unless otherwise indicated.

See "Corrosion Protection of Steel Framing" Article in the Evaluations for a discussion of corrosion-resistant coatings on components.

          - Protective Coating:  ASTM A 653/A 653M, G40 (Z120) (Z180) (Z120), hot-dip galvanized, unless otherwise indicated.

      - STEEL FRAMING FOR FRAMED ASSEMBLIES

        - Steel Studs and Runners:  ASTM C 645.

GA-600 recommends 0.0312-inch- (0.79-mm-) thick studs at fire-door frames supporting standard and heavyweight doors, but it includes an alternate detail for nested 0.0179-inch (0.45-mm) studs for standard-weight doors.

For framing supporting ceramic tile substrates, 0.0312-inch- (0.79-mm-) thick studs are generally recommended.

If runners must have a heavier base-metal thickness than studs, revise subparagraph below.

          - Minimum Base-Metal Thickness:  20 GA (0.45 mm) (0.7 mm) (0.79 mm).

          - Depth:  As indicated on Drawings (92.1 mm) (152.4 mm) (101.6 mm) (63.5 mm) (41.3 mm).

For slip joint at stud and top-track interface (head joint) that avoids axial loading of partition by overhead structure, retain paragraph and one or all of the following types of head joints.  See "Crack Control" Article in the Evaluations.

        - Slip-Type Head Joints:  Where indicated, provide one of the following:

If retaining single-track systems below, indicate type of bridging required on Drawings or by inserts.  Strap and 1-1/2-inch (38.1-mm) cold-rolled channel are commonly used.

          - Deflection Track:  Steel sheet top runner manufactured to prevent cracking of finishes applied to interior partition framing resulting from deflection of structure above; in thickness not less than indicated for studs and in width to accommodate depth of studs.

If naming products, retain last subparagraph above for nonproprietary or first subparagraph below for semiproprietary specification.  Refer to Division 1 Section "Product Requirements."

            - Products:  Subject to compliance with requirements, provide one of the following or equal:

              - Steel Network Inc. (The); VertiTrack VTD Series.

              - Superior Metal Trim; Superior Flex Track System (SFT).

Paragraph below specifies firestop tracks for isolating partition framing from structure above in specific fire-resistance-rated assemblies.  If retaining, revise to coordinate with rated assemblies selected and indicate design designations of assemblies on Drawings.

See Editing Instruction No. 1 in the Evaluations for cautions about naming manufacturers and products.  Retain one of two subparagraphs and list of manufacturers and products below.  See Division 1 Section "Product Requirements."

          - Products:  Subject to compliance with requirements, provide one of the following or equal:

            - Fire Trak Corp.; Fire Track attached to studs with Fire Trak Slip Clip.

            - Metal-Lite, Inc.; The System.

Retain paragraph and subparagraph below for flat-strap blocking and bracing for fixture attachment.  Indicate locations, lengths, and widths on Drawings.

        - Flat Strap and Backing Plate:  Steel sheet for blocking and bracing in length and width indicated.

          - Minimum Base-Metal Thickness:  0.027 inch (0.7 mm) (0.79 mm).

Retain first paragraph and subparagraphs below for channel bridging for fixture attachment or lateral bracing.  Indicate locations and details of installation on Drawings.

        - Cold-Rolled Channel Bridging:  0.0538-inch (1.37-mm) bare-steel thickness, with minimum 1/2-inch- (12.7-mm-) wide flanges.

          - Depth:  1-1/2 inches (38.1 mm).

          - Clip Angle:  Not less than 1-1/2 by 1-1/2 inches (38.1 by 38.1 mm), 0.068-inch- (1.73-mm-) thick, galvanized steel.

Z-shaped furring is available in 1-, 1-1/2-, 2-, 2-1/2-, and 3-inch (25.4-, 38.1-, 50.8-, 63.5-, and 76.2-mm) depths.  Coordinate Z-shaped furring requirements with insulation specified in Division 7 Section "Building Insulation."

      - AUXILIARY MATERIALS

        - General:  Provide auxiliary materials that comply with referenced installation standards.

          - Fasteners for Metal Framing:  Of type, material, size, corrosion resistance, holding power, and other properties required to fasten steel members to substrates.

        - Isolation Strip at Exterior Walls:  Provide one of the following:

Retain both subparagraphs below and option in paragraph above to allow Contractor to choose type of isolation strip.

          - Foam Gasket:  Adhesive-backed, closed-cell vinyl foam strips that allow fastener penetration without foam displacement, 1/8 inch (3.2 mm) thick, in width to suit steel stud size.

- EXECUTION

      - EXAMINATION

        - Examine areas and substrates, with Installer present, and including welded hollow-metal frames, cast-in anchors, and structural framing, for compliance with requirements and other conditions affecting performance.

          - Proceed with installation only after unsatisfactory conditions have been corrected.

Delete first paragraph and subparagraph below if only postinstalled anchors are used for installing suspended assemblies.

      - INSTALLATION, GENERAL

        - Installation Standard:  ASTM C 754, except comply with framing sizes and spacing indicated.

Standards listed in four subparagraphs below include framing installation requirements not in ASTM C 754.  Retain applicable subparagraphs to suit Project.

          - Gypsum Board Assemblies:  Also comply with requirements in ASTM C 840 that apply to framing installation.

        - Install supplementary framing, and blocking to support fixtures, equipment services, heavy trim, grab bars, toilet accessories, furnishings, or similar construction.

        - Install bracing at terminations in assemblies.

Detail control and expansion joints on Drawings.

        - Do not bridge building control and expansion joints with non-load-bearing steel framing members.  Frame both sides of joints independently.

      - INSTALLING FRAMED ASSEMBLIES

Retain first paragraph below if studs will abut dissimilar metals at exterior walls or exterior masonry walls that may become damp.

        - Where studs are installed directly against exterior masonry walls or dissimilar metals at exterior walls, install isolation strip between studs and exterior wall.

        - Install studs so flanges within framing system point in same direction.

Generally, delete subparagraph below and indicate stud spacings on Drawings.  ASTM C 754 includes tabulations of maximum framing spacing based on thickness and orientation of gypsum board panels and deflection and lateral loading.

          - Space studs as follows:

            - Single-Layer Application:  24 inches (610 mm) o.c., unless otherwise indicated.

        - Install tracks (runners) at floors and overhead supports.  Extend framing full height to structural supports or substrates above suspended ceilings, except where partitions are indicated to terminate at suspended ceilings.  Continue framing around ducts penetrating partitions above ceiling.

Indicate locations and details of slip-type and fire-rated head joints on Drawings.  See "Crack Control" Article in the Evaluations.

          - Slip-Type Head Joints:  Where framing extends to overhead structural supports, install to produce joints at tops of framing systems that prevent axial loading of finished assemblies.

          - Door Openings:  Screw vertical studs at jambs to jamb anchor clips on door frames; install runner track section (for cripple studs) at head and secure to jamb studs.

Usually retain subparagraph below.  For heavy doors, 200 to 300 lb (91 to 136 kg) that are up to 48 inches (1219 mm) wide, GA-600 recommends using 0.0312-inch- (0.79-mm-) thick studs.  Design framing for doors more than 48 inches (1219 mm) wide, double doors, and extra-heavy doors to meet loading conditions.

            - Install two studs at each jamb, unless otherwise indicated.

Retain subparagraph below if one-piece control joints are required at heads of doors.

            - Install cripple studs at head adjacent to each jamb stud, with a minimum 1/2-inch (12.7-mm) clearance from jamb stud to allow for installation of control joint in finished assembly.

Retain subparagraph below if suspended ceilings cannot withstand forces generated by opening and closing doors.

            - Extend jamb studs through suspended ceilings and attach to underside of overhead structure.

Delete first subparagraph below if no framed openings other than doors, or revise to suit Project.  Fully detail framing for large openings on Drawings.

          - Other Framed Openings:  Frame openings other than door openings the same as required for door openings, unless otherwise indicated.  Install framing below sills of openings to match framing required above door heads.

          - Sound-Rated Partitions:  Install framing to comply with sound-rated assembly indicated.

        - Direct Furring:

          - Screw to wood framing.

          - Attach to concrete or masonry with stub nails, screws designed for masonry attachment, or powder-driven fasteners spaced 24 inches (610 mm) o.c.

Paragraph below is based on recommendation in GA-216 for wood framing and ASTM C 840 for metal framing.

        - Installation Tolerance:  Install each framing member so fastening surfaces vary not more than 1/8 inch (3 mm) from the plane formed by faces of adjacent framing.

END OF SECTION 092216