Copyright 2004 and 2005 by The American Institute of Architects (AIA)

Exclusively published and distributed by Architectural Computer Services, Inc. (ARCOM) for the AIA

SECTION 095113 - ACOUSTICAL PANEL CEILINGS

This Section uses the term "Architect."  Change this term to match that used to identify the design professional as defined in the General and Supplementary Conditions.

Verify that Section titles referenced in this Section are correct for this Project's Specifications; Section titles may have changed.

- GENERAL

      - RELATED DOCUMENTS

        - Drawings and general provisions of the Contract, including General and Supplementary Conditions and Division 01 Specification Sections, apply to this Section.

      - SUMMARY

        - This Section includes acoustical panels and exposed suspension systems for ceilings.

Delete paragraph below if cast-in-place attachment devices are not required for Project or if piercing hangar tabs are specified in Division 05 Section "Steel Decking" for floor deck.

        - Products furnished, but not installed under this Section, include anchors, clips, and other ceiling attachment devices to be cast in concrete at ceilings.

      - DEFINITIONS

Retain abbreviations that remain after this Section has been edited.

        - AC:  Articulation Class.

        - CAC:  Ceiling Attenuation Class.

        - LR:  Light Reflectance coefficient.

        - NRC:  Noise Reduction Coefficient.

      - SUBMITTALS

        - Product Data:  For each type of product indicated.

Retain first paragraph and subparagraphs below if Drawings do not include detailed reflected ceiling plans or if Project involves unusual coordination requirements.

        - Coordination Drawings:  Reflected ceiling plans, drawn to scale, on which the following items are shown and coordinated with each other, based on input from installers of the items involved:

Edit four subparagraphs and associated subparagraph below to suit Project.

          - Ceiling suspension system members.

          - Method of attaching hangers to building structure.

Delete first subparagraph below if cast-in-place attachment devices are not required for Project.

            - Furnish layouts for cast-in-place anchors, clips, and other ceiling attachment devices whose installation is specified in other Sections.

          - Ceiling-mounted items including lighting fixtures, diffusers, grilles, speakers, sprinklers, access panels, and special moldings.

          - Minimum Drawing Scale:  1/4 inch = 1 foot.

        - Samples for Initial Selection:  For components with factory-applied color finishes.

Delete paragraph above if colors and other characteristics are preselected and specified or scheduled.  Retain paragraph and subparagraphs below with or without above.

        - Samples for Verification:  For each component indicated and for each exposed finish required, prepared on Samples of size indicated below.

Delete first option in first subparagraph below if size of panels makes requirement impractical.

          - Acoustical Panel:  Set of full-size Samples of each type, color, pattern, and texture.

          - Exposed Suspension System Members, Moldings, and Trim:  Set of 12-inch-long Samples of each type, finish, and color.

        - Product Data:

Retain subparagraph and associated subparagraph below if recycled content is required for LEED-NC or LEED-CI Credits MR 4.1 and MR 4.2; coordinate with requirements for materials in Part 2.

          - For products having recycled content, documentation indicating percentages by weight of postconsumer and preconsumer recycled content.

            - Include statement indicating costs for each product having recycled content.

Retain subparagraph below if low-emitting materials are required for LEED-NC or LEED-CI Credit EQ 4.1; coordinate with requirements selected in Part 2 for sealants.

          - For sealants, including printed statement of VOC content.

Coordinate paragraph below with qualification requirements in Division 01 Section "Quality Requirements" and as supplemented in "Quality Assurance" Article.

Retain first paragraph below if Contractor is responsible for field quality-control testing.

Insert specific model code organization in first paragraph below or revise if report must be from another source.  Option below may be required by the IBC.

        - Maintenance Data:  For finishes to include in maintenance manuals.

      - QUALITY ASSURANCE

Retain first paragraph below if Contractor or manufacturer selects testing agency.

Retain paragraph and subparagraphs above or first paragraph below.  Retain below if warranty requirements favor single-source ceiling systems or if ceiling units will fit only same manufacturer's suspension system.  Armstrong World Industries, BPB USA, Chicago Metallic Corporation, Ecophon CertainTeed, and USG Interiors offer both ceiling units and generic exposed suspension systems.

        - Source Limitations:  Obtain each type of acoustical ceiling panel and supporting suspension system through one source from a single manufacturer.

        - Fire-Test-Response Characteristics:  Provide acoustical panel ceilings that comply with the following requirements:

Delete subparagraph and associated subparagraphs below if no fire-resistance-rated assemblies are required.

          - Fire-Resistance Characteristics:  Where indicated, provide acoustical panel ceilings identical to those of assemblies tested for fire resistance per ASTM E 119 by UL or another testing and inspecting agency acceptable to authorities having jurisdiction.

Revise first subparagraph below if only assemblies tested by one or more specifically named testing and inspecting agencies are acceptable.  Indicate rating, testing agency, and testing agency's design designation on Drawings.

            - Fire-Resistance Ratings:  Indicated by design designations from UL's "Fire Resistance Directory" or from the listings of another testing and inspecting agency.

            - Identify materials with appropriate markings of applicable testing and inspecting agency.

Retain one option in subparagraph below.  Class A systems have a flame-spread index not exceeding 25 and a smoke-developed index not exceeding 50; Class B systems have a flame-spread index not exceeding 75 on face side; and Class C systems have a flame-spread index not exceeding 200 on face side.  Most products available in the U.S. are Class A.

          - Surface-Burning Characteristics:  Provide acoustical panels with the following surface-burning characteristics complying with ASTM E 1264 for Class A materials as determined by testing identical products per ASTM E 84:

Retain subparagraph below if retaining option for Class B or C in subparagraph above.  ASTM E 1264 does not specify a smoke-developed index for these two classes.  The IBC and NFPA 5000 set 450 as the maximum allowed smoke-developed index for finish materials of Classes A through C.  Verify value in subparagraph with authorities having jurisdiction.

            - Smoke-Developed Index:  450 or less.

        - Seismic Standard:  Provide acoustical panel ceilings designed and installed to withstand the effects of earthquake motions according to the following:

Verify requirements of authorities having jurisdiction as to which subparagraphs below to reference if any.

          - Standard for Ceiling Suspension Systems Requiring Seismic Restraint:  Comply with ASTM E 580.

          - CISCA's Recommendations for Acoustical Ceilings:  Comply with CISCA's "Recommendations for Direct-Hung Acoustical Tile and Lay-in Panel Ceilings--Seismic Zones 0-2."

          - CISCA's Guidelines for Systems Requiring Seismic Restraint:  Comply with CISCA's "Guidelines for Seismic Restraint of Direct-Hung Suspended Ceiling Assemblies--Seismic Zones 3 & 4."

          - UBC Standard 25-2, "Metal Suspension Systems for Acoustical Tile and for Lay-in Panel Ceilings."

          - ASCE 7, "Minimum Design Loads for Buildings and Other Structures":  Section 9, "Earthquake Loads."

Delete paragraph and subparagraph below if not required.  If retaining, indicate location, size, and other details of mockups on Drawings or by inserts.  Revise wording if only one mockup is required.

Delete paragraph below if Work of this Section is not extensive or complex enough to justify a preinstallation conference.  If retaining, coordinate with Division 01.

        - Preinstallation Conference:  Conduct conference at Project site to comply with requirements in Division 01 Section "Project Management and Coordination."

      - DELIVERY, STORAGE, AND HANDLING

        - Deliver acoustical panels, suspension system components, and accessories to Project site in original, unopened packages and store them in a fully enclosed, conditioned space where they will be protected against damage from moisture, humidity, temperature extremes, direct sunlight, surface contamination, and other causes.

        - Before installing acoustical panels, permit them to reach room temperature and a stabilized moisture content.

        - Handle acoustical panels carefully to avoid chipping edges or damaging units in any way.

      - PROJECT CONDITIONS

Retain this Article to prevent voiding manufacturer's warranty.  Limitations in paragraph and subparagraph below are also necessary for best results in many locations.  If construction scheduling or other conditions conflict with requirements below, consider products such as "Humiguard" by Armstrong, "Ultra 90+" by BPB USA, and "ClimaPlus" by USG, that can be installed under less-than-ideal conditions without voiding warranty.  Revise below if humidity-resistant products do not need stringent environmental limitations.  See Evaluations and manufacturers' product data and warranties.

        - Environmental Limitations:  Do not install acoustical panel ceilings until spaces are enclosed and weatherproof, wet work in spaces is complete and dry, work above ceilings is complete, and ambient temperature and humidity conditions are maintained at the levels indicated for Project when occupied for its intended use.

Retain subparagraph below to comply with CISCA's recommendations for cleaning duct system and protecting ceiling units in pressurized plenums from damage and soiling caused by blowing dirt and dust that may be present when duct system is first operated.

          - Pressurized Plenums:  Operate ventilation system for not less than 48 hours before beginning acoustical panel ceiling installation.

      - COORDINATION

        - Coordinate layout and installation of acoustical panels and suspension system with other construction that penetrates ceilings or is supported by them, including light fixtures, HVAC equipment, fire-suppression system, and partition assemblies.

      - EXTRA MATERIALS

Extra materials may not be allowed for publicly funded projects.

        - Furnish extra materials described below that match products installed and that are packaged with protective covering for storage and identified with labels describing contents.

Revise subparagraphs below to suit Project.  If preferred, replace percentage with a specific number of panels and pieces of each suspension component and their lengths.

          - Acoustical Ceiling Panels:  Full-size panels equal to 2.0 percent of quantity installed.

          - Suspension System Components:  Quantity of each exposed component equal to 2.0 percent of quantity installed.

          - Hold-Down Clips:  Equal to 2.0 percent of quantity installed.

- PRODUCTS

If more than one acoustical ceiling is required, give each set of panel and suspension systems a ceiling designation, such as "APC-1" for the first set and "APC-2" for the second, and create an additional set of requirements and products for each variation.

If one component, such as the suspension system, is paired with more than one panel type, add a second ceiling designation to the article title describing the suspension system and coordinate with each panel type.

      - PERFORMANCE REQUIREMENTS

        - Surface-Burning Characteristics: Comply with ASTM E 84; testing by a qualified testing agency. Identify products with appropriate markings of applicable testing agency.

          - Flame-Spread Index: Comply with ASTM E 1264 for Class A materials.

          - Smoke-Developed Index: 50 or less.

      - ACOUSTICAL PANELS, GENERAL

Retain "Acoustical Panels for Acoustical Panel Ceiling" Article along with this Article.

Retain first paragraph below to specify recycled content if required for LEED-NC or LEED-CI Credits MR 4.1 and MR 4.2.  Recycled content of panels varies; consult manufacturers before inserting requirement.  An alternative method of meeting LEED Credit MR 4.1 and MR 4.2 requirements is to retain requirement in Division 01 Section "Sustainable Design Requirements" that gives Contractor option and responsibility for determining how LEED Credit MR 4.1 and MR 4.2 requirement will be met.

        - Recycled Content:  Provide acoustical panels with recycled content.

        - Acoustical Panel Standard:  Provide manufacturer's standard panels of configuration indicated that comply with ASTM E 1264 classifications as designated by types, patterns, acoustical ratings, and light reflectances, unless otherwise indicated.

Subparagraph below represents standard mounting referenced in ASTM E 1264.  Revise if testing units with another mounting method is required for Project.

          - Mounting Method for Measuring NRC:  Type E-400; plenum mounting in which face of test specimen is 15-3/4 inches away from test surface per ASTM E 795.

        - Acoustical Panel Colors and Patterns:  Match appearance characteristics indicated for each product type.

Retain subparagraph below only if patterns are exclusively specified by using pattern designations in ASTM E 1264.

          - Where appearance characteristics of acoustical panels are indicated by referencing pattern designations in ASTM E 1264 and not manufacturers' proprietary product designations, provide products selected by Contracting Officer from each manufacturer's full range that comply with requirements indicated for type, pattern, color, light reflectance, acoustical performance, edge detail, and size.

        - Broad Spectrum Antimicrobial Fungicide and Bactericide Treatment:  Provide acoustical panels treated with manufacturer's standard antimicrobial formulation that inhibits fungus, mold, mildew, and gram-positive and gram-negative bacteria and showing no mold, mildew, or bacterial growth when tested according to ASTM D 3273 and evaluated according to ASTM D 3274 or ASTM G 21.

Treatment in paragraph above is both a fungicide and antimicrobial and is available from manufacturers, each with their own licensed formulation.  Treatment in paragraph below is to inhibit mold and mildew and is available from Armstrong.  If retaining above or below, coordinate with panel product characteristics in subsequent Part 2 article.  Verify extent of application and, if a specific fungal or bacterial agent is a concern, the performance of these coatings with manufacturers.  Delete if not needed or if treatment is not available for product selected.  Retaining below risks a proprietary specification.

Insert a ceiling designation below to indicate the ceiling that acoustical panels are intended for.  Use same designation for both acoustical panels in the Article below and the suspension system in "Metal Suspension System for Acoustical Panel Ceiling" Article that together make up the ceiling.  Use these designations on Drawings to show where each ceiling is required.  Copy this Article for each ceiling and each acoustical panel required and re-edit to suit Project.

      - ACOUSTICAL PANELS FOR ACOUSTICAL PANEL CEILING

Retain "Acoustical Panels, General" Article with this Article.

See Editing Instruction No. 1 in the Evaluations for cautions about naming manufacturers and products.  Retain one of three paragraphs below.  Retain list of manufacturers with one of first two paragraphs; insert list of manufacturers if retaining third paragraph.  See Division 01 Section "Product Requirements."

        - Basis of Design Product:  Subject to compliance with requirements, provide comparable product by one of the following:

          - Armstrong World Industries, Inc.; Dune, Second Look, Anguled Tegular, Bright White.

          - Certainteed.

          - USG Corp.

Do not assume that every combination of fire-resistance rating, classification, pattern, color, light reflectance, acoustical rating, edge detail, thickness, and size listed under each product description is available.  Before selecting options below to insert salient characteristics, verify availability with manufacturers' product data.

Retain option in first paragraph below along with permitted fire-resistance-rated suspension system if fire-rated assembly is required for Project.  Indicate rating, testing agency, and testing agency's design designation on Drawings.

        - Classification:  Provide panels complying with ASTM E 1264 for type, form, and pattern as follows:

Retain one of six subparagraphs below.  Classifications are examples of commonly specified products and are not all-inclusive.  First three examples are mineral-based panels.

          - Type and Form:  Type III, mineral base with painted finish; Form 2, water felted.

Subparagraph below is an example of a glass-fiber-based panel.

First subparagraph below is an example of a panel with scrubbable finish.

Select or insert at least one pattern designation from options in subparagraph below; use more than one for combination patterns.  For panels with a "K" (surface scored) in their designation, insert a detailed description or indicate on Drawings the type of scoring required.  For panels with a "Z" in their designation, insert both the designation and a detailed description.

          - Pattern:  CE (perforated, small holes and lightly textured K (surface scored).

        - Color:  White.

        - LR:  Not less than 0.83.

        - NRC:  Not less than 0.50.

        - CAC:  Not less than 35.

        - Thickness:  3/4 inch.

Retain paragraph above or first paragraph below.  For most panels, retain above.  For glass-fiber-based panels, retain below.

        - Modular Size:  Tegular 24” x 24”.

Retaining paragraph below risks a proprietary specification.  Verify availability of antimicrobial treatment option selected.

        - Antimicrobial Treatment:  Fungicide based.

        - Sag Resistance:  Homiguard Plus.

      - METAL SUSPENSION SYSTEMS, GENERAL

Retain this Article with "Metal Suspension System for Acoustical Panel Ceiling" Article.

Retain first paragraph below if recycled content is required for LEED-NC or LEED-CI Credits MR 4.1 and MR 4.2.  USGBC allows a default value of 25 percent to be used for steel without documentation; higher percentages can be claimed if they are supported by appropriate documentation.  The Steel Recycling Institute indicates that steel sheet is made by the basic oxygen furnace method, which typically has 23 percent postconsumer recycled content and 1.5 percent preconsumer recycled content.

        - Recycled Content:  Provide products made from steel sheet with average recycled content such that postconsumer recycled content plus one-half of preconsumer recycled content is not less than 25 percent.

        - Metal Suspension System Standard:  Provide manufacturer's standard direct-hung metal suspension systems of types, structural classifications, and finishes indicated that comply with applicable requirements in ASTM C 635.

        - Finishes and Colors, General:  Comply with NAAMM's "Metal Finishes Manual for Architectural and Metal Products" for recommendations for applying and designating finishes.  Provide manufacturer's standard factory-applied finish for type of system indicated.

Delete subparagraph below if not required.  Finish is generally applicable to hot-dip galvanized steel with G60 (Z180) or greater coating and aluminum systems with anodized finish.  ASTM C 635 requires salt-spray testing and high-humidity testing.  On Drawings, show where high-humidity finishes are required.

          - High-Humidity Finish:  Comply with ASTM C 635 requirements for "Coating Classification for Severe Environment Performance" where high-humidity finishes are indicated.

        - Attachment Devices:  Size for five times the design load indicated in ASTM C 635, Table 1, "Direct Hung," unless otherwise indicated.  Comply with seismic design requirements.

Subparagraph below does not apply to power-actuated fasteners.  Delete if no anchorage to concrete is required or if power-actuated fasteners are acceptable.  Verify safety factor with Project's structural engineer.  Revise testing methods below if required by authorities having jurisdiction.

          - Anchors in Concrete:  Anchors of type and material indicated below, with holes or loops for attaching hangers of type indicated and with capability to sustain, without failure, a load equal to five times that imposed by ceiling construction, as determined by testing per ASTM E 488 or ASTM E 1512 as applicable, conducted by a qualified testing and inspecting agency.

Select type from subparagraph below; verify suitability with Project's structural engineer.

            - Type:  Postinstalled expansion anchors.

Retain one of three subparagraphs below or, if more than one is required, indicate by inserting location of each on Drawings.  Zinc plating of mild class indicated protects against corrosion from an indoor atmosphere with rare condensation and subject to minimum wear or abrasion; revise thickness to suit more corrosive conditions or use stainless steel or nickel-copper alloy, depending on conditions.  If postinstalled expansion anchors are used to attach nickel-copper-alloy wire hangers and braces, consider retaining nickel-copper anchors after verifying availability with manufacturers.

            - Corrosion Protection:  Carbon-steel components zinc plated to comply with ASTM B 633, Class Fe/Zn 5 (0.005 mm) for Class SC 1 service condition.

            - Corrosion Protection:  Stainless-steel components complying with ASTM F 593 and ASTM F 594, Group 1 Alloy 304 or 316 for bolts; Alloy 304 or 316 for anchor.

            - Corrosion Protection:  Components fabricated from nickel-copper-alloy rods complying with ASTM B 164 for UNS No. N04400 alloy.

Delete subparagraph below if power-actuated fasteners are not allowed.  Verify safety factor with Project's structural engineer.

          - Power-Actuated Fasteners in Concrete:  Fastener system of type suitable for application indicated, fabricated from corrosion-resistant materials, with clips or other accessory devices for attaching hangers of type indicated, and with capability to sustain, without failure, a load equal to 10 times that imposed by ceiling construction, as determined by testing per ASTM E 1190, conducted by a qualified testing and inspecting agency.

        - Wire Hangers, Braces, and Ties:  Provide wires complying with the following requirements:

Retain one of three subparagraphs below unless more than one type of wire is required.  If more than one type of wire is required, indicate location of each on Drawings.  See Evaluations for discussion on corrosion resistance of hangers and fasteners.  Revise hangers to strap type if required by authorities having jurisdiction or by local union regulations.

          - Zinc-Coated, Carbon-Steel Wire:  ASTM A 641/A 641M, Class 1 zinc coating, soft temper.

          - Stainless-Steel Wire:  ASTM A 580/A 580M, Type 304, nonmagnetic.

          - Nickel-Copper-Alloy Wire:  ASTM B 164, nickel-copper-alloy UNS No. N04400.

Retain second option in subparagraph below if required by authorities having jurisdiction or if desired for extra security and quality (including corrosion allowance).  Because large sizes are difficult to work with, their use could result in poor leveling tolerance.

          - Size:  Select wire diameter so its stress at 3 times hanger design load (ASTM C 635, Table 1, "Direct Hung") will be less than yield stress of wire, but provide not less than 0.106-inch - 0.135-inch diameter wire.

        - Hanger Rods:  Mild steel, zinc coated or protected with rust-inhibitive paint.

Delete paragraph above and below if not required.  Insert sizes here or show on Drawings.

        - Angle Hangers:  Angles with legs not less than 7/8 inch wide; formed with 0.04-inch-thick, galvanized steel sheet complying with ASTM A 653/A 653M, G90 coating designation; with bolted connections and 5/16-inch- diameter bolts.

Retain first paragraph below if required.  Revise if stabilizer bars are needed to improve stability of the suspension system and panel alignment in non-seismic installations.

        - Seismic Stabilizer Bars:  Manufacturer's standard perimeter stabilizers designed to accommodate seismic forces.

        - Seismic Struts:  Manufacturer's standard compression struts designed to accommodate seismic forces.

Retain paragraph above and below if required.  Coordinate with manufacturer's requirements and authorities having jurisdiction.

        - Seismic Clips:  Manufacturer's standard seismic clips designed and spaced to secure acoustical panels in-place.

Delete paragraph below if not required.  Indicate location on Drawings or by inserts.  All fire-resistance- rated panels of manufacturers listed in the Evaluations weigh more than 1 lb/sq. ft. (4.88 kg/sq. m); hold-down clips are not required to comply with UL design requirements.  Verify weights of products selected for UL fire-resistant designs, and revise below to require hold-down clips for fire-resistance-rated ceilings with panels weighing less than 1 lb/sq. ft. (4.88 kg/sq. m).

Delete paragraph below if not required.  Indicate location on Drawings or by inserts.

Delete paragraph below if not required.  Verify, with manufacturers, product availability and compatibility of gasket type with panels and suspension system specified.  Indicate location on Drawings or by inserts.

Insert drawing designation in article title below if more than one acoustical panel ceiling system is required.  Use same designation for both the suspension system in Article below and acoustical panels in "Acoustical Panels for Acoustical Panel Ceiling" Article that together make up the ceiling.  Copy and re-edit this Article for each ceiling and each suspension system required for Project.

      - METAL SUSPENSION SYSTEM FOR ACOUSTICAL PANEL CEILING

Retain this Article along with "Metal Suspension Systems, General" Article.

See Editing Instruction No. 1 in the Evaluations for cautions about naming manufacturers and products.  Retain one of three paragraphs below.  Retain list of manufacturers with one of first two paragraphs; insert list of manufacturers if retaining third paragraph.  See Division 01 Section "Product Requirements."

        - Available Products:  Subject to compliance with requirements, products that may be incorporated into the Work include, but are not limited to, the following:

        - Products:  Subject to compliance with requirements, provide one of the following:

          - Armstrong World Industries, Inc.; PreLude XL Ceiling Grid, Bright White.

Retain option in first paragraph below along with permitted fire-resistance-rated acoustical panel if fire-rated assembly is required for Project.

      - METAL EDGE MOLDINGS AND TRIM

See Editing Instruction No. 1 in the Evaluations for cautions about naming manufacturers and products.  Retain one of first three paragraphs below.  Retain list of manufacturers with one of first two paragraphs; insert list of manufacturers if retaining third paragraph.  See Division 01 Section "Product Requirements."

If more than one type of metal edge molding and trim is required to suit Project, indicate on Drawings or in a schedule.

        - Available Products:  Subject to compliance with requirements, provide comparable product by one of the following:

          - Armstrong World Industries, Inc.

          - Certainteed.

          - USG Corp.

        - Roll-Formed, Sheet-Metal Edge Moldings and Trim:  Type and profile indicated or, if not indicated, manufacturer's standard moldings for edges and penetrations that comply with seismic design requirements; formed from sheet metal of same material, finish, and color as that used for exposed flanges of suspension system runners.

Revise subparagraphs below to suit Project and products selected; show profiles on Drawings.

          - Provide manufacturer's standard edge moldings that fit acoustical panel edge details and suspension systems indicated and that match width and configuration of exposed runners, unless otherwise indicated.

          - For lay-in panels with reveal edge details, provide stepped edge molding that forms reveal of same depth and width as that formed between edge of panel and flange at exposed suspension member.

          - For circular penetrations of ceiling, provide edge moldings fabricated to diameter required to fit penetration exactly.

        - Extruded-Aluminum Edge Moldings and Trim:  Where indicated, provide manufacturer's extruded-aluminum edge moldings and trim of profile indicated or referenced by manufacturer's designations, including splice plates, corner pieces, and attachment and other clips, complying with seismic design requirements and the following:

Revise subparagraphs below to suit Project and products selected; show profiles on Drawings.

          - Aluminum Alloy:  Alloy and temper recommended by aluminum producer and finisher for type of use and finish indicated, and with not less than the strength and durability properties of aluminum extrusions complying with ASTM B 221 for Alloy and Temper 6063-T5.

          - Finish designations prefixed by AA comply with system established by the Aluminum Association for designating aluminum finishes.

Retain finish requirements in one of first four subparagraphs and associated subparagraphs below, or delete below and insert others.

          - Conversion-Coated Finish:  AA-M12C42 (Chemical Finish:  cleaned with inhibited chemicals; acid-chromate-fluoride-phosphate conversion coating).

          - Conversion-Coated and Factory-Primed Finish:  AA-M12C42R1x (Chemical Finish:  cleaned with inhibited chemicals; acid-chromate-fluoride-phosphate conversion coating; organic coating as follows):

Specify field-painting requirements in Division 09 Sections.  Verify compatibility of factory finishes with field-applied coats.

            - Manufacturer's standard, factory-applied prime-coat finish ready for field painting.

          - Class II, Clear Anodic Finish:  AA-M12C22A31 (Mechanical Finish:  nonspecular as fabricated; Chemical Finish:  etched, medium matte; Anodic Coating:  Architectural Class II, clear coating 0.010 mm or thicker) complying with AAMA 611.

          - Baked-Enamel Finish:  AA-C12C42R1x (Chemical Finish:  cleaned with inhibited chemicals; Chemical Finish:  acid-chromate-fluoride-phosphate conversion coating; organic coating:  as specified below).  Apply baked enamel complying with paint manufacturer's written instructions for cleaning, conversion coating, and painting.

            - Organic Coating:  Thermosetting, primer/topcoat system with a minimum dry film thickness of 0.8 to 1.2 mils.

      - ACOUSTICAL SEALANT

See Editing Instruction No. 1 in the Evaluations for cautions about naming manufacturers and products.  Retain one of three paragraphs below.  Retain list of manufacturers with one of first two paragraphs; insert list of manufacturers if retaining third paragraph.  See Division 01Section "Product Requirements."

        - Available Products:  Subject to compliance with requirements, products that may be incorporated into the Work include, but are not limited to, the following:

        - Products:  Subject to compliance with requirements, provide one of the following:

          - Acoustical Sealant for Exposed and Concealed Joints:

            - Pecora Corporation; AC-20 FTR Acoustical and Insulation Sealant.

            - USG Corporation; SHEETROCK Acoustical Sealant.

          - Acoustical Sealant for Concealed Joints:

            - OSI Sealants, Inc.; Pro-Series SC-175 Rubber Base Sound Sealant.

            - Pecora Corporation; BA-98.

            - Tremco, Inc.; Tremco Acoustical Sealant.

Retain option in two paragraphs below if low-emitting materials are required for LEED-NC or LEED-CI Credit EQ 4.1.  VOC limit is that for architectural sealants.  If retaining option, verify that manufacturers' products selected can comply with VOC limit.

- EXECUTION

      - EXAMINATION

        - Examine substrates, areas, and conditions, including structural framing to which acoustical panel ceilings attach or abut, with Installer present, for compliance with requirements specified in this and other Sections that affect ceiling installation and anchorage and with requirements for installation tolerances and other conditions affecting performance of acoustical panel ceilings.

          - Proceed with installation only after unsatisfactory conditions have been corrected.

      - PREPARATION

        - Measure each ceiling area and establish layout of acoustical panels to balance border widths at opposite edges of each ceiling.  Avoid using less-than-half-width panels at borders, and comply with layout shown on reflected ceiling plans.

      - INSTALLATION

        - General:  Install acoustical panel ceilings to comply with ASTM C 636 and seismic design requirements indicated, per manufacturer's written instructions and CISCA's "Ceiling Systems Handbook."

Delete subparagraph below if no fire-rated assembly.

        - Suspend ceiling hangers from building's structural members and as follows:

          - Structural members have spray on fireproofing.  Contractor shall take care during installation to minimize impact on existing fireproofing.  Contractor is responsible for any damages they make to S.E.P.

          - Install hangers plumb and free from contact with other objects within ceiling plenum that are not part of supporting structure or of ceiling suspension system.

          - Splay hangers only where required to miss obstructions; offset resulting horizontal forces by bracing, countersplaying, or other equally effective means.

Delete subparagraph above or first subparagraph below.  Retain below only if fire-resistance-rated ceilings are selected.

          - Splay hangers only where required to miss obstructions; offset resulting horizontal forces by bracing, countersplaying, or other equally effective means.

          - Where width of ducts and other construction within ceiling plenum produces hanger spacings that interfere with location of hangers at spacings required to support standard suspension system members, install supplemental suspension members and hangers in form of trapezes or equivalent devices.

          - Secure wire hangers to ceiling suspension members and to supports above with a minimum of three tight turns.  Connect hangers directly either to structures or to inserts, eye screws, or other devices that are secure and appropriate for substrate and that will not deteriorate or otherwise fail due to age, corrosion, or elevated temperatures.

Delete subparagraph above or below unless both types of hangers are required and their locations are indicated on Drawings or by inserts.

          - Secure flat, angle, channel, and rod hangers to structure, including intermediate framing members, by attaching to inserts, eye screws, or other devices that are secure and appropriate for both structure to which hangers are attached and type of hanger involved.  Install hangers in a manner that will not cause them to deteriorate or fail due to age, corrosion, or elevated temperatures.

Delete subparagraphs below that refer to inapplicable construction types.  Revise first subparagraph if power-actuated fasteners are not allowed.

          - Do not support ceilings directly from permanent metal forms or floor deck.  Fasten hangers to cast-in-place hanger inserts, postinstalled mechanical or adhesive anchors, or power-actuated fasteners that extend through forms into concrete.

          - When steel framing does not permit installation of hanger wires at spacing required, install carrying channels or other supplemental support for attachment of hanger wires.

          - Do not attach hangers to steel deck tabs.

Revise first subparagraph below if structural members are spaced too far apart for hangers and another method is required.  For alternatives that may need to be detailed on Drawings, consult structural engineer and refer to CISCA's guidelines.  See SDI Publication No. 29 and the Evaluations in Division 05 Section "Steel Decking."

          - Do not attach hangers to steel roof deck.  Attach hangers to structural members.

          - Space hangers not more than 48 inches o.c. along each member supported directly from hangers, unless otherwise indicated; provide hangers not more than 8 inches from ends of each member.

          - Size supplemental suspension members and hangers to support ceiling loads within performance limits established by referenced standards and publications.

First paragraph below is based on requirements by the Division of the State Architect in California, for ceilings in hospitals and schools.  Revise or delete if not required.

        - Secure bracing wires to ceiling suspension members and to supports with a minimum of four tight turns.  Suspend bracing from building's structural members as required for hangers, without attaching to permanent metal forms, steel deck, or steel deck tabs.  Fasten bracing wires into concrete with cast-in-place or postinstalled anchors.

        - Install edge moldings and trim of type indicated at perimeter of acoustical ceiling area and where necessary to conceal edges of acoustical panels.

Retain subparagraphs below to eliminate air movement and light and sound leaks at edges of ceiling.

          - Apply acoustical sealant in a continuous ribbon concealed on back of vertical legs of moldings before they are installed.

          - Screw attach moldings to substrate at intervals not more than 16 inches o.c. and not more than 3 inches from ends, leveling with ceiling suspension system to a tolerance of 1/8 inch in 12 feet.  Miter corners accurately and connect securely.

Delete subparagraph below or qualify if exposed fasteners are allowed.

          - Do not use exposed fasteners, including pop rivets, on moldings and trim.

        - Install suspension system runners so they are square and securely interlocked with one another.  Remove and replace dented, bent, or kinked members.

        - Install acoustical panels with undamaged edges and fit accurately into suspension system runners and edge moldings.  Scribe and cut panels at borders and penetrations to provide a neat, precise fit.

Delete subparagraph and associated subparagraphs below if no directionally patterned panels are specified.

          - Arrange directionally patterned acoustical panels as follows:

Retain one of three subparagraphs below.

            - As indicated on reflected ceiling plans to conform to existing layout.

Retain applicable subparagraphs below that coordinate with panel edge details and suspension system types specified in a schedule.

          - For square-edged panels, install panels with edges fully hidden from view by flanges of suspension system runners and moldings.

          - For reveal-edged panels on suspension system runners, install panels with bottom of reveal in firm contact with top surface of runner flanges.

          - For reveal-edged panels on suspension system members with box-shaped flanges, install panels with reveal surfaces in firm contact with suspension system surfaces and panel faces flush with bottom face of runners.

Delete first subparagraph below if all edges are concealed by suspension system flanges.

          - Paint cut edges of panel remaining exposed after installation; match color of exposed panel surfaces using coating recommended in writing for this purpose by acoustical panel manufacturer.

          - Install hold-down clips in areas indicated, in areas required by authorities having jurisdiction, and for fire-resistance ratings; space as recommended by panel manufacturer's written instructions, unless otherwise indicated.

Delete subparagraph below if no clean-room requirements.

Delete subparagraph below if no fire-resistance-rated assemblies.

          - Protect lighting fixtures and air ducts to comply with requirements indicated for fire-resistance-rated assembly.

Insert requirements for semiconcealed system (splined joints, etc.) if any.

Insert other field quality-control procedures required for Project.

      - CLEANING

        - Clean exposed surfaces of acoustical panel ceilings, including trim, edge moldings, and suspension system members.  Comply with manufacturer's written instructions for cleaning and touchup of minor finish damage.  Remove and replace ceiling components that cannot be successfully cleaned and repaired to permanently eliminate evidence of damage.

END OF SECTION 095113