Copyright 2005 by The American Institute of Architects (AIA)

Exclusively published and distributed by Architectural Computer Services, Inc. (ARCOM) for the AIA

SECTION 096513 - RESILIENT BASE AND ACCESSORIES

Revise this Section by deleting and inserting text to meet Project-specific requirements.

This Section uses the term "Architect."  Change this term to match that used to identify the design professional as defined in the General and Supplementary Conditions.

Verify that Section titles referenced in this Section are correct for this Project's Specifications; Section titles may have changed.

- GENERAL

      - RELATED DOCUMENTS

Retain or delete this article in all Sections of Project Manual.

        - Drawings and general provisions of the Contract, including General and Supplementary Conditions and Division 1 Specification Sections, apply to this Section.

      - SUMMARY

        - Section Includes:

          - Resilient base.

      - SUBMITTALS

Five paragraphs in this article are defined in Division 1 Section "Submittal Procedures" as "Action Submittals."

        - Product Data:  For each type of product indicated.

Delete "Samples for Initial Selection" Paragraph above if colors and other characteristics are preselected and specified or scheduled.  Retain first paragraph below with or without above.

        - Samples for Verification:  For each type of product indicated, in manufacturer's standard-size Samples but not less than 12 inches (300 mm) long, of each resilient product color, texture, and pattern required.

        - Product Schedule:  For resilient products.  Use same designations indicated on drawings.

      - QUALITY ASSURANCE

Delete first paragraph below if fire-test-response testing is not required by authorities having jurisdiction.  In most building codes, resilient products are exempt from fire-test-response requirements.  See "Fire-Test-Response Characteristics" Article in the Evaluations for additional information.

        - Mockups:  Provide resilient products with mockups specified in other sections.

      - DELIVERY, STORAGE, AND HANDLING

        - Store resilient products and installation materials in dry spaces protected from the weather, with ambient temperatures maintained within range recommended by manufacturer, but not less than 50 deg F (10 deg C) or more than 90 deg F (32 deg C).

      - PROJECT CONDITIONS

Ambient temperature range for installation varies among manufacturers.  Consult manufacturers for recommendations and revise first paragraph below to suit Project.

        - Maintain ambient temperatures within range recommended by manufacturer, but not less than 70 deg F (21 deg C) or more than 95 deg F (35 deg C), in spaces to receive resilient products during the following time periods:

          - 48 hours before installation.

          - During installation.

          - 48 hours after installation.

        - Install resilient products after other finishing operations, including painting, have been completed.

      - EXTRA MATERIALS

        - Furnish extra materials that match products installed and that are packaged with protective covering for storage and identified with labels describing contents.

          - Furnish not less than 10 linear feet (3 linear m) for every 500 linear feet (150 linear m) or fraction thereof, of each type, color, pattern, and size of resilient product installed.

- PRODUCTS

      - RESILIENT BASE B-1

Refer to the tables at the end of the Evaluations for a list of manufacturers' products.  Use these tables in combination with manufacturers' catalogs or product data to insert series, type, model, and designations of other characteristics.

Copy this article and re-edit for each product.

Insert number to complete drawing designation.  Use these designations on Drawings to identify each product.

        - Resilient Base:

See Editing Instruction No. 1 in the Evaluations for cautions about naming manufacturers.  Retain subparagraph and list of manufacturers below.  See Division 1 Section "Product Requirements."

          - Manufacturers:  Subject to compliance with requirements, provide products as follows by one of the following:

            - Roppe Corporation, USA.

            - Johnsonite; Millwork Reveal.

        - Resilient Base Standard:  ASTM F 1861.

See "Resilient Base" Article in the Evaluations for a discussion of material requirement, manufacturing method, and style.

          - Material Requirement:  Type TP (rubber, thermoplastic).

          - Manufacturing Method:  Group I (solid, homogeneous).

          - Style:  Straight (flat or toeless).

Both types of rubber base are generally 0.125 inch (3.2 mm) thick; however, some manufacturers offer various thicknesses.  Vinyl base is available in both thickness options in first paragraph below but not from every manufacturer.

        - Minimum Thickness:  0.125 inch (3.2 mm) (2.0 mm).

        - Height:  Per finish schedule.

Coordinate height selected in paragraph above with length selected in first paragraph below; some manufacturers do not offer coils for every height.

        - Lengths:  Coils in manufacturer's standard length (1219 mm).

        - Outside Corners:  Preformed.

Coordinate first paragraph below with "Resilient Base Installation" Article.

        - Inside Corners:  Preformed.

        - Finish:  As selected by Contracting Officer from manufacturer's full range.

Retain paragraph below if colors and patterns are not indicated in a separate schedule.

        - Colors:  As selected by Contracting Officer from manufacturer’s full range.

      - INSTALLATION MATERIALS

        - Trowelable Leveling and Patching Compounds:  Latex-modified, portland cement based or blended hydraulic-cement-based formulation provided or approved by manufacturer for applications indicated.

        - Adhesives:  Water-resistant type recommended by manufacturer to suit resilient products and substrate conditions indicated.

Retain subparagraph below if low-emitting materials are required for LEED-NC or LEED-CI Credit EQ 4.1.

          - Use adhesives that comply with the following limits for VOC content when calculated according to 40 CFR 59, Subpart D (EPA Method 24):

            - Cove Base Adhesives:  Not more than 50 g/L.

Retain subparagraph below if rubber stair treads or molding accessories are required.

Retain first paragraph below if resilient stair treads are specified in this Section.

Retain first paragraph below if metal edge strips are required.

Before retaining paragraph below, verify manufacturers' written instructions for floor polish products.  Delete paragraph if this Section is used with resilient floor coverings at stair landings (with floor polish) specified in other Sections.

- EXECUTION

      - EXAMINATION

Coordinate requirements specified in other Sections for substrate construction and tolerances to ensure that they are appropriate for resilient products selected.

        - Examine substrates, with Installer present, for compliance with requirements for maximum moisture content and other conditions affecting performance of the Work.

        - Verify that finishes of substrates comply with tolerances and other requirements specified in other Sections and that substrates are free of cracks, ridges, depressions, scale, and foreign deposits that might interfere with adhesion of resilient products.

        - Proceed with installation only after unsatisfactory conditions have been corrected.

      - PREPARATION

Extensive surface preparation is required over substrates from which existing products have been removed.  Requirements vary among manufacturers.  Insert requirements to suit Project.

        - Prepare substrates according to manufacturer's written instructions to ensure adhesion of resilient products.

        - Fill cracks, holes, and depressions in substrates with trowelable leveling and patching compound and remove bumps and ridges to produce a uniform and smooth substrate.

        - Do not install resilient products until they are same temperature as the space where they are to be installed.

          - Move resilient products and installation materials into spaces where they will be installed at least 48 hours in advance of installation.

        - Sweep and vacuum clean substrates to be covered by resilient products immediately before installation.

      - RESILIENT BASE INSTALLATION

        - Comply with manufacturer's written instructions for installing resilient base.

        - Apply resilient base to walls, columns, pilasters, casework and cabinets in toe spaces, and other permanent fixtures in rooms and areas where base is required.

        - Install resilient base in lengths as long as practicable without gaps at seams and with tops of adjacent pieces aligned.

        - Tightly adhere resilient base to substrate throughout length of each piece, with base in continuous contact with horizontal and vertical substrates.

        - Do not stretch resilient base during installation.

Retain first paragraph below if required or revise to suit Project.

        - On masonry surfaces or other similar irregular substrates, fill voids along top edge of resilient base with manufacturer's recommended adhesive filler material.

Retain first paragraph below for resilient base with preformed corners; retain second paragraph for resilient base with job-formed corners.

        - Preformed Corners:  Install preformed corners before installing straight pieces.

      - CLEANING AND PROTECTION

        - Comply with manufacturer's written instructions for cleaning and protection of resilient products.

        - Perform the following operations immediately after completing resilient product installation:

          - Remove adhesive and other blemishes from exposed surfaces.

Delete first subparagraph below if only resilient base is included in this Section.

          - Sweep and vacuum surfaces thoroughly.

          - Damp-mop surfaces to remove marks and soil.

        - Protect resilient products from mars, marks, indentations, and other damage from construction operations and placement of equipment and fixtures during remainder of construction period.

Before retaining first paragraph below, verify resilient stair tread manufacturer's written instructions for floor polish.  Generally, floor polish is used with most vinyl products.

        - Cover resilient products until Substantial Completion.

END OF SECTION 096513