Copyright 2003 by The American Institute of Architects (AIA)

Exclusively published and distributed by Architectural Computer Services, Inc. (ARCOM) for the AIA

SECTION 096813 - CARPET TILE

This Section uses the term "Architect."  Change this term to match that used to identify the design professional as defined in the General and Supplementary Conditions.

Verify that Section titles referenced in this Section are correct for this Project's Specifications; Section titles may have changed.

- GENERAL

      - RELATED DOCUMENTS

        - Drawings and general provisions of the Contract, including General and Supplementary Conditions and Division 1 Specification Sections, apply to this Section.

      - SUMMARY

        - This Section includes modular, tufted cut and loop carpet tile.

        - Related Sections include the following:

List below only products that the reader might expect to find in this Section but are specified elsewhere.

          - Division 9 Section "Resilient Wall Base and Accessories" for resilient wall base and accessories installed with carpet tile.

      - SUBMITTALS

        - Product Data:  For each type of product indicated.  Include manufacturer's written data on physical characteristics, durability, and fade resistance.  Include installation recommendations for each type of substrate.

        - Samples:  For each of the following products and for each color and texture required.  Label each Sample with manufacturer's name, material description, color, pattern, and designation indicated on Drawings and in schedules.

Delete subparagraphs below if not applicable.

          - Carpet Tile:  Full-size Sample.

Coordinate first paragraph below with qualification requirements in Division 1 Section "Quality Requirements" and as supplemented in "Quality Assurance" Article.

        - Maintenance Data:  For carpet tiles to include in maintenance manuals.  Include the following:

          - Methods for maintaining carpet tile, including cleaning and stain-removal products and procedures and manufacturer's recommended maintenance schedule.

          - Precautions for cleaning materials and methods that could be detrimental to carpet tile.

        - Warranty:  Special warranty specified in this Section.

      - QUALITY ASSURANCE

Delete paragraph below if Installer certification is not required; other Installer qualifications are included in Division 1 Section "Quality Requirements."

        - Installer Qualifications:  An experienced installer who is certified by the Floor Covering Installation Board or who can demonstrate compliance with its certification program requirements.

Delete paragraph below if no fire-resistance-rated units are required.

        - Fire-Test-Response Characteristics:  Provide products with the critical radiant flux classification indicated in Part 2, as determined by testing identical products per ASTM E 648 by an independent testing and inspecting agency acceptable to authorities having jurisdiction.

Delete paragraph and subparagraph below if not required.  If retaining, indicate location, size, and other details of mockups on Drawings or by inserts.  Revise wording if only one mockup is required.

      - DELIVERY, STORAGE, AND HANDLING

        - Comply with CRI 104, Section 5, "Storage and Handling."

      - PROJECT CONDITIONS

        - Comply with CRI 104, Section 7.2, "Site Conditions; Temperature and Humidity" and Section 7.12, "Ventilation."

        - Environmental Limitations:  Do not install carpet tiles until wet work in spaces is complete and dry, and ambient temperature and humidity conditions are maintained at the levels indicated for Project when occupied for its intended use.

Delete first paragraph below if products are not adhesively applied to concrete substrates.  Revise to suit Project.

        - Do not install carpet tiles over concrete slabs until slabs have cured and are sufficiently dry to bond with adhesive and concrete slabs have pH range recommended by carpet tile manufacturer.

        - Where demountable partitions or other items are indicated for installation on top of carpet tiles, install carpet tiles before installing these items.

      - WARRANTY

When warranties are required, verify with Owner's counsel that special warranties stated in this Article are not less than remedies available to Owner under prevailing local laws.  Coordinate with Division 1 Section "Product Requirements."

Revise paragraph and subparagraphs below if manufacturers have separate warranties for various characteristics.

        - Special Warranty for Carpet Tiles:  Manufacturer's standard form in which manufacturer agrees to repair or replace components of carpet tile installation that fail in materials or workmanship within specified warranty period.

          - Warranty does not include deterioration or failure of carpet tile due to unusual traffic, failure of substrate, vandalism, or abuse.

          - Failures include, but are not limited to, more than 10 percent loss of face fiber, edge raveling, snags, runs, loss of tuft bind strength, dimensional stability, excess static discharge, and delamination.

Verify available warranties and insert number in subparagraph below.

          - Warranty Period:  10 years from date of Substantial Completion.

If Installer warranty is available and required to suit Project, add here.

      - EXTRA MATERIALS

Extra materials may not be allowed for publicly funded projects.

        - Furnish extra materials described below, before installation begins, that match products installed and that are packaged with protective covering for storage and identified with labels describing contents.

Revise subparagraph below to suit Project.  Replace percentage with a specific number of units.

          - Carpet Tile:  Full-size units equal to 5 percent of amount installed for each type indicated, but not less than 10 sq. yd. (8.3 sq. m).

- PRODUCTS

Insert designation in article title below to coordinate Specifications with Drawings.  Replace insert note with the designation used on Drawings or in schedules to indicate locations where a specific type of carpet tile is required.  Copy Article below and re-edit it for each product required.  If a project includes only one type of carpet tile, consider deleting the designation in article title.

      - CARPET TILE

Retain one of two paragraphs and list of manufacturers and products below.  See Division 1 Section "Product Requirements."

        - Products:  Subject to compliance with requirements, provide the following product or equal:

For competitive bidding, copy subparagraph and associated subparagraphs below for each additional product.

          - Carpet tile as selected by Contracting Officer from manufacturer’s full range.

For competitive bidding from several sources, delete subparagraph below.

Delete remainder of this Article if products are limited to those listed above.

Retain first paragraph below and insert proprietary fiber type such as Antron, Ultron, or Zeftron if required.

        - Pile Characteristic:  Level loop.

Revise paragraphs and subparagraphs remaining in this Article to suit Project; add additional requirements if any.  There are many criteria for specifying carpet tile construction and many test methods for specifying carpet tile performance, but there is little consistency among manufacturers in test methods used or in readily providing comprehensive technical information for carpet tile.  See Evaluations for guidelines.

Test method in first paragraph below is applicable only to finished level-loop and cut-pile constructions with tuft heights less than 0.25 inch (6.3 mm).

Do not include weight of backings in first paragraph below.

        - Surface Pile Weight:  26 oz./sq. yd. (g/sq. m)

Revise paragraph below to suit Project; for example, if antimicrobial treatment is integral to backing formulation, if backing density and thickness are optional characteristics to be selected, or if performance characteristics and test method results for backings are available.

Delete paragraph below if not required.

Retain one or both paragraphs above or first paragraph below to describe backing system.

        - Size:  24 by 24 inches (457 by 457 mm) (610 by 610 mm) (457 by 914 mm) (914 by 914 mm).

Delete paragraph below if soil resistance is integral to carpet tile fiber or if soil resistance is not needed for Project; consult manufacturer.

Delete first paragraph below if antimicrobial treatment is not needed for Project; consult manufacturer.

Delete first subparagraph below if not required.  Verify requirements of authorities having jurisdiction.

Insert other performance characteristics to suit Project, such as smoke density, texture retention, additional colorfastness characteristics, and quantification of soil-resistance treatment.

      - INSTALLATION ACCESSORIES

Revise paragraph below to suit Project.

        - Trowelable Leveling and Patching Compounds:  Latex-modified, hydraulic-cement-based formulation provided or recommended by carpet tile manufacturer.

Revise paragraph below to suit product(s) and Project.

        - Adhesives:  Water-resistant, mildew-resistant, nonstaining, pressure-sensitive type to suit products and subfloor conditions indicated, that complies with flammability requirements for installed carpet tile and is recommended by carpet tile manufacturer for releasable installation.

Retain subparagraph below if low-emitting materials are required for LEED-NC or LEED-CI Credit EQ 4.3.

          - VOC Limits:  Provide adhesives with VOC content not more than 50 g/L when calculated according to 40 CFR 59, Subpart D (EPA method 24).

- EXECUTION

      - EXAMINATION

        - Examine substrates, areas, and conditions, with Installer present, for compliance with requirements for maximum moisture content, alkalinity range, installation tolerances, and other conditions affecting carpet tile performance.  Examine carpet tile for type, color, pattern, and potential defects.

Retain paragraph and subparagraphs below if products are adhesively attached to concrete subfloors.

        - Concrete Subfloors:  Verify that concrete slabs comply with ASTM F 710 and the following:

Revise first subparagraph below to suit Project.

          - Slab substrates are dry and free of curing compounds, sealers, hardeners, and other materials that may interfere with adhesive bond.  Determine adhesion and dryness characteristics by performing bond and moisture tests recommended by carpet tile manufacturer.

          - Subfloor finishes comply with requirements specified by manufacturer for slabs receiving carpet tile.

          - Subfloors are free of cracks, ridges, depressions, scale, and foreign deposits.

Delete paragraph and subparagraphs below if no wood subfloors.

Delete paragraph and subparagraph below if no metal subfloors.

Delete paragraph and subparagraph below if no painted subfloors.

Delete first paragraph and subparagraphs below if no raised access flooring systems.  See Evaluations.  Consider tightening installation tolerances for access flooring in Division 10 Section "Access Flooring."

        - Proceed with installation only after unsatisfactory conditions have been corrected.

      - PREPARATION

        - General:  Comply with CRI 104, Section 6.2, "Site Conditions; Floor Preparation," and with carpet tile manufacturer's written installation instructions for preparing substrates indicated to receive carpet tile installation.

Revise paragraph below to suit product and Project.  Dimensions below are recommended by CRI.

        - Use trowelable leveling and patching compounds, according to manufacturer's written instructions, to fill cracks, holes, depressions, and protrusions in substrates.  Fill or level cracks, holes and depressions 1/8 inch (3 mm) wide or wider and protrusions more than 1/32 inch (0.8 mm), unless more stringent requirements are required by manufacturer's written instructions.

Retain paragraph below if products are adhesively attached to concrete substrates.

        - Remove coatings, including curing compounds, and other substances that are incompatible with adhesives and that contain soap, wax, oil, or silicone, without using solvents.  Use mechanical methods recommended in writing by carpet tile manufacturer.

Delete first paragraph below if no metal substrates.

        - Clean metal substrates of grease, oil, soil and rust, and prime if directed by adhesive manufacturer.  Rough sand painted metal surfaces and remove loose paint.  Sand aluminum surfaces, to remove metal oxides, immediately before applying adhesive.

        - Broom and vacuum clean substrates to be covered immediately before installing carpet tile.

      - INSTALLATION

        - General:  Comply with CRI 104, Section 14, "Carpet Modules," and with carpet tile manufacturer's written installation instructions.

Revise first paragraph below if mill-applied peel-and-stick adhesives or standard adhesives are used.  Full-spread glue-down installation is appropriate for wheel traffic.  Partial glue-down installation is appropriate for carpet tiles of moderate dimensional stability, weight, and mass.  Free-lay installation is appropriate for dimensionally stable carpet tiles with heavy backings.  Revise second option if spot-glued installation is recommended by manufacturer and suits product and Project.

        - Installation Method:  As recommended in writing by carpet tile manufacturer.

        - Maintain dye lot integrity.  Do not mix dye lots in same area.

        - Cut and fit carpet tile to butt tightly to vertical surfaces, permanent fixtures, and built-in furniture including cabinets, pipes, outlets, edgings, thresholds, and nosings.  Bind or seal cut edges as recommended by carpet tile manufacturer.

        - Extend carpet tile into toe spaces, door reveals, closets, open-bottomed obstructions, removable flanges, alcoves, and similar openings.

        - Maintain reference markers, holes, and openings that are in place or marked for future cutting by repeating on finish flooring as marked on subfloor.  Use nonpermanent, nonstaining marking device.

        - Install pattern parallel to walls and borders.

Delete paragraph below if no raised access flooring systems.  Adhesive in seams may impede panel removal and replacement.

        - Stagger joints of carpet tiles so carpet tile grid is offset from access flooring panel grid.  Do not fill seams of access flooring panels with carpet adhesive; keep seams free of adhesive.

      - CLEANING AND PROTECTION

        - Perform the following operations immediately after installing carpet tile:

          - Remove excess adhesive, seam sealer, and other surface blemishes using cleaner recommended by carpet tile manufacturer.

          - Remove yarns that protrude from carpet tile surface.

          - Vacuum carpet tile using commercial machine with face-beater element.

        - Protect installed carpet tile to comply with CRI 104, Section 16, "Protection of Indoor Installations."

        - Protect carpet tile against damage from construction operations and placement of equipment and fixtures during the remainder of construction period.  Use protection methods indicated or recommended in writing by carpet tile manufacturer.

END OF SECTION 096813