Copyright 2003 and 2005 by The American Institute of Architects (AIA)

Exclusively published and distributed by Architectural Computer Services, Inc. (ARCOM) for the AIA

SECTION 224000 - PLUMBING FIXTURES

Verify that Section titles referenced in this Section are correct for this Project's Specifications; Section titles may have changed.

- GENERAL

      - RELATED DOCUMENTS

        - Drawings and general provisions of the Contract, including General and Supplementary Conditions and Division 01 Specification Sections, apply to this Section.

      - SUMMARY

        - This Section includes the following conventional plumbing fixtures and related components:

Adjust list below to suit Project.

          - Faucets for sinks.

          - Protective shielding guards.

          - Hot-water dispensers.

          - Kitchen sinks.

      - DEFINITIONS

Retain abbreviations and terms that remain after this Section has been edited.

        - ABS:  Acrylonitrile-butadiene-styrene plastic.

        - Accessible Fixture:  Plumbing fixture that can be approached, entered, and used by people with disabilities.

        - Cast Polymer:  Cast-filled-polymer-plastic material.  This material includes cultured-marble and solid-surface materials.

        - Cultured Marble:  Cast-filled-polymer-plastic material with surface coating.

        - Fitting:  Device that controls the flow of water into or out of the plumbing fixture.  Fittings specified in this Section include supplies and stops, faucets and spouts, shower heads and tub spouts, drains and tailpieces, and traps and waste pipes.  Piping and general-duty valves are included where indicated.

        - FRP:  Fiberglass-reinforced plastic.

        - PMMA:  Polymethyl methacrylate (acrylic) plastic.

        - PVC:  Polyvinyl chloride plastic.

        - Solid Surface:  Nonporous, homogeneous, cast-polymer-plastic material with heat-, impact-, scratch-, and stain-resistance qualities.

      - SUBMITTALS

        - Product Data:  For each type of plumbing fixture indicated.  Include selected fixture and trim, fittings, accessories, appliances, appurtenances, equipment, and supports.  Indicate materials and finishes, dimensions, construction details, and flow-control rates.

Retain first paragraph below if fixtures include electrical components and wiring.

        - Shop Drawings:  Diagram power, signal, and control wiring.

        - Operation and Maintenance Data:  For plumbing fixtures to include in emergency, operation, and maintenance manuals.

        - Warranty:  Special warranty specified in this Section.

      - QUALITY ASSURANCE

Revise first paragraph and subparagraph below to suit manufacturers and types of fixtures specified.  No manufacturer makes all types of fixtures, faucets, etc.

        - Source Limitations:  Obtain plumbing fixtures, faucets, and other components of each category through one source from a single manufacturer.

          - Exception:  If fixtures, faucets, or other components are not available from a single manufacturer, obtain similar products from other manufacturers specified for that category.

        - Electrical Components, Devices, and Accessories:  Listed and labeled as defined in NFPA 70, Article 100, by a testing agency acceptable to authorities having jurisdiction, and marked for intended use.

Retain option in first paragraph below if required by authorities having jurisdiction.

        - Regulatory Requirements:  Comply with requirements in ICC A117.1, "Accessible and Usable Buildings and Facilities"; Public Law 90-480, "Architectural Barriers Act"; and Public Law 101-336, "Americans with Disabilities Act" for plumbing fixtures for people with disabilities.

        - Regulatory Requirements:  Comply with requirements in Public Law 102-486, "Energy Policy Act," about water flow and consumption rates for plumbing fixtures.

        - NSF Standard:  Comply with NSF 61, "Drinking Water System Components--Health Effects," for fixture materials that will be in contact with potable water.

        - Select combinations of fixtures and trim, faucets, fittings, and other components that are compatible.

        - Comply with the following applicable standards and other requirements specified for plumbing fixtures:

Retain only subparagraphs below that match fixtures, components, and features specified.

          - Stainless-Steel Residential Sinks:  ASME A112.19.3.

        - Comply with the following applicable standards and other requirements specified for sink faucets:

Retain only subparagraphs below that match faucets, components, and features specified.

          - Faucets:  ASME A112.18.1.

          - NSF Potable-Water Materials:  NSF 61.

          - Pipe Threads:  ASME B1.20.1.

          - Supply Fittings:  ASME A112.18.1.

          - Brass Waste Fittings:  ASME A112.18.2.

        - Comply with the following applicable standards:

Retain only subparagraphs below that match faucets, components, and features specified.

          - Hot-Water Dispensers:  ASSE 1023 and UL 499.

          - Supply and Drain Protective Shielding Guards:  ICC A117.1.

      - WARRANTY

When warranties are required, verify with Owner's counsel that special warranties stated in this Article are not less than remedies available to Owner under prevailing local laws.  Coordinate with Division 01 Section "Product Requirements."

        - Special Warranties:  Manufacturer's standard form in which manufacturer agrees to repair or replace components of whirlpools that fail in materials or workmanship within specified warranty period.

          - Failures include, but are not limited to, the following:

            - Structural failures of unit shell.

            - Faulty operation of controls, blowers, pumps, heaters, and timers.

            - Deterioration of metals, metal finishes, and other materials beyond normal use.

Verify available warranties for units and components and insert numbers below.

          - Warranty Period for Commercial Applications:  One year from date of Substantial Completion.

      - EXTRA MATERIALS

Extra materials may not be allowed for publicly funded projects.

        - Furnish extra materials described below that match products installed and that are packaged with protective covering for storage and identified with labels describing contents.

Revise subparagraphs below to suit Project.

          - Faucet Washers and O-Rings:  Equal to 1 percent of amount of each type and size installed.

          - Faucet Cartridges and O-Rings:  Equal to 1 percent of amount of each type and size installed.

- PRODUCTS

Part 2 articles specify typical plumbing fixtures and components.  Retain and edit only those articles applicable to Project.

If applying for LEED certification, retain options in articles below that limit flow rates and usage to those used in the calculation to show water use savings.

      - SINK FAUCETS

Copy and edit paragraph and subparagraphs in this Article for each type sink faucet required.

        - Sink Faucets:

See Editing Instruction No. 1 in the Evaluations for cautions about naming manufacturers and products.  Retain one of first three subparagraphs and list of manufacturers below.  See Division 01 Section "Product Requirements."

          - Basis-of-Design Product:  Subject to compliance with requirements, provide the product indicated on Drawings or a comparable product by one of the following:

Retain one of five lists below.

Select from list below for commercial, solid-brass faucets.

            - American Standard Companies, Inc.

            - Bradley Corporation.

            - Broadway Collection.

            - Chicago Faucets.

            - Delta Faucet Company.

            - Dormont Manufacturing Company.

            - Eljer.

            - Elkay Manufacturing Co.

            - Fisher Manufacturing Co.

            - Grohe America, Inc.

            - Just Manufacturing Company.

            - Kohler Co.

            - Moen, Inc.

            - Royal Brass Mfg. Co.

            - Sayco; a Briggs Plumbing Products, Inc. Company.

            - Speakman Company.

            - T & S Brass and Bronze Works, Inc.

            - Zurn Plumbing Products Group; Commercial Brass Operation.

          - Description:  Kitchen faucet without spray.  Include hot- and cold-water indicators; coordinate faucet inlets with supplies and fixture holes; coordinate outlet with spout and fixture receptor.

If basis-of-design product manufacturer's name, product name or designation, and description above are explicit enough, requirements in subparagraphs below may be reduced or omitted.

            - Body Material:  Commercial, solid brass.

            - Finish:  Polished chrome plate.

            - Maximum Flow Rate:  2.5 gpm, unless otherwise indicated.

            - Mixing Valve:  As per model number.

            - Backflow Protection Device for Hose Outlet:  Not required.

            - Backflow Protection Device for Side Spray:  Not required.

            - Centers:  4 inches.

            - Mounting:  Deck.

            - Handle(s):  Wrist blade, 4 inches.

            - Inlet(s):  NPS 3/8 tubing with NPS 1/2 male adapter.

            - Spout Type:  Swing, round tubular.

            - Spout Outlet:  Aerator.

            - Vacuum Breaker:  Not required.

            - Operation:  Compression, manual.

            - Drain:  Basket.

      - PROTECTIVE SHIELDING GUARDS

        - Protective Shielding Pipe Covers:

See Editing Instruction No. 1 in the Evaluations for cautions about naming manufacturers and products.  Retain one of first two subparagraphs and list of manufacturers below.  See Division 01 Section "Product Requirements."

          - Manufacturers:  Subject to compliance with requirements, provide products by one of the following:

            - Engineered Brass Co.

            - Insul-Tect Products Co.; a Subsidiary of MVG Molded Products.

            - McGuire Manufacturing Co., Inc.

            - Plumberex Specialty Products Inc.

            - TCI Products.

            - TRUEBRO, Inc.

            - Zurn Plumbing Products Group; Tubular Brass Plumbing Products Operation.

            - Approved equal.

          - Description:  Manufactured plastic wraps for covering plumbing fixture hot- and cold-water supplies and trap and drain piping.  Comply with Americans with Disabilities Act (ADA) requirements.

        - Protective Shielding Piping Enclosures:

See Editing Instruction No. 1 in the Evaluations for cautions about naming manufacturers and products.  Retain one of first two subparagraphs and list of manufacturers below.  See Division 01 Section "Product Requirements."

          - Manufacturers:  Subject to compliance with requirements, provide products by one of the following:

            - TRUEBRO, Inc.

            - Approved equal.

          - Description:  Manufactured plastic enclosure for covering plumbing fixture hot- and cold-water supplies and trap and drain piping.  Comply with ADA requirements.

      - HOT-WATER DISPENSERS

Copy and edit paragraph and subparagraphs in this Article for each type hot-water dispenser required.

        - Hot-Water Dispensers:

See Editing Instruction No. 1 in the Evaluations for cautions about naming manufacturers and products.  Retain one of first three subparagraphs and list of manufacturers below.  See Division 01 Section "Product Requirements."

          - Basis-of-Design Product:  Subject to compliance with requirements, provide the product indicated on Drawings or a comparable product by one of the following:

Retain one of two lists below.

Select from list below for gooseneck spout with lever-handle, flow-control hot-water dispensers.

            - Anaheim Manufacturing, Inc.; a Subsidiary of Western Industries, Inc.

            - Franke Consumer Products, Inc.; Kitchen Systems Div.

            - In-Sink-Erator; a div. of Emerson Electric Co.

            - Just Manufacturing Company.

            - Elkay Manufacturing Co.

            - KitchenAid.

            - Approved equal.

          - Description:  Spout with twist-knob or push-button, flow control, household-type dispenser with instant on-off control; insulated, corrosion-resistant-metal storage tank that is open to atmosphere; electric heating element; chrome-plated faucet or spout; removable strainer; thermostat control for water temperature up to 190 deg F (88 deg C); and thermal-overload protection.

If basis-of-design product manufacturer's name, product name or designation, and description above are explicit enough, requirements in subparagraphs below may be reduced or omitted.

            - Storage Tank Capacity:  0.5 gal. (1.5 L).

            - Heating Element:  750 W minimum, 115-V ac.

      - KITCHEN SINKS

Copy and edit paragraph and subparagraphs below for each type kitchen sink required.

        - Kitchen Sinks:

See Editing Instruction No. 1 in the Evaluations for cautions about naming manufacturers and products.  Retain one of first three subparagraphs and list of manufacturers below.  See Division 01 Section "Product Requirements."

          - Basis-of-Design Product:  Subject to compliance with requirements, provide the product indicated on Drawings or a comparable product by one of the following:

Retain one of five lists below.

Select from list below for enameled, cast-iron kitchen sinks.

            - Elkay Manufacturing Co.

            - Eljer.

            - Jacuzzi, Inc.

            - Kohler Co.

Select from list below for PMMA kitchen sinks.

            - Gerber Plumbing Fixtures LLC.

Select from list below for porcelain-enameled, formed-steel kitchen sinks.

            - American Standard Companies, Inc.

          - Description:  Two-bowl, residential, counter-mounting, stainless-steel kitchen sink.

If basis-of-design product manufacturer's name, product name or designation, and description above are explicit enough, requirements in 13 subparagraphs and associated subparagraphs below may be reduced or omitted.

            - Overall Dimensions:  As scheduled.

            - Metal Thickness:  0.050 inch.

Retain "Bowl" Subparagraph and associated subparagraphs above for single-bowl sink or first two or all three bowl subparagraphs and associated subparagraphs below for multibowl sink.

            - Left Bowl:

              - Dimensions:  As scheduled.

              - Drain:  3-1/2-inch (89-mm) crumb cup.

                - Location:  Near back of bowl.

            - Right Bowl:

              - Dimensions:  As scheduled.

              - Drain:  3-1/2-inch (89-mm) crumb cup.

                - Location:  Near back of bowl.

            - Sink Faucet:  As scheduled.

            - Supplies:  NPS 1/2 (DN 15) chrome-plated copper with stops.

            - Drain Piping:  NPS 1-1/2 (DN 40) chrome-plated, cast-brass P-trap; 0.045-inch- (1.1-mm-) thick tubular brass waste to wall; continuous waste; and wall escutcheon(s).

Retain subparagraph above or first subparagraph below.

            - Drain Piping:  Schedule 40 PVC, NPS 1-1/2 (DN 40) P-trap; tubular waste to wall; continuous waste; and wall escutcheon(s).

            - Disposer:  Not required.

            - Dishwasher Air-Gap Fitting:  Not required.

            - Hot-Water Dispenser:  See other paragraphs of this section.

Copy and edit paragraph and subparagraphs below for each type bar sink required.

- EXECUTION

      - EXAMINATION

        - Examine roughing-in of water supply and sanitary drainage and vent piping systems to verify actual locations of piping connections before plumbing fixture installation.

        - Examine cabinets, counters, floors, and walls for suitable conditions where fixtures will be installed.

        - Proceed with installation only after unsatisfactory conditions have been corrected.

      - INSTALLATION

        - Assemble plumbing fixtures, trim, fittings, and other components according to manufacturers' written instructions.

First paragraph below applies to counter-mounting fixtures such as lavatories and sinks.

        - Install counter-mounting fixtures in and attached to casework.

        - Install fixtures level and plumb according to roughing-in drawings.

        - Install water-supply piping with stop on each supply to each fixture to be connected to water distribution piping.  Attach supplies to supports or substrate within pipe spaces behind fixtures.  Install stops in locations where they can be easily reached for operation.

          - Exception:  Use full port ball, gate, or globe valves if supply stops are not specified with fixture.

        - Install trap and tubular waste piping on drain outlet of each fixture to be directly connected to sanitary drainage system.

        - Install tubular waste piping on drain outlet of each fixture to be indirectly connected to drainage system.

        - Install faucet-spout fittings with specified flow rates and patterns in faucet spouts if faucets are not available with required rates and patterns.  Include adapters if required.

        - Install water-supply flow-control fittings with specified flow rates in fixture supplies at stop valves.

        - Install faucet flow-control fittings with specified flow rates and patterns in faucet spouts if faucets are not available with required rates and patterns.  Include adapters if required.

        - Install traps on fixture outlets.

          - Exception:  Omit trap on fixtures with integral traps.

          - Exception:  Omit trap on indirect wastes, unless otherwise indicated.

        - Install hot-water dispensers in back top surface of sink or in countertop with spout over sink.

        - Install escutcheons at piping wall ceiling penetrations in exposed, finished locations and within cabinets and millwork.  Use deep-pattern escutcheons if required to conceal protruding fittings.  Escutcheons are specified in Division 22 Section "Escutcheons for Plumbing Piping."

Delete paragraph below if sealants are specified in Division 07 Section "Joint Sealants."

        - Seal joints between fixtures and walls, floors, and countertops using sanitary-type, one-part, mildew-resistant silicone sealant.  Match sealant color to fixture color.  Sealants are specified in Division 07 Section "Joint Sealants."

      - CONNECTIONS

Coordinate piping installations and specialty arrangements with schematics on Drawings and with requirements specified in piping systems.  If Drawings are explicit enough, these requirements may be reduced or omitted.

        - Piping installation requirements are specified in other Division 22 Sections.  Drawings indicate general arrangement of piping, fittings, and specialties.

        - Connect fixtures with water supplies, stops, and risers, and with traps, soil, waste, and vent piping.  Use size fittings required to match fixtures.

        - Ground equipment according to Division 26 Section "Grounding and Bonding for Electrical Systems."

        - Connect wiring according to Division 26 Section "Low-Voltage Electrical Power Conductors and Cables."

      - FIELD QUALITY CONTROL

        - Verify that installed plumbing fixtures are categories and types specified for locations where installed.

        - Check that plumbing fixtures are complete with trim, faucets, fittings, and other specified components.

        - Inspect installed plumbing fixtures for damage.  Replace damaged fixtures and components.

        - Test installed fixtures after water systems are pressurized for proper operation.  Replace malfunctioning fixtures and components, then retest.  Repeat procedure until units operate properly.

      - ADJUSTING

        - Operate and adjust faucets and controls.  Replace damaged and malfunctioning fixtures, fittings, and controls.

In paragraph below, delete appliances not included in this Section.

        - Operate and adjust hot-water dispensers.  Replace damaged and malfunctioning units and controls.

Delete fixture types in first paragraph below not in Project.

        - Adjust water pressure at faucets to produce proper flow and stream.

        - Replace washers and seals of leaking and dripping faucets and stops.

      - CLEANING

        - Clean fixtures, faucets, and other fittings with manufacturers' recommended cleaning methods and materials.  Do the following:

          - Remove faucet spouts and strainers, remove sediment and debris, and reinstall strainers and spouts.

          - Remove sediment and debris from drains.

        - After completing installation of exposed, factory-finished fixtures, faucets, and fittings, inspect exposed finishes and repair damaged finishes.

      - PROTECTION

        - Provide protective covering for installed fixtures and fittings.

        - Do not allow use of plumbing fixtures for temporary facilities unless approved in writing by Owner.

END OF SECTION 224000