PLANNING REPORT #4-04

OFFICE/WAREHOUSE EXPANSION

VICINITY OF 1039 RIVERSIDE STREET

SUBDIVISION, SITE PLAN AND SITE LOCATION OF DEVELOPMENT REVIEW

1039 RIVERSIDE STREET LLC, APPLICANT

Submitted to:

Portland Planning Board

Portland, Maine

February 10, 2004

Submitted by:

Kandice Talbot, Planner

I.	INTRODUCTION

1039 Riverside Street LLC is proposing a 64,980 sq. ft. office/warehouse expansion to be located on their property at 1039 Riverside Street.  The applicant proposes to develop an additional 9 units of warehouse/industrial space on the site.  Each space will be developed for the purpose of leasing to tenants.  The site is zoned I-M and is approximately 16.68 acres in size.  The project shall be reviewed for site plan and Site Location of Development compliance.

The site currently consists of two buildings.  The first building is the principal structure and is approximately 52,852 sq. ft. in size and a smaller 4,106 sq. ft. building is also located on site.  The proposed development will include eight (8) one-story 6,000 sq. ft. warehouse/office buildings (three are possibly expandable to 8,000 sq. ft.), and one (1) one-story 14,700 sq. ft. warehouse.  The 14,700 sq. ft. building may be divided into multiple tenant spaces.  The applicant will retain ownership of the property and will establish a condominium association for tenants.

36 notices were sent to area residents.  A notice also appeared in the February 2nd edition of the Portland Press Herald. A neighborhood meeting was held on December 30, 2003.  No members of the public attended the neighborhood meeting.

II.	SUMMARY OF FINDINGS

Zoning:			I-M Moderate Industrial Zone

Parcel Size:		16.68 acres

Parking Spaces:		184 parking spaces proposed; 105 parking spaces required

Uses:			Warehouse/Industrial

III.	STAFF REVIEW

The proposed development has been reviewed by staff for conformance with the relevant review standards of the subdivision and site plan ordinances and compliance with the site development of location act.  Staff comments are highlighted in this report.

IV.	SUBDIVISION REVIEW

1.	Water and Air Pollution

The development will not result in undue water or air pollution.

2/3.	Water/Electricity

The utility services for water and power to the proposed development will come from Riverside Street and are considered adequate for the modest needs of the site.  A new 6” water line will be extended to the site off the existing 12” water on Riverside Street.  A water capacity letter from Portland Water District is included as Attachment 16.

Electrical services include an overhead primary line being extended off Riverside Street.  The subdivision ordinance requires that all utilities be placed underground.  A potential condition of approval is:

-	that all electrical lines shall be installed underground.

4.	Soil Erosion

A soil erosion plan was submitted and has been reviewed by the Development Review Coordinator.  The DRC’s memo is included as Attachment 22.

5.	Traffic

The site will be accessed via an existing 24-foot wide paved drive located in the middle of the lot frontage off Riverside Street and a 30-footwide paved drive located along the west boundary of the site.  The proposed development will include upgrades and paving to both drives including the extension of the 30-foot drive along the western boundary to provide access to the new development area located at the back of the site.

184 parking spaces are proposed, 105 parking spaces are required.  Granite curb exists along the entire frontage of the site.  Staff is currently reviewing the status of sidewalk along the frontage and will have information available at the public hearing.

6.	Sanitary Sewer/Soils

Sanitary sewer service will include a6” PVC sewer laterals from each proposed building conveyed through a series of proposed sewer manholes prior to tie-in to the 10-inch PVC sewer interceptor pipe with ultimate discharge into the Presumpscot North Main Interceptor Sewer.

A sewer capacity letter from the Portland Sewer Division is included as Attachment 15.

7.	Stormwater

The 16.19 acre rectangular-shaped site consists of an approximately 55,000 sq. ft. office/warehouse and approximately 55,000 sq. ft. of existing hard surface around the office/warehouse.  The remainder of the site consists of graveled surfaces, undeveloped grassed areas and woodlands.  The site contains an existing 30-foot wide utility easement that traverses the back portion of the lot.  This easement consists of a swale containing an unnamed tributary that ultimately discharges into the Presumpscot River and a 10-inch PVC sewer interceptor pipe that conveys sewerage from the upper Riverside Street sever area with ultimate discharge into the Presumpscot North Main Interceptor Sewer.  Existing drainage swales along the east/west lot boundaries convey stormwater runoff into the existing tributary swale.

The applicant is proposing a new stormwater collection system, which will include catch basins and 15-inch, 12-inch and 10-inch HDPE storm drains to convey stormwater from the proposed development.  Three stormwater collection systems are included in the developed site.  The first stormwater system will serve the front of the site along Riverside Street.  Runoff from the paved areas and buildings will primarily sheet flow to collection inlets and be discharged into an existing 15-inch storm drain on Riverside Street.  A second pipe network will collect runoff from around the new 14,700 sq. ft. building development area.  The third pipe network includes multiple catch basins for proposed units 6-9 at the rear of the site.  Runoff from the paved areas and buildings will primarily sheet flow to collection inlets and be routed through water quality units prior to discharge to riprap outlet aprons located at the top of bank of the tributary swale.  The remainder of the site will remain as in its current condition.

The plans were recently revised to decrease the amount of wetland impacts.  The Development Review Coordinator has reviewed the plans and has concerns with a number of detail aspects of the plan.  The Development Review Coordinator’s memo is included as Attachment 22.  A potential condition of approval is:

-	that the plans be revised in accordance with the Development Review Coordinator’s memo dated February 6, 2004 to be reviewed and approved by the Development Review Coordinator.

8.	Scenic Beauty

The applicant is proposing to preserve some existing vegetation along Riverside Street and at the rear of the property, which abuts the Maine Turnpike.

9.	Comprehensive Plan

The proposal is in compliance with the City’s comprehensive plan.

10.	Financial Capability

The applicant has submitted a letter of financial capability, which is included as Attachment 4.

11.	Groundwater

It does not appear that this development will adversely affect the quality or quantity of groundwater.

12.	Flood Hazard/Shoreland

The development is not located in a flood-prone area.

13.	Wetlands

The applicant is proposing to fill 4,214 sq. ft. of the wetlands area.  This is below the 4,300 sq. ft. threshold outlined in the MEDEP NRPA regulations, therefore, the project is no longer subject to the reporting requirements under NRPA.

14.	Condominium Documents

The applicant is currently preparing the condominium documents for Corporation Counsel’s review.  A potential condition of approval is:

-	that the condominium documents be provided to staff for review and approval by Corporation Counsel prior to building permit.

V.	SITE PLAN REVIEW

1.	Traffic

See Subdivision Review, Traffic Section

2.	Bulk, Location, Height of Proposed Buildings

As stated previously, the applicant proposes to develop an additional 9 units of warehouse/industrial space on the site.  Each space will be developed for the purpose leasing to tenants.  The proposed buildings will be metal-frame structures and will include access entrances, loading docks, and overhead doors.  A typical building elevation has been submitted and included as Attachment 24.  Elevations of all sides of the buildings must be submitted, along with façade material.  A potential condition of approval is:

-	that elevations of all sides of the buildings showing façade material shall be submitted for review and approval by staff prior to issuance of a building permit.

The proposed plans are showing future expansions of some of the buildings.  The review of this project does not include future expansion of buildings and any expansion shall be reviewed and approved by the Planning Authority.  A potential condition of approval is:

-	that the review of the project does not include future expansion of buildings and any expansion shall be reviewed and approved by the Planning Authority.

3.	Sewers, Stormdrains, Water

See Subdivision Review, Sewer and Water Sections

4.	Landscaping and Existing Vegetation

The applicant has stated that landscaping will be minimal, since the project is located in a light industrial area and is also located on a site sheltered by mature trees all around it.  The applicant is proposing twenty-nine (29) white pines around the site.  This will include fourteen white pines between the easterly edge of the property and the Maine Turnpike.  The applicant is also proposing four (4) Crabapples within the site.  Staff is recommending that the applicant provide foundation planting around the proposed buildings. Dumpsters are provided around the site for the buildings.  Dumpsters are required to be enclosed on all four sides.  A detail must be submitted of the dumpster enclosure.  A potential condition of approval is:

-	that the applicant provide foundation plantings around the proposed buildings and that the City Arborist review and approve the landscaping plan prior to issuance of a building permit.  A detail of the dumpster enclosures shall be submitted to staff for review and approval.

5.	Soils and Drainage

See Subdivision Review, Drainage Section

6.	Exterior Lighting

The applicant is proposing a number of light fixtures around the site.  The applicant is proposing twenty-two (22) light poles along the drive and within the parking areas.  The light poles will be 22 ft. high and 250-watt light fixtures.  The applicant is also proposing twenty-three (23) wall-mounted fixtures.  These wall-mounted fixtures will be 250-watts.  Lighting catalogue cuts and a photometric plan has been submitted.  The lighting plan is in accordance with the City’s lighting standards.

7.	Fire

The Fire Department has reviewed the plans and recommended approval of additional fire hydrants.  The applicant has submitted revised plans showing two additional fire hydrants.  A potential condition of approval is:

-	that the Fire Department approve the proposed locations of the additional fire hydrants prior to issuance of a building permit.

VI.	STAFF RECOMMENDATION

Staff recommends that the Planning Board approve the 1039 Riverside Street LLC plan with the proposed conditions of approval.

VII.	MOTIONS FOR THE BOARD TO CONSIDER

On the basis of plans and materials submitted by the applicant and on the basis of information contained in Planning Report #4-04 relevant to standards for subdivision and site plan regulations, and other findings as follows:

i.	That the proposed development [is/is not] in conformance with the Site Location of Development Review.

ii.	That the proposed development [is/is not] in conformance with the Subdivision Ordinance of the Land Use Code.

Potential Conditions of Approval:

1.	that all electrical lines shall be installed underground.

2.	that the plans be revised in accordance with the Development Review Coordinator’s memo dated February 6, 2004 to be reviewed and approved by the Development Review Coordinator.

3.	that the condominium documents be provided to staff for review and approval by Corporation Counsel prior to building permit.

iii.	That the proposed development is/is not in conformance with the Site Plan Ordinance of the Land Use Code.

Potential Conditions of Approval:

1.	that elevations of all sides of the buildings showing façade material shall be submitted for review and approval by staff prior to issuance of a building permit.

2.	that the review of the project does not include future expansion of buildings and any expansion shall be reviewed and approved by the Planning Authority.

3.	that the applicant provide foundation plantings around the proposed buildings and that the City Arborist review and approve the landscaping plan prior to issuance of a building permit.  A detail of the dumpster enclosures shall be submitted to staff for review and approval.

4.	that the Fire Department approve the proposed locations of the additional fire hydrants prior to issuance of a building permit.

Attachments:

1.	Applicant’s Letter dated September 24, 2003

2.	Development Description with Attached Maps

3.	Deed

4.	Financial Capacity

5.	Technical Ability

6.	Review Criteria

7.	Solid Waste

8.	Stormwater Management Narrative

9.	Erosion Control

10.	Landscape Plan Discussion

11.	Public Works’ Memo dated October 2, 2003 and January 21, 2004

12.	DRC’s Memo dated December 2, 2003

13.	Neighborhood Notice dated December 15, 2003

14.	Applicant’s Submittal dated January 8, 2004

15.	Sewer Capacity Letter

16.	Water Capacity Letter

17.	Site Photographs

18.	Applicant’s Submittal dated February 2, 2004

19.	Lighting Catalogue Cuts

20.	Applicant’s Submittal dated February 5, 2004

21.	Applicant’s Submittal dated February 5, 2004 regarding Wetlands Impact

22.	DRC’s Memo dated Februrary 6, 2004

23.	Lighting Photometric Plan

24.	Elevation Drawing

25.	Plans