05P152

TO:       	Kandice S. Talbot – Planner

FROM:  	Jim Seymour – Development Review Coordinator, Sebago Technics, Inc.

# RE:       	Major Site Plan Review:  2nd Tee Business Park – 1039 Riverside LLC

# DATE:   	January 10, 2006

Sebago Technics has reviewed the supporting documentation dated January 9, 2005 for the proposed redevelopment of seven (7) units at the originally approved site, and addition of 3(three) new units off of the former Douglas Brothers facility and current land owned by the Riverside Municipal Golf Course located at 1039 Riverside Street in the City of Portland.  We have reviewed the recent submittals and now agree that the applicant has provided adequate treatment for the site. The follwiing responses shall be considered as minor items which should be placed on any conditional approval

1.	Stormwater Management

Overall, the stormwater management plan and subsequent runoff quantity calculations appear to be acceptable and meet requirements of the City of Portland.  We agree with the principle of “beating the peak” and thereby not requiring detention and that all prectical measures have been exhausted to treat the runoff to remove sediment and other pollutants.

A.	We feel that the parking area associated with the existing warehouse has now been adequately treated runoff with the design of an infiltration swale and because it accomplishes far more treatment over more impervious area. The treatment of this existing parking lot is more beneficial than placing the water quality unit # 5. Unit 5 is so small it has no significant beneficial impact, and feel its elimination with the revised infiltration swale treatment measures for the existing parking area, provide better quality to the adjacent wetlands and Presumpscot River.

B.	The applicant has agreed to add a note stating this on the grading plan and/or note which specific roof tops may daylight to the wetlands in the rear, if any.

2.	Road Access/Circulation

A.	Please refer to comments from the City Traffic Engineer for concerns of traffic movements, trip generation, and internal movements if any.

3.	Utilities

A.	Letters to serve and available capacities appear to have been obtained showing that adequate service exists for the development. A condition is that these letters are needed.

4.	Water Quality Treatment

A.	Prior to construction the actual details of the individual water quality units and treatment swales selected shall be submitted and reviewed in/on plans for Staff/DRC/ Dev. Review Engineer review. All devices must be in compliance with MeDEP sizing criteria for performance standards and an overall stormwater maintenance plan shall be provided for each treatment measure proposed on site.

5.	General

A.	Please see planners report regarding violations of wetland impacts during construction of Phase-1 and needs to restore non-permitted wetland impacts. Planning staff or Dev. Review Engineer must review final geotechnical reports.

B.	See City Arborist report for any landscape comments if any.

Overall, the revised 1039 Riverside Street 2nd Tee Condominium Business Park project has been well planned out and meets the ordinance compliance for Site Plan requirements. The applicant has made the necessary revisions, and made certain compromises per our requests to provide improved stormwater quality for the project to conform to both MeDEP and City of Portland standards as noted in comments above. We are confident that this project should be considered for approval with minor conditions. Please contact our office if you have any questions.

JRS/jrs