## Memorandum

Department of Planning and Development

Planning Division

To:		Chair Caron and Members of the Portland Planning 	Board

From:		Kandice Talbot, Planner

Date:		December 9, 2003

Re:		Office/Warehouse Expansion, 1039 Riverside Street

1039 Riverside Street LLC, Applicant

### Introduction

1039 Riverside Street LLC is proposing a 64,980 sq. ft. office/warehouse expansion to be located on their property at 1039 Riverside Street.  The applicant proposes to develop an additional 9 units of warehouse/industrial space on the site.  Each space will be developed for the purpose of leasing to tenants.  The site is zoned I-M and is approximately 16.68 acres in size.  The project shall be reviewed for site plan and Site Location of Development compliance.

The site currently consists of two buildings.  The first building is the principal structure and is approximately 52,852 sq. ft. in size and a smaller 4,106 sq. ft. building is also located on site.  The proposed development will include eight (8) one-story 6,000 sq. ft. warehouse/office buildings (three are possibly expandable to 8,000 sq. ft.), and one (1) one-story 14,700 sq. ft. warehouse.  The 14,700 sq. ft. building may be divided into multiple tenant spaces.  The applicant will retain ownership of the property and will establish a condominium association for tenants.

### Access/Circulation

The site will be access via an existing 24-foot wide paved drive located in the middle of the lot frontage off Riverside Street and a 30-foot-wide gravel drive located along the west boundary of the site.  The proposed development will include upgrades and paving to both drives include the extension of the 30-foot drive along the western boundary to provide access to the new development area located at the back of the site.

It appears that the applicant has already paved the proposed 30 ft. driveway on the west edge of the property.  It also appears that this driveway was connected to the driveway of the abutting property owned by 400 Riverside Properties.  If this is the case, the applicant will need to provide access easements for the driveways.

184 parking spaces are proposed, 105 are required.

There is existing granite curb and sidewalk along the frontage of this site.  The plans shall be revised to reflect the sidewalk and curb.

### Utilities

The utility services for water and power to the proposed development will come from Riverside Street and are considered adequate for the modest needs of the site.  A new 6” water line will be extended to the site off the existing 12” water on Riverside Street.  Electrical services include an overhead primary line being extended off Riverside Street.

Sanitary sewer service will include 6” PVC sewer laterals from each proposed building conveyed through a series of proposed sewer manholes prior to tie-in to the 10-inch PVC sewer interceptor pipe with ultimate discharge into the Presumpscot North Main Interceptor Sewer.

The applicant will have to submit sewer and water capacity letters from the Portland Water District and Portland Sewer Division prior to public hearing.

Public Works has reviewed the plans and is recommending that the applicant specify the limits of disturbance with Riverside Street and the pavement repairs required per the City’s Street Opening Ordinance.  Riverside Street, between Warren Avenue and Forest Avenue is a 5-year moratorium street.  Public Works’ memo is included as Attachment 11.

### Drainage

The 16.19 acre rectangular-shaped site consists of an approximately 55,000 sq. ft. office/warehouse and approximately 55,000 sq. ft. of existing hard surface around the office/warehouse.  The remainder of the site consists of graveled surfaces, undeveloped grassed areas and woodlands.  The site contains an existing 30-foot wide utility easement that traverses the back portion of the lot.  This easement consists of a swale containing an unnamed tributary that ultimately discharges into the Presumpscot River and a 10-inch PVC sewer interceptor pipe that conveys sewerage from the upper Riverside Street service area with ultimate discharge into the Presumpscot North Main Interceptor Sewer.  Existing drainage swales along the east/west lot boundaries convey stormwater runoff into the existing tributary swale.

The applicant is proposing a new stormwater collection system, which will include catch basins and 15-inch, 12-inch, and 10-inch HDPE storm drains to convey stormwater from the proposed development.  Three stormwater collection systems are included in the developed site.  The first stormwater system will serve the front of the site along Riverside Street. Runoff from the paved areas and buildings will primarily sheet flow to collection inlets and be discharged into an existing 15-inch storm drain on Riverside Street.  A second pipe network will collect runoff from around the new 14,700 sq. ft. building development area.  The third pipe network includes multiple catch basins for proposed units 6-9 at the rear of the site.  Runoff from the paved areas and buildings will primarily sheet flow to collection inlets and be routed through water quality units prior to discharge to riprap outlet aprons located at the top of bank of the tributary swale.  The remainder of the site will remain as in its current condition.

The Development Review Coordinator has reviewed the plans and finds that overall, the stormwater management plan and subsequent calculations appear to be acceptable and meet most of the requirements of both the MDEP and the City.  However, there are a few discrepancies that will need to be addressed prior to Public Hearing.  Also, although the applicant has provided Vortechnics units for the parking areas on the eastern portion of the site, a third unit should be provided at the 24-foot existing paved entrance to the site.  This would serve to treat runoff from the remaining onsite parking areas.  The DRC’s memo is included as Attachment 12.

### Lighting

The applicant has stated that the site’s lighting will primarily consist of wall-pack units.  Staff is requesting that the applicant submit lighting catalogue cuts and a lighting photometric plan to determine that the proposed lighting is in compliance with the City’s technical standards.

### Landscaping

The applicant has stated that landscaping will be minimal, since the project is located in a light industrial area and is also located on a site sheltered by mature trees all around it.  The applicant is proposing ten (10) White Pine and two (2) Zumi Crabapple around the site.  The City Arborist is currently reviewing the plans.

### Building Design

As stated previously, the applicant proposes to develop an additional 9 units of warehouse/industrial space on the site.  Each space will be developed for the purpose of leasing to tenants.  It is expected that the buildings will be traditional metal-framed structures with one or more entrances, on or two overhead doors and possible loading areas.  The typical floor plan will consist of open warehouse, a small office, which will be less than 750 sq. ft. and a restroom in each building.  Staff is requesting that the applicant submit elevations of the building with materials for review.

Staff is wondering if it is possible for the applicant to consolidate some of the buildings, so as to not disturb as much area on the site.

### Fire

The Fire Department has reviewed the plans and is recommending approval of additional fire hydrants.

### Wetlands

A significant amount of wetlands will be filled as a result of this development.  The plans should specify the actual area to be filled, and the permit should be forwarded to the City, prior to plan approval.

Other

The applicant is required to file a Maine Construction General Permit for this project.  This must be obtained prior to the start of construction.

### Items To Be Resolved Prior To Public Hearing

1.	Access Easements

2.	Utility Capacity Letters

3.	Public Works’ Concerns

4.	Development Review Coordinator’s Memo

5.	Lighting Issues

6.	City Arborist Review

7.	Hydrant Locations

8.	Wetlands

9.	Elevations

Attachments:

1.	Applicant’s Letter dated September 24, 2003

2.	Development Description with Attached Maps

3.	Deed

4.	Financial Capacity

5.	Technical Ability

6.	Review Criteria

7.	Solid Waste

8.	Stormwater Management Narrative

9.	Erosion Control

10.	Landscape Plan Discussion

11.	Public Works’ Memo dated October 2, 2003

12.	DRC’s Memo dated December 2, 2003

13. 	Plans